Gold Mobile Mechanic Ultimate Diagnosis Manual

Internal technician knowledge base • Batches 01–02

Mission: Turn elite technician evidence into fast, safe, confirmable field diagnosis: symptom → branch → tool placement → expected result → repair or escalate.

# How to use this manual

- Internal field aid, not a substitute for vehicle-specific service information, specifications, training, or judgment.

- Do not skip safety gates, load conditions, meter-reference proof, or post-repair confirmation.

- Treat every numeric value as vehicle-specific unless the entry explicitly labels it as a general threshold.

- Current scope is the internal Ultimate Diagnosis Manual only; no public-app workflow is included.

# System map

Verified video → Process Extract → Agent Knowledge decision tree → Master manual → Machine-readable app knowledge → Technician-guided diagnosis

# Batch 01 coverage

- Starter no-crank voltage-drop split

- Loaded power and ground testing

- Dead electrical output feed/ground/component split

- No PCM communication with low 5 V reference

- Cranking no-start fuel-delivery branch

Source spreadsheet: Gold Mobile Mechanic - Technician Video Pipeline

# 1. Starter no-crank: loaded voltage-drop split

Symptom: No crank; the starter appears to have battery voltage but does not operate normally.

Root cause demonstrated: High resistance at the starter B+ stud/nut/cable-eyelet connection.

Decision tree: NO CRANK → prove B+ under CRANK → prove S-terminal command → check housing/block ground drop → split post / nut / eyelet → clean or repair → repeat loaded tests and start.

## Evidence source

Voltage Drop Testing a Starter Motor (Quick Tips) — full timestamped transcript and real-frame inspection completed.

## Ordered field steps

1. Secure the vehicle in Park/Neutral, set the parking brake, control the wheels, and keep clear of belts and fans.

2. Prove an incandescent test light at battery positive; clip it to battery negative.

3. Probe the starter B+ post while a helper holds CRANK. If the lamp dims or goes out under load, the B+ path has excessive resistance.

4. Probe the S terminal while cranking. It should light only during CRANK; this proves the command reaches the solenoid.

5. Probe the starter housing/engine block from battery negative during CRANK. Any illumination indicates ground-side voltage drop.

6. Move the probe from the post to the nut and cable eyelet while cranking. The point where brightness returns brackets the bad connection.

7. Disconnect battery negative, clean/tighten the confirmed connection, reconnect, and repeat the same loaded tests.

8. Confirm normal crank/start, a bright B+ test under load, and no abnormal housing-to-battery-negative indication.

## Grab list

Tools: Incandescent test light; fine probe; insulated hand tools; battery-terminal tools; optional remote starter switch; eye protection.

## Safety and escalation

Do not short the B+ stud to the housing. Disconnect the battery before cleaning the connection. Escalate when safe starter access requires a lift or exhaust removal.

## Common misdiagnosis

Replacing a starter because voltage is present with no load, or testing only the cable nut instead of the actual starter post.

# 2. Loaded power and ground circuits

Symptom: Poor running, unstable warning-lamp behavior, or no communication even though an unloaded ground appears to have continuity.

Root cause demonstrated: A high-resistance PCM ground can read approximately zero ohms when disconnected yet develop a large voltage rise under operating current.

Decision tree: REPRODUCE UNDER LOAD → prove meter ground → measure suspect ground to battery negative → split shared path → controlled fused jumper only as an aid → repair → verify communication and operation.

## Evidence source

Voltage Drop vs Resistance Testing — full timestamped transcript and real-frame inspection completed.

## Ordered field steps

1. Reproduce the symptom with the module connected and powered.

2. Prove the selected meter ground by measuring a known B+ source.

3. Measure from the module ground terminal or wire to battery negative while the circuit is operating.

4. Interpret near-zero voltage as the goal. A meaningful voltage rise means resistance exists between the module ground and battery negative.

5. Compare multiple grounds and shared points. Use a fused temporary jumper only as a controlled diagnostic aid.

6. Trace the splice, eyelet, fastener, corrosion, paint, looseness, or broken strands.

7. Repair the path, repeat the loaded voltage-drop test, and verify communication and running quality.

## Grab list

Tools: DVOM; incandescent test light/load tool; wiring diagram; fused jumper; back-probes.

## Safety and escalation

Never use an ohmmeter on a powered circuit or place uncontrolled jumpers at module pins.

## Common misdiagnosis

Accepting 0 Ω on an unplugged ground and replacing the PCM.

# 3. Dead electrical output: feed / ground / component split

Symptom: An electrical output such as a horn, solenoid, relay-controlled device, or motor is completely dead.

Root cause demonstrated: In the demonstrated case, the feed and ground passed under command; the horn had an internal failure, with corrosion also visible at the immediate terminal.

Decision tree: DEAD OR WEAK? → if DEAD, keep load connected and command it → B+ test → ground-to-ground test → component/connector verdict. If WEAK, exit to measured voltage-drop and current testing.

## Evidence source

Voltage Drop Testing with a Test Light? Part 1 — full timestamped transcript and real-frame inspection completed.

## Ordered field steps

1. Classify the output as completely dead or merely weak. This lamp method is for a dead output/open circuit.

2. Keep the component connected when accessible and command it through the normal switch, bidirectional control, or a correctly identified fused relay bypass.

3. Prove an incandescent, non-LED test light at the battery.

4. From battery negative, check the B+ feed while the output is commanded. Bright under the real load supports a good feed; dim or out means split the positive path.

5. Without changing test-light polarity, probe the ground wire while commanded. The lamp should stay dark; illumination indicates a bad ground path.

6. Good feed plus a dark ground test plus a dead output isolates the component and its immediate connector/pigtail.

7. Inspect corrosion, spread terminals, heat, and broken wire under insulation; repair or replace only what is confirmed.

8. Reconnect, command the output, and repeat the loaded checks.

## Grab list

Tools: Incandescent test light; fine piercing/back-probe; wiring diagram; optional bidirectional scan tool; fused relay bypass; terminal tools.

## Safety and escalation

Do not use unfused jumpers, spread female terminals, or use bulb brightness to quantify a weak motor.

## Common misdiagnosis

Unplugging the real load and trusting a bright lamp, or using an LED test light that applies almost no load.

# 4. No PCM communication with low 5 V reference

Symptom: Cranking no-start, no PCM communication, and approximately 1.8–2.3 V on a circuit that should carry a 5 V sensor reference.

Root cause demonstrated: A PCM power relay clicked but its burned load contacts dropped the supply voltage; a controlled substitute restored PCM power, 5 V reference, communication, and starting.

Decision tree: NO COMM → prove meter ground → check a known 5 V sensor → key-off shared-sensor isolation → PCM fuse/power/ground checks → load-test relay feed and output → restore 5 V, communication, and start.

## Evidence source

Troubleshooting a No Start, No Spark, No Fuel, No Com — full timestamped transcript and real-frame inspection completed.

## Ordered field steps

1. Confirm no communication and note cluster, tachometer, and battery behavior.

2. Prove the meter ground at a known B+ source.

3. Back-probe a known three-wire pressure sensor or TPS and identify reference, signal, and ground.

4. If reference is low, key off before unplugging candidate shared-reference sensors. Isolate one at a time and key-cycle where the system may latch reference protection.

5. If 5 V does not return, inspect PCM powers and grounds from the wiring diagram; use accessible fuse outputs before opening difficult PCM connectors.

6. Identify PCM relay load feed and output. Do not accept an unloaded socket reading or a relay click as proof.

7. Load-test the feed and bypass/substitute only with correctly identified terminals and a fused/current-limited tool.

8. Replace or repair the confirmed relay/socket fault and verify stable 5 V, scan communication, and start/run.

## Grab list

Tools: DVOM; wiring information; fine back-probes; scan tool; current-limited relay activator or fused bypass; battery support.

## Safety and escalation

Key off before unplugging sensors; do not short 5 V reference to B+ or spread connector terminals.

## Common misdiagnosis

Testing spark or fuel before resolving no communication/low reference, condemning sensors or PCM, or accepting a clicking relay as healthy.

# 5. Cranking no-start: fuel-delivery branch

Symptom: The engine cranks at a normal cadence but will not start.

Root cause demonstrated: The demonstrated vehicle had spark and injector command and briefly ran on a controlled alternate fuel source. Relay output and pump connector power/ground were good; the pump was internally open. Replacement restored operation and starting.

Decision tree: NORMAL CRANK → prove spark → prove injector command → controlled alternate fuel → pressure if adaptable → load-test pump relay → connector power/ground → pump verdict → confirm pressure/current/start.

## Evidence source

No Start Testing Basics - Spark, Injector Pulse and Fuel Pressure — full timestamped transcript and real-frame inspection completed.

## Ordered field steps

1. Judge cranking cadence and visible mechanical movement. Uneven or abnormally fast cadence exits to compression/timing tests.

2. Prove spark with a safe spark tester or appropriate test-light/scope method.

3. Prove injector command with a noid light, incandescent test light, or scope; identify feed and PCM-controlled sides correctly.

4. When safe, use a small controlled amount of approved alternate fuel. A brief run moves the primary branch to fuel delivery.

5. Check fuel pressure if an adapter is available; lack of an easy test port does not justify guessing.

6. Identify and load-test the pump fuse/relay output. Use a fused/current-limited bypass only after identifying the correct terminals.

7. At the pump connector, protect terminals and verify commanded power plus ground. A lamp across pump power and ground proves the external circuit can supply a small load.

8. Good external circuit plus a silent/open pump confirms the pump. Replace only when site conditions permit safe fuel handling.

9. Verify pump operation, vehicle-specific pressure, reasonable current, start, and restart.

## Grab list

Tools: Spark tester; noid/incandescent test light; DVOM; wiring diagram; fuel-pressure adapters; fused relay tool; optional scope/current clamp; approved alternate fuel; extinguisher and PPE.

## Safety and escalation

Fuel-vapor control, ventilation, spill containment, ignition-source control, and terminal protection are mandatory. Tow or escalate when tank removal/site conditions are unsafe.

## Common misdiagnosis

Replacing a pump before proving spark/injector command, or guessing because the vehicle lacks an easy pressure test port.

# Next coverage lanes

- No injector power / main relay and open-wire location

- Misfire: spark, fuel, or compression triage

- Parasitic draw without module wake-up or fuse-pulling traps

- Real mobile no-crank service-call workflow

- Noise localization, cooling/overheat, ABS/brakes, network/CAN, charging, and advanced scope workflows

# Promotion standard

A process enters Agent Knowledge only when two strong sources agree, or one elite trusted technician demonstrates it end-to-end on camera. Every promoted tree retains source provenance, tool requirements, safety limits, confirmation tests, and tow/escalation conditions.

# Batch 02 — Suspension, brakes, leaks, head gasket and timing

Internal technician knowledge base • Visually verified 2026-08-09

This batch adds eleven evidence-backed field trees. Vehicle-specific service information controls every wear limit, torque, pressure, support point, and timing reference.

## Batch 02 coverage

- Front/rear steering and suspension inspection

- Rear suspension and shock bushing wear

- Shock/strut failure versus installation bind

- Suspension noise localization

- Wheel bearing confirmation

- Complete brake inspection

- Exact coolant leak localization

- Exact engine-oil leak localization with UV

- Head-gasket suspicion test stack

- Mechanical timing despite normal cam/crank correlation

- Dynamic suspension play detector (shop escalation)

## Field rule before any suspension or brake verdict

Reproduce the complaint, secure the vehicle on stable ground, and watch the actual joint or component move. Hand position, sound direction, an ABS code, or visible dampness is a clue—not a final diagnosis.

# 6. Front/rear steering and suspension inspection

Decision tree: Complaint → loaded dry-park → raised 3-and-9/12-and-6 while observing joints → bushing/damper/rear inspection → design-specific ball-joint load → measure → repair before alignment → confirm

Provenance: GMM-EX-006; GMM-EX-009; GMM-EX-010. Elite/pro anchor + two corroborating visuals.

## Evidence source

Source: How To Check Your Steering & Suspension Like A Professional! — Royalty Auto Service

## Ordered field steps

1. Record trigger/impact/tire wear.

2. Loaded dry-park and touch/watch joints.

3. Lift/support; repeat both axes.

4. Pry/compare bushings.

5. Inspect front/rear dampers, mounts, springs and links.

6. Load/unload ball joints per maker.

7. Dial-indicator marginal play.

8. Repair, torque, align if required, recheck/road-test.

## Grab list

Tools: Chocks; rated jack/stands; light; pry bars; helper; chassis ears; dial indicator; service data; torque wrench; PPE

## Safety and escalation

Tow/escalate for unstable ground/support, structural cracks, severe loose safety joint, unavailable spec, or repair beyond mobile support capacity

## Common misdiagnosis

Hand position alone as verdict; raised-only test; cosmetic cracks; skipping rear; aligning before loose parts

## Confirmation standard

Observed relative movement exceeds exact spec; post-repair play gone; torque/alignment correct; complaint route passes

Owner note: Number-one Batch 02 priority; internal field use

# 7. Rear suspension and shock bushing wear

Decision tree: Identify design → secure/support → maker-specific measurement → inspect washers, shock eyes, mounts/metal and true leaks → document/repair/torque/recheck

Provenance: GMM-EX-007; GMM-EX-006. Design-specific visual + professional full-vehicle corroboration.

## Evidence source

Source: How to Check for Wear on Suspension and Shock Bushings — North Central Utility

## Ordered field steps

1. Identify suspension.

2. Secure trailer/vehicle and air system.

3. Measure maker points.

4. Inspect wear washers.

5. Twist-check shock eyes.

6. Inspect bolts/brackets/welds.

7. Separate film from wet run.

8. Document and recheck.

## Grab list

Tools: Chocks; rated stands; light; ruler/gauge; pry bar; torque wrench; service data; PPE

## Safety and escalation

Escalate if unit cannot be immobilized, air suspension cannot be secured, structural metal is cracked, or repair needs heavy equipment

## Common misdiagnosis

Applying 3/4-inch trailer value to cars; mist as leak; missing mounts/bolts; washer wear as main-bushing proof

## Confirmation standard

Measured clearance within exact design limit; no abnormal eye play; mounts intact; no fresh oil run

Owner note: 3/4 inch belongs only to demonstrated trailer design

# 8. Shock/strut failure versus installation bind

Decision tree: Compare signatures → identify oil leak, side-load/bushing twist, boot/rod corrosion, overload/impact or cartridge error → service at maker ride height/torque → confirm travel and ride

Provenance: GMM-EX-008; GMM-EX-006. Manufacturer training + professional inspection anchor.

## Evidence source

Source: Monroe Diagnosis of Shocks & Struts Part II — Garage Gurus

## Ordered field steps

1. Compare sides/ride height.

2. Inspect oil path, paint, mounts, bushings, rod, boot and impact.

3. Diagnose side-loading.

4. Torque bonded bushings at specified ride height.

5. Inspect cartridge retention.

6. Verify travel/road test.

## Grab list

Tools: Light; mirror; ride-height gauge; jack/stands; torque wrench; specialty tools; service data; PPE

## Safety and escalation

Escalate spring/strut disassembly without rated compressor/training or when mount/body structure is damaged

## Common misdiagnosis

Calling installation bind a bad part; cosmetic rust as cause; torque at full droop; impact-gun cartridge service

## Confirmation standard

Correct ride height; no bind; correct torque; no fresh leak/noise; controlled road result

Owner note: Failure-signature supplement

# 9. Suspension noise localization

Decision tree: Exact trigger → controlled route → loaded stationary checks → raised observation and contact evidence → tire/brake/mount/exhaust alternatives → chassis ears → same-route confirmation

Provenance: GMM-EX-010; GMM-EX-006. Strong visual + current professional corroboration.

## Evidence source

Source: How To Find Suspension Noises 101 — EricTheCarGuy

## Ordered field steps

1. Interview.

2. Reproduce one variable at a time.

3. Loaded steering/bounce.

4. Lift/support and observe joints.

5. Inspect full front/rear suspension plus brakes, tires, mounts and exhaust.

6. Use chassis ears if needed.

7. Repair and repeat trigger.

## Grab list

Tools: Chocks; jack/stands; light; pry bar; helper; stethoscope/chassis ears; tread gauge; torque wrench; service data; PPE

## Safety and escalation

Escalate if noise needs unsafe road behavior, structural damage exists, or vehicle cannot be supported stably

## Common misdiagnosis

Replacing customer-indicated side; skipping route; overlooking cargo/brakes/tires/exhaust; hand-position shortcut

## Confirmation standard

Noise gone on identical trigger route; no new pull/warning; fasteners torqued at specified position

Owner note: Complaint-first branch

# 10. Wheel bearing confirmation

Decision tree: Road-speed complaint → tire/hardware check → observed wheel-play split → spin/roughness → ABS live-data/sensor/encoder branch → chassis ears → torque and route confirm

Provenance: GMM-EX-011; GMM-EX-006. Clear visual + professional mechanical corroboration.

## Evidence source

Source: How to Check a Wheel Bearing (Sound, play, ABS light) — ChrisFix

## Ordered field steps

1. Road-test speed/load.

2. Inspect/rotate tires if permitted.

3. Lift/support.

4. Shake both axes while watching joints/hub.

5. Spin/compare.

6. Graph wheel speeds and inspect sensor/harness/encoder.

7. Chassis ears.

8. Repair/torque/clear/retest.

## Grab list

Tools: Chocks; jack/stands; helper; scan tool/ABS live data; chassis ears; dial indicator; torque wrench; service data; PPE

## Safety and escalation

Escalate if bearing is catastrophically loose, axle/hub removal requires unsafe field force/support, or ABS diagnosis remains ambiguous

## Common misdiagnosis

Side by sound alone; no play means good; ABS code means hub; missing tire/brake/joint/sensor fault

## Confirmation standard

Hub signal/roughness/play localized; equal live wheel speeds after repair; route noise and faults gone

Owner note: ABS code is a clue, never proof

# 11. Complete brake inspection

Decision tree: Complaint → hydraulic precheck → all four wheels → line/hose routing → pad/caliper/hardware measurements → rotor/drum/parking brake → contamination and ABS branch → corner report → post-service safety check

Provenance: GMM-EX-012. Strong end-to-end visual; numeric specs gated to current service data.

## Evidence source

Source: How to Perform a Complete Brake Inspection — How to Automotive

## Ordered field steps

1. Road-test.

2. Fluid/master/ABS/line/hose inspection.

3. Support/remove four wheels.

4. Measure pads and rotors; inspect calipers/hardware.

5. Drum/parking brake and contamination source.

6. ABS inspection/live data as needed.

7. Report.

8. After repair torque, bleed, pump pedal, leak-check and road-test.

## Grab list

Tools: ABS scan tool; brake-fluid tester; jack/stands; micrometer; drum gauge; dial indicator; torque wrench; brake tools; PPE

## Safety and escalation

Tow for hydraulic leak, unsafe hose/line, no pedal, severe crack, unstable support, or inability to bleed ABS correctly

## Common misdiagnosis

Noisy axle only; top off without cause; appearance-only rotor verdict; missing inside pad/hose rub/axle-oil contamination

## Confirmation standard

Measurements meet current spec; firm pedal; no leaks/rub; correct wheel speed data; safe road/burnish result

Owner note: Four-corner inspection

# 12. Exact coolant leak localization

Decision tree: Cold system → correct adapter/rating → timed pressure hold → highest-point external search → dye/extended hold → internal branch only if no external source → repair/bleed/repeat

Provenance: GMM-EX-013; GMM-EX-015. Manufacturer-training method + long-form diagnostic corroboration.

## Evidence source

Source: How to Use a Pressure Test to Find a Coolant Leak — Garage Gurus

## Ordered field steps

1. Cool/zero pressure.

2. Identify fluid/baseline.

3. Fit adapter.

4. Pump below exact rating.

5. Record hold.

6. Search top-down.

7. Dye/extended hold.

8. Internal branch if needed.

9. Repair, bleed, cold and hot-cycle retest.

## Grab list

Tools: Pressure tester/adapters; service data; light; mirror/borescope; UV dye/light; scan tool; spill tray; PPE

## Safety and escalation

Tow for active overheat, major loss, contaminated fluids, unsafe pressure behavior, or inaccessible internal testing

## Common misdiagnosis

Hot cap; generic pressure; leaking adapter; following coolant downward; pressure drop equals head gasket

## Confirmation standard

Timed hold stable; no fresh external/internal evidence; level stable after full heat cycle and cool-down

Owner note: Case pressure is not universal

# 13. Exact engine-oil leak localization with UV

Decision tree: Confirm fluid/level → PCV pressure branch → clean/dry → compatible dye/reproduce → highest fluorescent emergence → repair/service-data torque → clean/retest for second leak

Provenance: GMM-EX-014. Strong visual method; torque/spec guarded.

## Evidence source

Source: Oil Leak Diagnosis with UV Dye! — Faye Hadley

## Ordered field steps

1. Identify fluid and safe level.

2. Check PCV/blow-by/overfill.

3. Clean/dry.

4. Add correct dye/reproduce.

5. Scan top-down.

6. Mark highest emergence.

7. Repair proven source.

8. Clean/rerun for second leak.

9. Verify level/no new trace.

## Grab list

Tools: UV dye; UV lamp/glasses; degreaser; light; mirror/borescope; torque wrench; service data; containment/PPE

## Safety and escalation

Tow for unsafe oil level/pressure, heavy leak onto exhaust, inaccessible major seal, or field-cleanup restrictions

## Common misdiagnosis

Replacing lowest wet gasket; runoff as head gasket; wrong/too much dye; no clean/retest; ignoring PCV

## Confirmation standard

No active UV emergence under original condition; correct level; no belt/exhaust contamination

Owner note: Highest leak first

# 14. Head-gasket suspicion test stack

Decision tree: History/bleeding → cold external inspection → correctly executed combustion-gas test → interpret positive as pathway not part → negative remains probabilistic → pressure/extended external search → borescope/compression/leak-down → full bleed/retest

Provenance: GMM-EX-015; GMM-EX-013. Long-form visual + independent pressure-method corroboration.

## Evidence source

Source: Leak Down, Pressure, Chemical Test — Bubbles In Cooling System — Rainman Ray's Repairs

## Ordered field steps

1. Record symptoms/service air.

2. Cold inspect.

3. Warm safely.

4. Draw vapor through fresh reagent.

5. Positive: pinpoint path.

6. Negative: pressure test/hold.

7. Search external highest-first.

8. If none, borescope/compression/leak-down at TDC.

9. Repair/bleed/heat-cycle/cool/retest.

## Grab list

Tools: Pressure tester; block-test kit; closed adapter/funnel; scan tool; borescope; UV; compression/leak-down tools; regulated air; service data/PPE

## Safety and escalation

Tow for active overheat, hydro-lock risk, contaminated oil, severe pressure from cold, or advanced leak-down/teardown beyond field capability

## Common misdiagnosis

Bubbles alone; coolant-contaminated reagent; negative is absolute; service air as combustion; teardown before external leak search

## Confirmation standard

Repeatable test result plus located path; pressure stable and level stable after correct bleed/full cool-down

Owner note: Test stack reduces probability; no single-test verdict

# 15. Mechanical timing despite normal cam/crank correlation

Decision tree: Low power/low vacuum → basic restriction and VVT checks → in-cylinder exhaust/intake/running-vs-cranking analysis → valve-event timing versus known-good → cam/crank → shared mechanical reference-shift branch → converge before teardown → recapture after repair

Provenance: GMM-EX-016. Elite instructor end-to-end advanced diagnosis.

## Evidence source

Source: Low Power Diagnosis: Good Cam/Crank Doesn’t Mean Good Timing | Part 2 — ScannerDanner

## Ordered field steps

1. Reproduce under load.

2. Rule fuel/ignition/intake/exhaust basics.

3. Capture cranking/running in-cylinder pressure with sync.

4. Evaluate exhaust stroke/vacuum/filling.

5. Measure EVO/IVC against known-good.

6. Compare cam/crank.

7. If signals agree but piston events are late, inspect shared keyway/gear/reluctor reference.

8. Cross-check spark/VVT/VIN data.

9. Teardown only after convergence.

10. Recapture/road-test.

## Grab list

Tools: Multi-channel scope; pressure transducer; sync; vacuum/backpressure gauges; scan tool; compression/leak-down; known-good/service data; PPE

## Safety and escalation

Advanced escalation when waveform competence, known-good data, engine disable procedure or safe mechanical access is unavailable

## Common misdiagnosis

Converter from feel; normal cam/crank proves timing; brake booster despite low engine vacuum; teardown from generic recall

## Confirmation standard

Post-repair in-cylinder valve events, vacuum, cam/crank, power and brake assist all normal

Owner note: Timing-chain-to-keyway escalation tree

# 16. Dynamic suspension play detector (shop escalation)

Decision tree: Marginal manual finding → approved wheel-load detector → secure/lift-lock → controlled multi-axis load while observing → spec comparison → manual corroboration

Provenance: GMM-EX-009; GMM-EX-006. Tool visual + professional manual anchor.

## Evidence source

Source: Performing a ball joint and silent block diagnosis with the Monroe´s KSWL01 suspension tool — Garage Gurus

## Ordered field steps

1. Verify tool/vehicle rating.

2. Secure vehicle.

3. Position without full vehicle weight on tool.

4. Apply small loads.

5. Observe joints from protected zone.

6. Stop on slip/overload.

7. Compare spec and corroborate manually.

## Grab list

Tools: Approved play detector; locked lift; second technician; light; dial indicator; service data; PPE

## Safety and escalation

Always escalate from mobile service; exact method requires shop lift and approved detector

## Common misdiagnosis

Elastic bushing motion as failure; tool carries vehicle; dynamic movement without spec; hands in pinch path

## Confirmation standard

Observed repeatable relative movement outside current spec and corroborated manually

Owner note: Shop-only branch

# Batch 02 completion status

Eleven videos were downloaded through the authorized YouTube session, structurally scanned, visually sampled from real frames, and converted into Process Extracts GMM-EX-006 through GMM-EX-016 and Agent Knowledge GMM-KB-006 through GMM-KB-016.

The suspension/brake and leak/head-gasket/timing priorities are now covered at first-pass manual depth. Future batches should add make-specific wear specifications, alignment diagnosis, collision/front-end structural triage, brake hydraulics/ABS case studies, and timing-chain confirmation on additional engine families.

# 17. No-start, cranks normally — no injector power (main relay/PCM control circuit)

Decision tree: No-start, cranks normally → confirm spark present → check fuel pressure → backprobe injector control pulse vs power feed → if power missing, check fuses, then relay power-feed pins, then relay computer-control pin → trace and repair open/chafed control wire → confirm start

Provenance: GMM-EX-017. Elite single-source end-to-end demonstration (ScannerDanner).

## Evidence source

Source: No Start, Engine Cranks Okay, Troubleshooting With Basic Tools (No Power to Injectors) — ScannerDanner

## Ordered field steps

1. Test-light spark check.

2. Fuel pressure gauge check.

3. Backprobe injector control pulse.

4. Backprobe injector power feed.

5. Trace wiring diagram if power missing.

6. Check fuses.

7. Check relay power-feed pins.

8. Check relay computer-control pin for ground signal.

9. Inspect harness at chafe/contact points; repair and confirm start.

## Grab list

Tools: Incandescent test light (not LED); fuel pressure gauge; backprobe/terminal test kit; wiring diagrams; heat-shrink connectors; optional relay-bypass tool

## Safety and escalation

Never use an LED test light on ignition circuits. Escalate if the break is inside an inaccessible harness run or requires body/interior teardown beyond a mobile appointment.

## Common misdiagnosis

Replacing the relay or PCM from power-feed-only testing without isolating the control/ground side

## Confirmation standard

Engine starts and runs normally; relay power-feed and control-pin voltages re-check normal under crank

Owner note: Core no-start relay/control-circuit tree

# 18. Misfire triage — spark vs fuel vs compression

Decision tree: Misfire/P030x code → quick screens (cranking compression sound, vacuum gauge at idle) → isolate cylinder via test-light coil-primary drop test → if mechanical suspected, relative-compression current-ramp test synced to a companion cylinder → compare waveforms to localize the weak cylinder

Provenance: GMM-EX-018. Elite single-source end-to-end demonstration (ScannerDanner).

## Evidence source

Source: Misfire Quick Tests (Is it a spark, fuel or compression problem?) — ScannerDanner

## Ordered field steps

1. Cranking compression sound test.

2. Vacuum gauge at idle.

3. Test-light coil-primary drop test per cylinder.

4. Relative-compression current-ramp test (inductive clamp + scope) if mechanical suspected.

5. Add a sync channel on a companion cylinder to landmark waveform humps.

6. Compare cylinder waveforms to confirm the weak cylinder.

## Grab list

Tools: Vacuum gauge; incandescent test light (mobile-feasible screen); lab scope, inductive current clamp, secondary-ignition pickup (shop-tier mechanical branch)

## Safety and escalation

Live HV secondary ignition while cranking — avoid contact with plug wires/coil towers. Escalate to shop/lift for valve-cover removal or a leak-down test once a cylinder is isolated.

## Common misdiagnosis

Replacing coils/plugs/wires on a mechanical misfire with no cylinder-isolation test performed first

## Confirmation standard

Repeat waveform/gauge tests confirm the same cylinder; teardown or leak-down test confirms the mechanical fault before repair

Owner note: Mechanical-suspicion branch requires shop-tier scope/clamp gear; the spark/fuel screening steps alone are mobile-feasible with basic tools

# 19. Parasitic battery draw — fuse voltage-drop isolation

Decision tree: Battery dies overnight → confirm/load-test battery → prep vehicle as customer left it, let modules sleep 1–2 hrs → ammeter in series on negative, clamp-check load first → read settled draw → probe mV voltage-drop across each fuse without pulling → confirm with a single pull of the suspect fuse only → trace circuit on wiring diagram → confirm exact component via scan-tool live data → unplug component, re-verify draw returns to normal

Provenance: GMM-EX-019; GMM-EX-020. 2 strong independent sources agree on the core method (fuse mV voltage-drop, not blind fuse-pulling).

## Evidence source

Sources: Battery Dies Everyday: Testing For Parasitic Draw — South Main Auto LLC; The BEST Way TO Perform a Parasitic Draw Test — HumbleMechanic

## Ordered field steps

1. Load-test/replace the battery if marginal.

2. Prep vehicle as customer left it; latch doors/hood/trunk; let sleep 1–2 hrs.

3. Ammeter in series on negative; clamp-check load first.

4. Read draw, let it settle.

5. Rule out obvious accessories (OBD dongles, etc).

6. Probe mV voltage-drop across each fuse’s contacts without pulling; convert via fuse chart if needed.

7. Confirm with a single physical pull of the suspect fuse only.

8. Trace circuit on wiring diagram to downstream components.

9. Confirm exact component via scan-tool live data and behavioral clues.

10. Unplug suspect component; re-verify draw returns to normal resting current.

## Grab list

Tools: Series-capable graphing DMM (replaceable fuse); second DMM (mV DC); amp clamp (pre-check only); fuse voltage-drop chart; scan tool; wiring diagrams; door-trim tools

## Safety and escalation

Never pull fuses one at a time to hunt the draw — can wake/reset other modules sharing wake-up lines and false the reading. A full battery disconnect resets the sleep timer. Escalate if the sleep-wait plus disassembly exceeds a single mobile visit, or the fault traces into a module needing dealer reprogramming.

## Common misdiagnosis

Chasing the first plausible accessory (e.g. an aftermarket OBD dongle) instead of following the meter/scan-tool data

## Confirmation standard

Draw settles to a normal resting current (typically tens of mA) immediately after the suspect component is unplugged

Owner note: South Main Auto — real solved customer case, component-level fix confirmed. HumbleMechanic — teaching demo, adds the chart-conversion technique and sleep-prep protocol.

# Batch 03 completion status

Five videos were processed via the public YouTube route (no authenticated session used) into Process Extracts GMM-EX-017 through GMM-EX-021 and Agent Knowledge GMM-KB-017 through GMM-KB-019. This closes out Priority 1 — all foundation sources (voltage drop, no-start tree, misfire triage, parasitic draw, mobile service call) are now Extracted.

One source (GMM-EX-021, a riding lawn tractor no-crank call) was not promoted — off-scope equipment, not a car. It leaves a real coverage gap: a genuine mobile no-crank car service call demonstrating a tow/escalate decision point, to be filled by a later batch. Two source videos assumed at seeding time to demonstrate different draw-test methods (inline ammeter vs. amp clamp) turned out on inspection to both use the fuse voltage-drop technique — merged into one Agent Knowledge tree (GMM-KB-019) citing both sources rather than kept as separate contested methods.

# 20. Misfire — internally shorted ignition coil (current-ramp secondary-winding ID)

Decision tree: Confirmed misfire on one cylinder → test-light spark check at each coil → identify ignition design (waste-spark vs discrete coils) → back-probe coil-control signal for clean square wave → current-ramp test on the coil's primary power feed (not ground) → compare waveform shape/KV against a known-good companion cylinder → shorted-secondary shows a curved/slow ramp vs a good coil's sharp ramp → replace coil → full waveform/KV/current-ramp/codes/fuel-trim reverification

Provenance: GMM-EX-022. Elite single-source end-to-end demonstration (ScannerDanner).

## Evidence source

Source: Every Auto Technician Needs To Learn This! (Misfire Diagnostics) — ScannerDanner

## Ordered field steps

1. Test-light spark check per cylinder.

2. Identify ignition design via wiring diagram.

3. Back-probe coil-control signal, confirm clean square wave.

4. Current-ramp test on the coil's primary power feed.

5. Compare ramp shape/KV against a known-good companion cylinder.

6. Confirm plug wire is not the fault.

7. Replace coil/plug as indicated.

8. Full after-repair reverification: waveform, KV, current-ramp, event-sync, codes, O2, fuel trim.

## Grab list

Tools: Lab scope with inductive coil-on-plug pickup; incandescent test light; low-amp clamp probe; DVOM; scan tool with live data and wiring diagrams

## Safety and escalation

On Euro-style coils, probing the accessible ground point instead of the primary power feed gives a false current-ramp trace — that ground is the secondary-winding ground. Escalate if ignition architecture can't be positively identified from available diagrams.

## Common misdiagnosis

Replacing a coil from a static resistance check alone, without a current-ramp/waveform comparison against a known-good cylinder

## Confirmation standard

Repaired cylinder's waveform shape and KV match known-good companion cylinders; codes clear; fuel trim returns to normal

Owner note: Distinct from tree 18 (spark/fuel/compression triage) — this tree is for confirmed-spark-present misfires where the coil itself is suspected of an internal short

# 21. No-start — prove fuel delivery before replacing parts (parts-cannon prevention)

Decision tree: No-start after prior unsuccessful repairs → do not repeat prior parts-cannon replacements → verify crank signal → check codes without chasing unrelated/no-parts-applicable codes → confirm spark present → added-fuel start test → scope injector pulse → read actual fuel rail pressure → if abnormal, test pump current (steady-state vs inrush) to localize wiring vs pump vs delivery-path fault → physically inspect the fuel delivery path → repair → re-verify with a physical leak check under running conditions

Provenance: GMM-EX-024. Elite single-source end-to-end demonstration (ScannerDanner).

## Evidence source

Source: 3 ECMs and a Boatload of Parts… Can We Fix This No Start?? — ScannerDanner

## Ordered field steps

1. Verify crank signal present.

2. Check codes; do not chase codes explicitly marked no-parts-apply in service info.

3. Confirm spark present.

4. Added-fuel start test to isolate fuel vs ignition.

5. Scope injector pulse for a real pintle-open hump.

6. Read actual fuel rail pressure.

7. If abnormal, clamp pump current: compare steady-state draw to inrush — normal inrush rules out wiring/relay as primary fault.

8. Physically inspect the fuel delivery path for a real, visible defect.

9. Repair.

10. Re-verify by visually re-inspecting every touched connection with the pump running, not just a gauge reading.

## Grab list

Tools: Scan tool with bidirectional control and live PID graphing; adjustable spark tester; starting fluid; lab scope; low-amp clamp; back-probe pins; tank-drop hand tools (shop/lift-only for physical R&R)

## Safety and escalation

Escalate/tow for any step requiring tank drop or lift access. Fuel system work requires standard fuel-safety practice.

## Common misdiagnosis

Replacing ECMs, modules, or other expensive parts based on a code or a guess without first proving which system (spark/fuel/mechanical) is actually at fault with real data

## Confirmation standard

Fuel pressure reads normal, pump current inrush/steady-state normal, vehicle starts and runs, and every touched connection is visually re-inspected with the pump running (no leak)

Owner note: The transferable part is the sequence and the current-ramp inrush-vs-steady-state logic, not this vehicle's specific numbers or DTC

# 22. Intermittent electronic-throttle / reduced-power fault — capture-during-event + pin-drag isolation

Decision tree: Intermittent reduced-power/MIL fault clears on key cycle, live data looks clean after the fact → capture a full scan during an actual active fault event → pull the OEM TSB for the exact code combination to target a likely connector → check pro diagnostic forums for a known fix → pull wiring diagrams to identify every connector in the circuit → visually inspect for corrosion (may be clean) → pin-drag/tug-test every terminal in the suspect connector → re-tension or repair any terminal with abnormal retention → apply contact enhancer → clear codes and monitor for recurrence

Provenance: GMM-EX-026. Elite single-source demonstration; outcome unconfirmed in source video.

## Evidence source

Source: 2019 Buick Enclave Reduced Power Mode | Intermittent Throttle Pedal Nightmare — South Main Auto LLC

## Ordered field steps

1. Capture full scan tool data during an actual active fault event.

2. Pull the OEM TSB for the exact code combination to target a likely connector.

3. Check pro diagnostic forums for a known fix.

4. Pull OEM wiring diagrams to identify every connector in the circuit.

5. Visually inspect connectors for corrosion (may show nothing).

6. Pin-drag/tug-test every terminal in the suspect connector with a dedicated pin tester.

7. Re-tension or repair any terminal with abnormal (near-zero) retention.

8. Apply a contact enhancer.

9. Clear codes, reassemble, and set a monitoring/re-check plan.

## Grab list

Tools: Handheld/phone-based scan tool; OEM wiring diagrams and TSB access; terminal pin-drag tester; trim pry tools; contact enhancer (e.g. Stabilant 22A)

## Safety and escalation

Escalate if the fault cannot be captured live, or if pin-drag testing finds no abnormal terminal — points toward a PCM-internal or software cause needing dealer-level diagnosis.

## Common misdiagnosis

Chasing an intermittent electronic-throttle fault with parts replacement before doing a connector-level pin-drag test, when live data looks normal every time it's checked after the fault clears

## Confirmation standard

IMPORTANT: this specific repair's outcome was NOT confirmed on camera — the source video ended before the fault could be shown as resolved. Confirm by monitoring for recurrence over multiple drive cycles/weeks before treating any single repair under this tree as verified.

Owner note: First manual entry for electronic-throttle/reduced-power faults and for a general intermittent-fault-capture technique. Promoted for the method, not as a guaranteed fix.

# 23. No-comm / dim-lights combo — scope-confirm a shared ground before parts-cannon

Decision tree: Multiple modules show no-communication AND lights/accessories appear dim → treat as a combined clue pointing to a shared ground → scope the data bus at the OBD connector, an elevated non-pulsing trace indicates a lost ground → pull wiring diagram to find the shared ground point → back-probe an affected module's ground pin and jumper to a known-good ground while scoping, instant waveform correction confirms the fault before disassembly → independently verify any other affected circuit the same way → physically locate and inspect the ground stud(s) → repair → confirm via re-scan and function checks

Provenance: GMM-EX-027. Elite single-source end-to-end demonstration (South Main Auto).

## Evidence source

Source: It Was Just Here For An Oil Change… Then EVERYTHING Went Dead! — South Main Auto LLC

## Ordered field steps

1. Recognize the combined clue: multi-module no-comm plus dim lights/accessories.

2. Scope the data bus at the OBD connector; elevated non-pulsing trace = lost ground, not a data fault.

3. Pull wiring diagram to find the shared ground point.

4. Back-probe an affected module's ground pin, jumper to known-good ground while scoping.

5. Independently verify any other affected circuit, tracing to its own ground point — do not assume one bad ground explains every symptom.

6. Physically locate and inspect the ground stud(s) for corrosion/damage.

7. Repair: clean, re-crimp or replace the ring terminal, heat-shrink.

8. Confirm via re-scan for communication and function-check every previously-affected circuit.

## Grab list

Tools: Scan tool with oscilloscope function; back-probe pins; test light; wiring-diagram subscription; crimper, wire strippers, heat-shrink terminals and heat source

## Safety and escalation

Escalate if the affected ground point is structurally inaccessible without significant disassembly, or if scoping shows the bus itself (not a ground) is the actual fault.

## Common misdiagnosis

Condemning individual modules or the data bus itself, or blanket-cleaning every ground on the vehicle, without scoping to prove which specific ground and circuit is affected

## Confirmation standard

Data bus waveform reads normal after repair; all previously no-comm modules respond; all previously dim circuits read full brightness/voltage

Owner note: A no-comm-plus-dim-lighting combo is a fast, recognizable pattern worth teaching before assuming a data-bus or module failure

# 24. Verify, don't trust — re-diagnosing after a prior shop's failed repair

Decision tree: Vehicle arrives with a complaint a prior shop claimed to have fixed or diagnosed → do not trust the prior shop's stated test results without re-verifying → physically re-confirm the complaint → pull OEM wiring diagram/circuit specs before probing → re-test every claimed-good circuit/component from scratch → for any persistent DTC, clear codes and any required relearn, then re-check → a code that returns after a proper clear-and-relearn is real, current, not stale → cross-reference the exact DTC/symptom in OEM service info before ordering any part

Provenance: GMM-EX-028. Elite single-source end-to-end demonstration (South Main Auto).

## Evidence source

Source: Another Shop Couldn't Fix This Chevy… Here's Why! — South Main Auto LLC

## Ordered field steps

1. Physically re-confirm the complaint yourself.

2. Pull OEM wiring diagram/circuit specs before probing.

3. Re-test every circuit or component the prior shop claimed was good or fixed, from scratch.

4. For any persistent code, clear it and perform any required relearn/calibration, then re-scan.

5. A code that returns after a proper clear-and-relearn is a real, current fault.

6. Cross-reference the exact DTC and vehicle in OEM service info before ordering or recommending any part.

## Grab list

Tools: Heavy-duty test light; bi-directional scan tool with special-functions/relearn capability; OEM service-info subscription; basic hand/trim tools as needed

## Safety and escalation

N/A — this is a diagnostic-discipline entry, not a single-fault escalation gate; standard tow/escalate rules apply per the specific fault found.

## Common misdiagnosis

Accepting a prior shop's or customer's claim that a circuit was already tested/fixed without independently re-verifying it — the single most common way a second shop wastes time on a comeback vehicle

## Confirmation standard

Independently confirmed fault location for every original complaint; a cleared-and-relearned code that does not return; OEM service info supports the diagnosis before parts are ordered

Owner note: General-purpose comeback/second-opinion diagnostic discipline, not tied to any one fault type

# Batch 04 completion status

Eight videos were processed via the public YouTube route (no authenticated session used) into Process Extracts GMM-EX-022 through GMM-EX-029 and Agent Knowledge GMM-KB-020 through GMM-KB-024. This clears the remainder of Priority 2's ScannerDanner and South Main Auto sources.

Three sources (GMM-EX-025, GMM-EX-029) duplicated existing trees (misfire triage GMM-KB-018, relay isolation GMM-KB-017) and were not separately promoted; their genuinely new details (an RPM-bump mechanical-fault heuristic, and a GM relay bypass pinout plus a known-good-swap caution) were folded into the existing entries' owner notes instead of creating redundant trees. One promoted entry (GMM-KB-022, intermittent throttle/reduced-power) carries an explicit caveat that the source video's specific repair was not confirmed on camera — the technique is promoted, not a guaranteed fix.

# 25. Body-control-module intermittent output failure (CAS/BCM-style relay chatter)

Decision tree: Buzzing/clicking relay(s) plus intermittent lighting or wake-up faults → confirm powers/grounds at the module connector are good → back-probe the module's OWN output pin driving the affected relay while reproducing the fault → if it flickers in sync with the symptom, the module's output itself is erratic → force the output on directly to prove the downstream relay responds correctly to a clean signal → disconnect input switches feeding the module and retest → if the same fault returns with inputs disconnected, the fault is isolated to the module itself → condemn and replace

Provenance: GMM-EX-030. Elite single-source end-to-end demonstration (Pine Hollow); outcome independently confirmed correct by owner follow-up.

## Evidence source

Source: No-Crank...Relays BUZZ...Lights Flash...4x4 MALFUNCTION? (2011 BMW 328xi CAS-VTG) — Pine Hollow Auto Diagnostics

## Ordered field steps

1. Confirm powers and grounds at the module connector with a test light.

2. Back-probe the module's own output pin driving the affected relay while reproducing the fault.

3. A flickering test light in sync with the symptom proves the module's output is erratic.

4. Force the output on directly to prove the downstream relay responds correctly to a clean signal.

5. Disconnect input switches feeding the module and retest.

6. If the same fault returns with inputs disconnected, the fault is isolated to the module itself.

7. Condemn and replace the module (dealer-level part/programming on many platforms).

## Grab list

Tools: Bidirectional scan tool with live data graphing; test light; OEM wiring diagrams

## Safety and escalation

Escalate/refer to dealer if the replacement module requires manufacturer-level security programming that mobile equipment cannot perform.

## Common misdiagnosis

Chasing the downstream relay, fuse box, or an input switch when back-probing the module's own output with inputs disconnected proves the module itself is at fault

## Confirmation standard

Clean, non-flickering output signal at the module's output pin after replacement; no return of the fault code after clear-and-retest

Owner note: Distinct from tree 17 (relay ground/control/load isolation) — that tree isolates a fault between a good module and a bad relay; this tree isolates a fault to the module's own internal logic after ruling out everything up- and down-stream of it

# 26. AWD/xDrive transfer-case malfunction light — cross-check DSC-stored code against the transfer-case module itself

Decision tree: AWD/4x4/xDrive malfunction light while driving → scan the full vehicle, not just the module showing the warning → check whether the transfer case (VTG) module itself has ANY stored fault → if VTG shows zero faults while DSC shows a transfer-case code, treat as a logic/load-related signal, not confirmed hardware failure → road-test while monitoring the transfer case's own live data (clutch torque, servo current, angle) → a current spike/over-current trip under load points to the actuator/clutch draw → recommend OEM parts since aftermarket actuators are a plausible unconfirmed cause

Provenance: GMM-EX-030. Elite single-source end-to-end demonstration (Pine Hollow); outcome independently confirmed correct by owner follow-up.

## Evidence source

Source: No-Crank...Relays BUZZ...Lights Flash...4x4 MALFUNCTION? (2011 BMW 328xi CAS-VTG) — Pine Hollow Auto Diagnostics

## Ordered field steps

1. Scan the full vehicle, not just the module showing the dash warning.

2. Check the transfer case (VTG) module specifically for its own stored faults.

3. If DSC/ABS shows a transfer-case code but VTG shows none, do not treat the DSC code alone as confirmation.

4. Reset/recalibrate the transfer case control unit and confirm the calibration completes at idle.

5. Road-test while monitoring transfer-case live data: clutch torque, servo current, servo angle.

6. Reproduce under gentle acceleration while watching current consumption for a spike/over-current trip.

7. Cross-reference prior repair history for non-OEM parts that could explain an over-current pattern.

## Grab list

Tools: Bidirectional scan tool with live data graphing on the transfer-case module; road test capability

## Safety and escalation

Escalate/refer to dealer or specialist shop for the actual transfer-case or servo-motor replacement — this tree is diagnosis-only.

## Common misdiagnosis

Recommending or authorizing a full transfer-case replacement based solely on a DSC/ABS-side stored code without confirming a corresponding fault in the transfer-case module itself

## Confirmation standard

Transfer case module continues to show zero stored faults across multiple drive cycles after any repair, with clutch torque and servo current tracking normally under load

Owner note: First manual entry for AWD/xDrive transfer-case fault triage; the DSC-code-without-VTG-code cross-check likely generalizes beyond BMW

# 27. Parasitic draw — aftermarket accessory isolation (with immobilizer-splice safety check)

Decision tree: Elevated draw on a vehicle with any aftermarket electrical accessory → measure baseline draw with everything installed → identify the accessory as a candidate and estimate its typical at-rest draw → unplug the accessory's connectors without cutting factory wiring → remeasure draw → a substantial drop confirms the accessory → before declaring done, verify the vehicle still starts, since some installers splice into safety-relevant factory circuits (immobilizer/antenna) rather than piggybacking → repair only that specific spliced wire if found

Provenance: GMM-EX-031. Elite single-source end-to-end demonstration (Pine Hollow).

## Evidence source

Source: Nissan Battery DRAIN...& Subaru Window GLITCH? (2016 Frontier, 2019 Outback) — Pine Hollow Auto Diagnostics

## Ordered field steps

1. Measure baseline parasitic draw with everything installed, letting it settle.

2. Identify any aftermarket accessory as a candidate and note its typical at-rest draw.

3. Trace the accessory's wiring to confirm piggybacked vs spliced into factory circuits.

4. Unplug the accessory's connectors (not cutting wires) and remeasure the draw.

5. A substantial drop confirms the accessory as the dominant draw source.

6. Reconnect the battery and attempt a normal start BEFORE considering the job done.

7. If the vehicle fails to start or shows an immobilizer fault, identify the specific spliced wire.

8. Repair only that specific wire while leaving the rest of the accessory disconnected.

9. Confirm normal start and hand the accessory's fate to the customer.

## Grab list

Tools: In-series ammeter/DVM with milliamp resolution; basic hand tools; wire strippers; solder/crimp connectors and heat-shrink

## Safety and escalation

Unplugging an accessory without checking whether the vehicle still starts afterward — some installs splice directly into the factory immobilizer antenna or ignition-coil signal wire, disabling the vehicle even though the draw diagnosis was correct.

## Common misdiagnosis

Treating a parasitic-draw fix as complete once the draw number looks good, without verifying the vehicle still starts

## Confirmation standard

Draw drops to a normal resting level (typically under ~20mA) with the accessory unplugged, AND the vehicle starts normally with no new fault

Owner note: Distinct from tree 19 (fuse voltage-drop isolation of a factory circuit) — this tree is specifically for tracing a draw to an AFTERMARKET accessory and safely removing it

# 28. Power window no-response — initialize before condemning switches or wiring

Decision tree: Power window doesn't respond to a master or door switch, or auto up/down doesn't work → recognize the system is typically standalone per door and doesn't need a scan tool → ask/check whether the window has been initialized since any battery disconnect → run the hold-up/hold-down initialization procedure → retest local and master switches → if it now responds correctly, the complaint was initialization, not hardware → if the window reverses direction unexpectedly during auto-up with nothing obstructing it, treat that as a failing position/anti-pinch sensor in the regulator, not a switch fault

Provenance: GMM-EX-031. Elite single-source end-to-end demonstration (Pine Hollow); anti-pinch signature corroborated against a separately confirmed prior case.

## Evidence source

Source: Nissan Battery DRAIN...& Subaru Window GLITCH? (2016 Frontier, 2019 Outback) — Pine Hollow Auto Diagnostics

## Ordered field steps

1. Recognize the power-window system as standalone per door; a scan tool is not useful here.

2. Ask/check whether the window has been initialized since any battery disconnect or module work.

3. Run the window's specific hold-up/hold-down initialization procedure.

4. Retest from both the local and master switch.

5. If it now responds correctly including auto up/down, the complaint was initialization, not hardware.

6. During auto-up testing, watch for the window reversing with nothing obstructing it.

7. An unexplained reversal points to a failing position/anti-pinch sensor in the regulator/motor.

8. Confirm switches respond and initialize correctly, isolating any remaining fault to the regulator/motor.

9. Recommend regulator/motor replacement if the reversal signature is present; re-initialize after replacement.

## Grab list

Tools: None beyond the vehicle's own factory switches — no scan tool or meter required

## Safety and escalation

N/A — the regulator/motor replacement itself is standard mechanical work, not an escalation trigger.

## Common misdiagnosis

Chasing wiring or switch replacement before running the initialization procedure — a large fraction of window complaints are simply uninitialized systems

## Confirmation standard

Window opens/closes fully and smoothly via auto up/down from both switches, with no unexplained reversal

Owner note: First power-window entry in this manual; the initialization-first step alone likely resolves a large fraction of after-battery-disconnect window complaints without parts

# 29. Intermittent single-cylinder misfire that evades compression/injector/vacuum tests — exhaust pressure-pulse escalation

Decision tree: Idle-only intermittent misfire, smooth under load, ignition ruled out → rule out vacuum leaks and injector flow discrepancy → if compression tests are inconclusive because the fault is too intermittent to catch, escalate to a tailpipe exhaust pressure-pulse test synced to the suspect cylinder's coil firing event → a vacuum pocket instead of a positive pulse at that exact firing event is an unambiguous misfire signature → optionally borescope for visual correlation → report with an honest confidence level

Provenance: GMM-EX-032. Elite single-source end-to-end demonstration (Pine Hollow); technician's own confidence-bounded conclusion preserved.

## Evidence source

Source: P0300 Mystery MISFIRE - Difficult to Diagnose! (Intermittent FISHBITE) — Pine Hollow Auto Diagnostics

## Ordered field steps

1. Confirm ignition is already ruled out.

2. Rule out vacuum leaks with a spray test and/or smoke machine.

3. Rule out injector flow discrepancy with an electronic pulse/flow tester.

4. If compression testing is inconclusive, install a pressure/vacuum transducer loosely in the tailpipe.

5. Sync a second scope channel to the suspect cylinder's ignition coil firing event.

6. Watch for a vacuum pocket instead of the normal positive exhaust pulse, exactly in sync with that firing event.

7. Log the recurrence pattern (fixed interval vs random) as additional diagnostic context.

8. Optionally borescope the cylinder's valve for visual correlation.

9. Report the isolated cylinder with an honest confidence level; don't overstate certainty about the mechanism.

## Grab list

Tools: In-cylinder pressure transducer + two-channel lab scope; smoke machine; electronic fuel-injector pulse/flow tester; tailpipe pressure/vacuum transducer; borescope; remote starter/jumper wire

## Safety and escalation

Escalate to engine teardown/valve job only once the customer accepts the vehicle is otherwise safe to keep driving and the fault is monitored over time.

## Common misdiagnosis

Quoting a customer for engine teardown based on one short capture that happened not to catch a genuinely intermittent fault

## Confirmation standard

A clear vacuum-pocket signature in the tailpipe trace exactly synced to the suspect cylinder's firing event, ideally observed across multiple sessions given the fault's irregular recurrence

Owner note: Complements tree 18 (spark/fuel/compression triage) and tree 20 (shorted-coil ID) for cases where those standard tests come back clean

# 30. No-crank on dual-battery/auxiliary-support-battery vehicles — trace upstream, then load-test the aux battery

Decision tree: No-crank with cluster power-down/tach spike during cranking, main battery and main ground both test good → check a hot-at-all-times reference point (e.g. DLC power pin) for a flicker during cranking → trace that supply upstream fuse-by-fuse via powers-and-grounds diagrams → if the trace leads to a secondary auxiliary/support battery circuit via a Power Control Relay, load-test that battery directly rather than trusting static voltage → a battery that reads normal resting voltage but collapses under load is the root cause, even with zero codes

Provenance: GMM-EX-033. Elite single-source end-to-end demonstration (Super Mario Diagnostics); root cause independently confirmed by direct load test and full vehicle repair.

## Evidence source

Source: Chasing a No-Crank All the Way Through the Vehicle's Electrical Maze (2020 Jeep Compass TrailHawk) — Super Mario Diagnostics

## Ordered field steps

1. Duplicate the no-crank complaint and note the exact symptom.

2. Rule out the main starting battery with a proper load/CCA/reserve-capacity test plus a jump-box cross-check.

3. Rule out the main engine/block ground with a test light while cranking.

4. Scan for codes but treat networked-module codes as likely downstream symptoms until proven otherwise.

5. Check a hot-at-all-times reference point (e.g. DLC power pin) with a test light while cranking, checking both power and ground sides.

6. If a flicker is present, trace that circuit upstream fuse-by-fuse via powers-and-grounds diagrams.

7. If the trace leads to a secondary/auxiliary support-battery circuit via a Power Control Relay, don't assume the main battery circuit is involved.

8. Load-test the auxiliary battery directly rather than relying on static voltage.

9. A battery that reads normal resting voltage but collapses under load is confirmed bad regardless of codes.

10. Replace the auxiliary battery and confirm reliable starting.

## Grab list

Tools: Multimeter/DVOM, test light, jump box, battery tester (CCA/reserve capacity), scan tool with OEM wiring/powers-and-grounds diagrams, simple load (headlamp bulb) for battery load-testing

## Safety and escalation

N/A for the diagnosis itself; standard battery replacement is not an escalation-requiring repair.

## Common misdiagnosis

Trusting a battery's static/unloaded voltage as proof of good condition on ANY battery, especially a secondary/auxiliary support battery that may not set a diagnostic code when it fails; chasing networked-module DTCs as if the modules themselves are at fault

## Confirmation standard

Auxiliary/support battery holds a stable voltage under a real load; vehicle starts reliably across multiple attempts with no cluster power-down or tach spike

Owner note: First manual entry for dual-battery/auxiliary-support-battery no-crank architecture; the load-test-not-static-voltage principle generalizes to any battery diagnosis

# 31. Partial single-cylinder misfire — borescope tiebreaker for valve vs lower-end (cylinder wall scoring)

Decision tree: Non-dead single-cylinder misfire, ignition and fuel delivery ruled out → run relative-compression current-ramp test to screen for a mechanical discrepancy → if a PARTIAL (not zero) compression discrepancy is found, do not assume a valve problem by default → borescope through the spark plug hole, rotating the engine through a full revolution, to visually inspect the cylinder wall → scoring/discoloration confirms a lower-end cause rather than a valve cause → explain any lean fuel-trim shift as blow-by through the PCV system

Provenance: GMM-EX-034. Elite single-source end-to-end demonstration (Super Mario Diagnostics).

## Evidence source

Source: Mazda Misfire - IN REAL TIME — Super Mario Diagnostics

## Ordered field steps

1. Rule out ignition (coil-disconnect logic) and fuel delivery (fuel trim shift not large enough) first.

2. Check secondary ignition/COP burn time against a known-good cylinder.

3. Run a relative-compression current-ramp test early, not as a last resort.

4. Attempt to sync the capture to the specific cylinder; corroborate with misfire-counter data if sync fails.

5. When the discrepancy is partial, reason through valve/lash vs lower-end/rings as the two main categories.

6. Remove the spark plug and insert a borescope, rotating the engine through a full revolution.

7. Visible scoring or discoloration confirms a lower-end cause; a clean crosshatch points away from it.

8. Explain any lean fuel-trim shift as blow-by recirculated via the PCV system.

9. Report plainly that a scored cylinder wall on a high-mileage engine typically means engine replacement, not a top-end repair.

## Grab list

Tools: Scan tool with live data/misfire counters/secondary-ignition COP waveform capture; inductive amp clamp (2000A-class) and lab scope; borescope with adequate reach and lighting; basic hand tools

## Safety and escalation

N/A for the diagnosis itself; a confirmed scored cylinder is an engine-replacement-level finding, beyond mobile/field repair scope.

## Common misdiagnosis

Assuming any single-cylinder low-compression finding automatically means a valve problem, and quoting a valve job without borescoping to check for a lower-end cause that would make that repair pointless

## Confirmation standard

A visually clean, uniform cylinder wall crosshatch with no scoring through a full rotation; if scoring is found, the finding itself serves as the confirmation for the customer conversation

Owner note: Complements trees 18, 20, and 29 — this tree is specifically the mechanical tiebreaker once a partial compression-related cause is suspected

# Batch 05 completion status

Five videos were processed via the public YouTube route (no authenticated session used) into Process Extracts GMM-EX-030 through GMM-EX-034 and Agent Knowledge GMM-KB-025 through GMM-KB-031. Coverage added: BMW body-control-module intermittent failure and AWD/xDrive transfer-case triage, aftermarket-accessory parasitic-draw isolation with an immobilizer-splice safety check, first power-window entry, intermittent-misfire exhaust pressure-pulse escalation, dual-battery/auxiliary-support-battery no-crank triage, and a borescope tiebreaker for partial single-cylinder misfire (valve vs lower-end).

This batch was processed directly in the main Claude Code session (not via background subagents) after the account's subagent session limit was reached mid-batch. Two rows in this Priority-2 cluster (Corvette P0300, engine block ground test) remain Queued and are the next items to process. Every promotion in this batch was independently cross-checked against existing trees before being added — no duplicates were created.

# 32. Random misfire caused by a worn torque-tube coupling, not ignition — mechanical pseudo-misfire trap

Decision tree: Random misfire code scattered across both banks that doesn't fully resolve after ignition-component replacement and doesn't correlate with a fuel-trim/feedback issue → rule out ignition-wire leakage and feedback/sensor causes first → if only partial improvement after a real ignition fix, suspect a mechanical driveline cause on platforms with a separate torque-tube/driveshaft coupling → check applicable OEM TSBs for known coupling wear → physically test torque-converter-to-flywheel rotational play per the TSB procedure → excessive play matching the TSB's documented failure spec confirms a mechanical, not ignition, source of the misfire-like symptom.

Provenance: GMM-EX-035. Elite single-source end-to-end demonstration (Super Mario Diagnostics), cross-verified against an on-screen OEM TSB document.

## Evidence source

Source: Corvette P0300 - Running Rough — Super Mario Diagnostics

## Ordered field steps

1. Confirm the misfire code and pull live misfire-counter data; note whether it's scattered across both banks rather than one persistent cylinder.

2. Rule out ignition-component leakage with a test-light + soap-and-water spray test along the plug wires.

3. Check fuel trims and force open-loop operation to rule out a feedback or sensor-driven cause.

4. If ignition parts show a real fault, repair and retest — if improvement is only partial, do not stop there.

5. Look up OEM TSBs for the platform's driveline architecture, especially transaxle layouts with a separate torque-tube/driveshaft coupling.

6. Follow the TSB's physical inspection procedure: access the torque-converter bolt through the inspection covers and rock it while watching the flywheel for movement.

7. Compare observed travel against the TSB's documented failure spec.

8. If travel matches or exceeds spec, explain the mechanism: driveline lag/slap is read by the PCM as a misfire/knock-correction event, not a true ignition misfire.

9. Quote the coupling repair separately from any ignition work already completed.

## Grab list

Tools: Scan tool with live misfire-counter and fuel-trim data, grounded test light, soap-and-water spray bottle, basic hand tools/wrench, OEM TSB lookup access

## Safety and escalation

N/A for the diagnosis itself; the torque-tube coupling repair is a major driveline teardown beyond mobile-service scope — recommend shop referral rather than a safety-driven tow.

## Common misdiagnosis

Assuming a random, multi-bank-scattered misfire is purely ignition-caused and repeatedly replacing coils/wires/plugs without full resolution; not checking driveline-specific TSBs on platforms with a separate torque-tube coupling; mistaking a real but partial ignition-wire fix for full resolution

## Confirmation standard

Torque-converter-to-flywheel play test per the TSB reads within spec after the coupling repair; misfire counts drop to near-zero across a normal drive cycle

Owner note: First mechanical-driveline pseudo-misfire entry in the manual. Repair was declined by the customer, so this finding is TSB/physical-evidence-confirmed but not outcome-confirmed by a completed repair. Also carries a liability/documentation lesson about customer-supplied-or-requested used ignition parts muddying diagnosis.

# 33. Engine block/chassis ground failure — three-method test with a jumper-cable false-negative trap

Decision tree: Slow-crank, no-crank, or a vehicle that only starts with the OBD connector's ground path attached → suspect a bad engine-block/chassis ground → the test-light method and the voltage-drop method are the reliable confirmations → do NOT rely solely on the redundant-jumper-ground method, since thin/cheap jumper cables can mask a real bad ground and produce a false negative → replace the corroded ground strap and re-verify with the voltage-drop method.

Provenance: GMM-EX-036. Elite single-source end-to-end demonstration (Super Mario Diagnostics), full before/after numeric voltage-drop confirmation shown on camera.

## Evidence source

Source: TEST YOUR ENGINE BLOCK GROUND USING THESE THREE METHODS — Super Mario Diagnostics

## Ordered field steps

1. Duplicate/confirm the symptom pattern (slow crank, no crank, or only-starts-with-OBD-attached).

2. Test-light method: clip to a known-good ground, probe the suspect block ground point, crank — illumination confirms a bad ground.

3. Voltage-drop method: connect a DVOM across the ground path, crank, and read the drop using Min/Max hold — target 0.1V or less static/non-cranking, with a higher acceptable ceiling during actual cranking due to starter current draw.

4. Optionally run the redundant-ground jumper-cable method as a supplementary check only, never as the sole confirmation — cheap cables can falsely mask the fault.

5. If the ground fails the test-light or voltage-drop check, replace the ground strap/cable.

6. Re-verify with the voltage-drop method post-repair: static reading at or under 0.1V, cranking reading within the acceptable elevated range, and the test light dark throughout cranking.

## Grab list

Tools: Grounded test light, jumper cables, DVOM/multimeter with Min/Max hold, basic hand tools

## Safety and escalation

N/A — ground strap replacement is a standard mobile-feasible repair, no escalation needed.

## Common misdiagnosis

Trusting a passed redundant-ground jumper-cable test as full proof the ground is good, then chasing battery/starter/other causes after a misleading cheap-cable pass; treating any nonzero cranking voltage drop as automatically bad without accounting for the higher acceptable ceiling under high starter current

## Confirmation standard

Voltage-drop test reads 0.1V or less static/unloaded and stays within an acceptable elevated range during actual cranking with the test light dark; reliable, consistent cranking across multiple attempts

Owner note: First manual entry for engine-block/chassis ground testing specifically. Complements, not duplicates: GMM-KB-001's no-crank starter-circuit tree already has a block ground-drop test as one step, and GMM-KB-002 is the general loaded-powers-and-grounds foundation rule — this tree is the dedicated deep-dive on ground testing itself (all three methods side by side, specific thresholds, and the cheap-jumper-cable false-negative trap) that those two rely on but don't spell out in full.

# Batch 06 completion status

Two videos were processed via the public YouTube route (no authenticated session used) into Process Extracts GMM-EX-035 and GMM-EX-036, and Agent Knowledge GMM-KB-032 and GMM-KB-033. Coverage added: a torque-tube-coupling driveline defect that mimics an ignition random misfire (cross-verified against an on-screen GM TSB), and the three standard methods for testing an engine-block/chassis ground including a jumper-cable false-negative trap.

This closes the ScannerDanner/South Main + Pine Hollow/Super Mario Priority-2 cluster (rows 12-26, all Extracted). Next: Priority-2 rows 27-34 (DiagnoseDan, Rainman Ray's, The Car Care Nut).

# 34. Suspected parasitic battery drain that is actually a bad-battery ghost-voltage reading plus a technician/customer smart-key proximity wake-up false positive — no real drain exists

Decision tree: Battery dead every morning → test battery immediately after charging (may falsely pass) → load the battery (lights/dome on) and watch voltage under load; if it sags and doesn't recover, suspect a bad cell regardless of what the tester says → attempt a slow full recharge; a plateau below spec confirms the cell is bad → swap in a known-good test battery and re-run the load check as a control → prep the vehicle for a true sleep state and soak overnight → run non-intrusive checks first (thermal imager, quick amp-clamp snapshot) → if extended monitoring is needed, install an in-line current monitor with an audible alarm → when the alarm trips intermittently, correlate against your own (or anyone's) proximity-key movement near the vehicle before blaming the car → confirm true baseline draw with all keys kept away → if baseline is normal, conclude no real drain, replace the battery, coach the customer on key storage distance.

Provenance: GMM-EX-037. Elite single-source end-to-end demonstration (DiagnoseDan), including the technician catching and correcting his own initial misreads on camera.

## Evidence source

Source: Lexus battery DEAD by morning. FIXED — DiagnoseDan

## Ordered field steps

1. Charge dead battery fully before testing.

2. Battery-test with a reserve-capacity tester, distrust an immediate post-charge good result.

3. Confirm charging-system output under load.

4. Load-test manually (dome+headlights); voltage should sag briefly and fully recover, a bad cell sags and stays low.

5. Retest cold and cross-check against a known-good swap battery under identical load.

6. Attempt a slow extended recharge; a plateau below spec confirms a broken/shorted cell, stop testing.

7. Scan and save fault codes without chasing history codes.

8. Prep for a genuine sleep-state soak (doors open, locks manually latched, jamb switches defeated), leave overnight.

9. Run non-intrusive tests first (thermal imager sweep, amp-clamp snapshot) before disconnecting anything.

10. For extended/unattended monitoring, install an in-line current-threshold monitor (bridge with a parallel jumper during install, wait ~15min, remove jumper).

11. Every alarm trip, check whether you or anyone with a proximity key approached the vehicle before concluding a vehicle drain.

12. Re-run the monitored test with all keys stored well away for the true baseline.

13. If baseline settles normal, close out as no drain, replace battery, advise on key storage distance.

## Grab list

Tools: Multimeter, amp clamp, battery/reserve-capacity tester, scan tool, thermal imager, in-line current monitor with adjustable audible threshold

## Safety and escalation

Not applicable in the traditional no-start/tow sense; for a mobile tech the equivalent escalation point is bringing the vehicle to a secure location for an overnight soak rather than attempting a shortened/rushed drain test.

## Common misdiagnosis

Trusting a battery tester's 'good' verdict taken immediately after charging without a real load-and-recover check; selling a customer a new battery without ruling out an active intermittent drain first; chasing a phantom parasitic drain that is actually the technician's own or the customer's smart key waking the passive-entry system every time someone approaches with it in pocket; assuming a customer-disconnected accessory is unrelated without confirming the timeline.

## Confirmation standard

Load test under a known live draw with voltage recovery watched, not just resting voltage; slow full recharge to see the true achievable plateau; control comparison against a known-good battery under identical load; correlate any current-monitor alarm trip against technician/key movement before treating it as a vehicle fault; final baseline current draw check with all keys removed from proximity.

Owner note: Genuinely underrated trap for mobile techs specifically, since mobile work involves frequently approaching parked customer vehicles with a key fob in pocket while running exactly this kind of test. Keep your own key fob (and the customer's, and any spare) at least several car-lengths away during any extended parasitic-draw monitoring.

# 35. No scan-tool communication — K-Line short-to-ground isolation by sequential module disconnection

Decision tree: No scan-tool communication at all → check if the dongle powers on with ignition on → if dead, hook up an OBD-II breakout box to check for power and ground at the connector → if nothing, trace the power pin via wiring diagram to its fuse, test and replace if blown → recheck breakout box/dongle → if power restored but scan tool topology shows a partial network → consult the wiring diagram to find which pin carries the missing bus and which modules sit on it → scope that pin during a known-good vs known-failing comm attempt; zero voltage/activity means a wiring fault, not a handshake failure → power a test light from the connector's power pin and probe the suspect pin; light ON immediately confirms a dead short to ground → identify every module wired to that shared pin → disconnect them one at a time, easiest access first, watching the test light → light stays on = innocent, reconnect and move on; light goes OFF = short isolated to that module → physically inspect to confirm → replace the module, reconnect everything else, rescan, clear codes, verify.

Provenance: GMM-EX-038. Single elite trusted source (DiagnoseDan) demonstrating the method end-to-end on camera with real breakout-box, test-light, and scope evidence.

## Evidence source

Source: Multiple Techs Failed to Diagnose this Mini Cooper — Here's Why! — DiagnoseDan

## Ordered field steps

1. Confirm the no-comm complaint; check battery voltage/charge as needed.

2. Attempt scan-tool connection, note if the dongle powers on at all.

3. If dead, connect a 16-pin breakout box, check for power/ground LED activity.

4. If nothing lights, trace the power pin via wiring diagram to its fuse, test and replace if blown.

5. Recheck breakout box/dongle.

6. Reattempt communication; if partial topology, treat as a second, separate fault.

7. Identify which pin serves the missing branch and which modules are wired to it via the diagram.

8. Optionally scope that pin during a known-working vs known-failing comm attempt.

9. Zero voltage/flat line points to a wiring fault, not a bad module.

10. Power a test light from the power pin, probe the suspect signal pin — immediate illumination confirms a dead short to ground.

11. List every module wired to that shared pin.

12. Disconnect each one at a time, most accessible first, watching the test light after each.

13. Light stays lit = innocent, reconnect and move to next.

14. Light goes out = short isolated to that module/connector/branch.

15. Physically inspect for confirming evidence (burn smell, melted connector).

16. Reconnect all innocent modules.

17. Rescan to confirm the previously missing branch now communicates.

18. Repair/replace the confirmed-faulty module, priming fluid before energizing if applicable.

19. Clear codes generated while the network was down.

20. Final rescan confirming zero active codes and full communication.

## Grab list

Tools: 16-pin OBD-II breakout box, digital multimeter, 12V test light, factory/aftermarket wiring diagrams, scan tool with a network topology/module-list view, oscilloscope (optional confirmation), battery charger/maintainer, jack stands, hand tools

## Safety and escalation

Vehicle already undriveable generally needs shop-bay time, not roadside work. In a mobile-van context, escalate to a shop/lift when the isolated module requires extended underbody R&R impractical at a customer's location, or when the replacement part requires a multi-day special order.

## Common misdiagnosis

Treating 'no communication at all' as a dead scan tool/cable/dongle problem instead of checking for missing power/ground at the connector; defaulting to 'it's a CAN-bus/network problem' when the diagnostic-tool link may run over a different protocol than the inter-module network; a 'specialist' condemning a module by guesswork without proving the connection; disconnecting only the 'obvious' module and stopping instead of working through every module on the shared circuit; not verifying whether a connector was already disconnected by a previous shop before testing it.

## Confirmation standard

Test light no longer illuminates at the suspect pin once the faulty module is disconnected; full vehicle rescan shows the previously-missing bus/modules now present; codes clear and stay clear; teardown of the removed module shows physical damage consistent with the electrical short.

Owner note: Cross-reference: GMM-KB-024 shares the 'verify, don't trust a prior shop' meta-lesson, but this entry supplies the concrete, reusable breakout-box/wiring-diagram/test-light short-isolation technique that 024 does not spell out. Would have caught both the blown fuse and the K-line short in under an hour.

# 36. Suspected tool-induced no-start that is actually a chip-tuned ECU suppressing fault codes — the 'zero DTCs under any fault' trap

Decision tree: Customer/owner reports a no-start that began right after connecting a scan tool → do NOT assume tool damage → confirm complaint and run a full-system fault scan → if the scan shows zero codes despite a clear no-start, or if deliberately disconnecting known-faulty circuits still sets no codes, that absence-of-codes pattern IS the diagnostic finding → stop chasing wiring/sensors and ask the owner directly whether the vehicle has been tuned/chipped/remapped → if yes, tuned software commonly suppresses DTC storage to hide performance-related codes, and depending on the platform an active-but-hidden fault can itself block engine operation → reflash to OE/stock software via a genuine OE tool or remote OE-bridge service → re-run the same fault-inducing test to confirm codes now store correctly → clear codes and confirm normal start/run.

Provenance: GMM-EX-039. Single elite trusted source (DiagnoseDan) demonstrating the full chain end-to-end with real scope captures and real scan-tool DTC screens, conclusion directly proven on camera via the before/after injector-disconnect test.

## Evidence source

Source: Destroyed by Aftermarket Scantool?! (Car Ran Perfect Before This) — DiagnoseDan

## Ordered field steps

1. Confirm the complaint yourself, don't take a 'the scan tool broke it' story at face value.

2. Run a full-system fault scan across all modules before touching anything else.

3. If zero codes are stored despite a real fault, treat that as a red flag, not a clean bill of health.

4. Work the standard no-start elimination branches (fuel delivery/pressure, spark, cam signal, injector command via scope, injector actuation via bi-directional test) to isolate what's missing.

5. If deliberately creating an open circuit still sets no fault code, that is the tell — a healthy stock PCM should always flag a self-created open circuit.

6. Ask the owner directly: has this vehicle ever been tuned/chipped/remapped? Don't skip this even under a compelling 'the tool did it' narrative.

7. If tuned, get the vehicle back to OE/stock software via genuine OE diagnostic software or a remote OE-bridge/tap tool.

8. After reflash, re-run the same fault-inducing test to confirm the OE software now stores DTCs normally.

9. Clear codes stored during testing and confirm clean start/run.

10. Document that returning to stock resolved it and that the customer was informed/agreed before doing so.

## Grab list

Tools: Full-system OBD-II scan tool with live data + bi-directional/actuation tests, labscope with back-probe pins, fuel pressure gauge with quick-connects, battery maintainer, wiring diagram, OE diagnostic software or a remote OE-bridge/tap device

## Safety and escalation

If the vehicle is confirmed tuned and the shop doesn't carry OE reflash capability, escalate to a shop/dealer with genuine OE programming access rather than diagnosing further against unknown modified software.

## Common misdiagnosis

Assuming a scan tool physically damaged/bricked a vehicle just because symptoms began right after its use; chasing individual sensors/wiring one at a time on a no-DTC no-start without noticing the 'nothing ever sets a code' pattern; skipping the tuned/modified-vehicle question because the customer's story is distracting; treating a battery-disconnect 'ECU reset' as a real fix rather than a quick free test.

## Confirmation standard

Full-system scan showing zero DTCs despite a real fault; deliberately opening a circuit and confirming no code is set (the tuned-suppression signature); post-reflash re-test of the same deliberate fault showing the code now stores correctly.

Owner note: Bias-correction entry as much as a technical one — its main value is training the agent NOT to reflexively agree with a customer's 'the scan tool broke my car' narrative, and instead recognize the specific no-DTC-under-any-fault signature as the actual lead. Reflash/remote-OE-bridge capability may exceed current GMM mobile-kit standard — flag as a possible tooling gap.

# 37. Intermittent 'no communication' module fault (e.g. ABS/DSC) caused by an unseated power-feed connector — confirm all power feeds AND recheck during an active fault before condemning the control module

Decision tree: Warning light on + scan shows 'no communication' with a module → do NOT jump to 'bad module' → confirm the complaint, pull codes (note some scan tools synthesize a placeholder DTC for a true no-comm state) → check ALL power feeds and grounds at the module connector with ignition on using a test light → verify CAN high/low waveform at the disconnected connector with a scope → if everything reads good in that moment, do not condemn the module yet → watch for the fault to change state, proving it's intermittent → when comm briefly restores, a clear-and-rescan showing zero stored codes further confirms the module is not internally faulty → wait for/reproduce the fault again, immediately re-run the same power/ground checks during the active fault, comparing pin-by-pin against the earlier good reading → once one feed is isolated, wiggle-test that circuit tracing its full physical route → when the light flickers at a specific point, inspect that connector closely, even one that looks seated can be the fault → reseat it, verify it holds under wiggle, rescan to confirm clean comm.

Provenance: GMM-EX-040. Single elite trusted source (DiagnoseDan) demonstrating the full sequence end-to-end on camera with real scan-tool screens, oscilloscope capture, and physical proof of the fix.

## Evidence source

Source: I Almost Replaced a GOOD ABS Module... This Mini Cooper Fooled Me — DiagnoseDan

## Ordered field steps

1. Confirm the customer complaint.

2. Support the battery if extended ignition-on diagnosis is expected.

3. Scan for codes; cross-check which other modules report missing messages from the affected one.

4. Pull the wiring diagram, identify EVERY power feed and ground pin — don't assume only one of each.

5. Check every fuse physically.

6. Disconnect the module connector, verify power at every power pin and ground at every ground pin with a test light.

7. Verify CAN/bus signal at the connector with a scope if available.

8. If all statics pass, resist condemning the module — watch for the fault to change state.

9. If communication returns on its own, clear and rescan; zero stored codes on a freshly-communicating module is strong evidence the module is fine.

10. Wait for or attempt to reproduce the fault state again.

11. The instant the fault is active, immediately re-run the exact same pin-by-pin checks and compare against the earlier good readings.

12. Isolate which single feed/ground now differs.

13. Wiggle-test that isolated circuit with an in-series test light, tracing its full physical route.

14. Closely inspect the connector at that point — check whether it's fully seated, not just connected.

15. Fully seat the connector, re-wiggle to confirm no flicker.

16. Inspect neighboring connectors for corrosion history to understand root cause.

17. Final rescan across an ignition cycle to confirm stable communication.

## Grab list

Tools: Bi-directional scan tool with wiring diagram access, 12V test light, automotive oscilloscope for bus-signal verification, battery support charger, trim removal tools, pry bar for chassis-routed wiring access

## Safety and escalation

Not a tow scenario if the vehicle is drivable and scannable throughout; escalates beyond mobile scope if the fault cannot be reproduced/captured roadside, or if the connector/fuse box itself is damaged and needs replacement rather than reseating.

## Common misdiagnosis

Replacing the control module based on a persistent 'no communication' code without retesting during an active fault state; mistaking a scan-tool-synthesized placeholder DTC for a genuine stored module fault; stopping the diagnosis once one power feed and ground test good without checking every separate feed; assuming a connector that visually 'looks' plugged in is fully seated.

## Confirmation standard

Reproduce/capture the fault state, then immediately retest each power feed pin individually with a test light; wiggle test with an in-line test light tracing the entire physical wire run; visually confirm full seating of every connector along the circuit; reseat and rescan to confirm the fix holds under wiggle and across an ignition cycle.

Owner note: High-value comeback-prevention entry — this exact scenario is a classic expensive-comeback trap (replace a $400+ module, light comes back because the real fault was a $0 connector). The single most transferable lesson: 'no comm + every static check passes' is not proof of a bad module, it's proof you haven't caught the fault in the act yet.

# 38. Random, simultaneous multi-module misbehavior ('mind of its own') + recurring battery drain — caused by one failed node jamming a shared communication bus; diagnosed via oscilloscope-guided network bisection

Decision tree: Customer reports a vehicle acting 'possessed' — random module power cycling, intermittent no-crank, recurring dead battery → confirm objectively on camera without touching anything → rule out the battery itself by charging and retesting → full-system scan shows communication-loss codes across nearly every module → use an OBD breakout box + oscilloscope to find which specific comm pin/bus is abnormal → pull the wiring diagram for that bus to find its physical topology and a hub connector where the network splits → disconnect one half at the hub and watch the scope; if the waveform normalizes, the fault is in the disconnected half → repeat the halving on the guilty branch until disconnecting one specific module normalizes the waveform → that module is the root cause; use a same-type twin module on the other side as a negative control before condemning the part.

Provenance: GMM-EX-041. Single elite trusted source (DiagnoseDan) demonstrating the full sequence with real oscilloscope waveforms, real scan-tool DTC screens, real physical isolation, repeated re-tests confirming the fix, and independent corroboration via a real manufacturer TSB.

## Evidence source

Source: Customer States: My Car Has a Mind of Its Own — DiagnoseDan

## Ordered field steps

1. Reproduce the complaint on camera without touching controls; note whether it's simultaneous across unrelated modules.

2. Measure battery voltage; if low, charge and retest.

3. Run a full-vehicle scan; a near-uniform spread of comm-loss codes across most modules means a shared network problem.

4. Connect an OBD breakout box to see which pins carry active traffic.

5. Oscilloscope each active pin; the faulted bus shows intermittent valid packets mixed with a flat/regular square wave.

6. Identify the abnormal pin's network via wiring diagram; note total node count.

7. Find where the network physically splits into two branches (commonly at the BCM).

8. Back-probe the bus wire, disconnect one branch at the hub — if the waveform normalizes, the fault lives in the other branch.

9. Walk the guilty branch disconnecting one module at a time watching the scope until the waveform normalizes.

10. If the suspect part has a paired/twin module, disconnect the twin as a control.

11. Rescan and clear codes with the bad module isolated; communication should restore network-wide.

12. Cycle the vehicle multiple times to confirm the fix is stable.

13. Cross-check the finding against a TSB/known-fix database.

## Grab list

Tools: Full-system scan tool with topology/health-report view, digital multimeter, battery charger, OBD breakout box, automotive oscilloscope + back-probe pins, wiring diagram subscription, jack + jack stand, basic hand tools, TSB/known-fix database access if available

## Safety and escalation

Not a tow-worthy safety condition on its own, but this is a multi-hour diagnostic best scheduled as a dedicated diagnostic appointment rather than attempted as a quick roadside/driveway job; if a TSB match exists, the diagnosis can often be shortcut to 10-15 minutes.

## Common misdiagnosis

Replacing the battery (again) without proving it's the root cause; condemning the BCM itself because it shows the most codes/is the 'hub' when it's only relaying disruption from a downstream node; checking modules randomly without first bisecting the network with the diagram, burning hours checking 20+ nodes individually.

## Confirmation standard

Oscilloscope waveform on the affected bus pin returns to a clean, consistent pattern the instant the correct module is disconnected; full-vehicle rescan shows restored communication network-wide; repeated start/stop and ignition cycling behaves normally across multiple trials.

Owner note: The genuinely reusable piece for a mobile tech is the phone-triage pattern: RANDOM, SIMULTANEOUS misbehavior across multiple unrelated systems PLUS a battery that keeps going dead even after replacement should raise 'communication-bus interference/parasitic keep-awake fault' as the leading hypothesis, and should set the customer's expectation for a longer, scheduled diagnostic.

# 39. Parasitic battery draw caused by a powered component (rear wiper motor logic board) that never reaches its park/off state — continuous 'park-seek' current draw, isolated via in-series ammeter + door-latch hibernation trick + fuse-disable narrowing + scan-tool cross-check + component disconnect

Decision tree: Battery keeps dying overnight → disconnect negative terminal, splice ammeter in series on the battery post → defeat all door latches (2-click trick) to fool the BCM into 'doors closed,' let the vehicle fully sleep, watch amperage fall from several amps down to a resting mA baseline → rule out aftermarket accessories first (unplug all, test individually) → if a suspect powered subsystem exists, pull its dedicated fuse to disable the whole subsystem and see if draw drops → if draw persists or the subsystem is implicated, cross-check with scan-tool bidirectional data on any working analog → back-probe the suspect component's connector for power and signal wires to isolate switch vs wiring vs component → physically unplug the suspect component and re-test draw → once draw settles to a 'perfect' low-mA baseline with the component disconnected, confirm with a multi-day undisturbed voltage-soak test before committing to the repair.

Provenance: GMM-EX-042. Single elite trusted source (Rainman Ray's Repairs) demonstrating end-to-end on camera with real meter/scan-tool/voltage evidence across a full unedited session.

## Evidence source

Source: This is why Mechanics charge for Diagnostics! Car Battery Keeps Dying! — Rainman Ray's Repairs

## Ordered field steps

1. Disconnect negative battery terminal.

2. Wire a milliamp-capable multimeter/clamp meter in series between the battery post and the original negative cable.

3. Defeat every door latch with the two-click fully-latched trick; never force a door held in this state.

4. Let the vehicle sit and watch amperage fall to a resting mA draw; restart the wait fully after any accidental wake event.

5. Establish a vehicle-specific 'in spec' ceiling from the settled reading.

6. Disconnect all aftermarket accessories before the test, test each individually afterward.

7. If a specific subsystem is suspected, pull its dedicated fuse/relay to disable it and watch for a drop in draw.

8. Cross-check the suspect system's health using scan-tool bidirectional data on a working parallel control.

9. Back-probe the suspect component's connector for power and expected signal; verify you are probing the correct switch/connector before condemning wiring or the component.

10. If the component is confirmed to still misbehave even with its controlling switch verified OFF, physically unplug it and re-run the ammeter test.

11. Record a baseline voltage, leave the vehicle completely undisturbed for 24-72 hours, recheck voltage — a small drop confirms normal self-discharge and validates the fix.

## Grab list

Tools: Milliamp/amp-capable DMM or clamp meter (in-series capable), threaded marine battery-terminal adapter, scan tool with bidirectional controls, OEM wiring diagrams and theory-of-operation service data access

## Safety and escalation

Not a tow-worthy drivability issue by itself, but this is inherently a leave-behind or multi-day diagnostic, not a single mobile-visit fix; escalate to shop drop-off or leave test equipment on the vehicle at the customer's location across 1-3 days if a portable logger isn't available.

## Common misdiagnosis

Blaming aftermarket accessories as the sole cause without individually testing them; probing the wrong switch/connector when a signal reads unexpectedly; treating a same-day short soak as conclusive when a component that intermittently seeks or wakes may not show its worst behavior until well into an overnight or multi-day soak.

## Confirmation standard

In-series ammeter draw settling to the vehicle's established low-mA baseline with the suspect component disconnected; multi-day (24-72hr) undisturbed voltage soak showing only normal self-discharge rather than continued voltage collapse.

Owner note: This is a slow, multi-day job by nature — set expectations with the customer up front. For GMM's mobile model, this likely means either a portable current/voltage logger left on the vehicle, or classifying this job type as a scheduled multi-day diagnostic rather than a single mobile stop.

# 40. P0171/P0174 (System Too Lean, Bank 1/Bank 2) general elimination framework for Toyota/Lexus — the highest parts-cannon-risk DTC pair in the field

Decision tree: Confirm the correct bank via Toyota's official bank-ID document → read LTFT+STFT at warm idle to gauge severity → check Calculated Load at idle vs WOT to screen the MAF sensor (clean before replacing, confirm the fix by watching WOT load and fuel trim recover) → if MAF is good, run the scan tool's bi-directional AFR active-fuel-control test and watch both engine response and sensor voltage response → an engine that gets worse when fuel is added and better when fuel is cut, despite a stored lean code, flags a skewed/false-lean AFR sensor rather than a true lean condition → if MAF and AFR sensor are both good, check fuel pressure and check cylinder misfire count during the same active fuel-cut test to isolate a partially clogged injector → check for major and then hidden/internal vacuum leaks → if still unresolved, suspect rare mechanical causes and restart the checklist from the top rather than guessing a part.

Provenance: GMM-EX-043. Single-source but high-trust — one elite, channel-qualified Toyota/Lexus specialist walking the complete elimination logic end-to-end in whiteboard/lecture format, not live on-car footage. Companion GMM-KB-041 (live-demonstrated fuel-trim literacy, same channel) corroborates the shared fuel-trim-reading fundamentals underlying this checklist.

## Evidence source

Source: The Most Difficult Issues To Diagnose for Toyota and Lexus | P0171 Diagnosis Tips — The Car Care Nut

## Ordered field steps

1. Confirm which bank(s) set the code via the official Toyota Bank Identification document.

2. At warm idle, read LTFT/STFT on a scan tool with live data.

3. Classify severity: trims nearing/exceeding the ~40% compensation ceiling = catastrophic leak; LTFT elevated with STFT stabilized near zero = moderate, compensated leak.

4. Read Calculated Load at idle vs WOT to screen the MAF sensor; a cap around 55-60% at WOT flags a failing MAF.

5. Clean the suspect MAF before replacing it, re-check WOT load and trim recovery to confirm.

6. If MAF is good, run the bi-directional AFR active test and watch engine stability and sensor voltage.

7. Engine worse when fuel added, better when cut = false lean code from a skewed AFR sensor; cross-check opposite bank on V-engines.

8. If MAF and AFR sensor both good, check fuel pressure and check Cylinder Misfire Count during the same active fuel-cut test to isolate a partially clogged injector.

9. Check for major vacuum/oil leaks including a missing oil cap or dipstick.

10. For hidden leaks, clamp the EVAP purge valve line and brake booster line, watch trim for immediate correction.

11. Without a smoke machine, spray throttle-body cleaner on suspect hoses watching live trim for an erratic swing.

12. If still unresolved, consider rare causes and restart the checklist rather than replacing parts on a guess.

## Grab list

Tools: Scan tool with live PIDs (fuel trim, calculated load, AFR sensor voltage, cylinder misfire count) and bi-directional/active-test capability, fuel pressure gauge, hose clamp pliers, smoke machine or carb cleaner spray, Toyota Bank Identification reference document

## Safety and escalation

Not a tow-condition code by itself; this is a diagnostic-time escalation, not roadside — pause and restart the checklist from the beginning if diagnosis stalls after all steps are exhausted rather than parts-cannoning.

## Common misdiagnosis

Buying a MAF or AFR/O2 sensor off a parts-store generic code guess without confirming via live PID/active test data first; assuming any nonzero fuel trim is abnormal; assuming leaking injectors cause lean codes (they cause rich codes); missing that an EVAP purge valve leak will not show on an intake smoke test.

## Confirmation standard

WOT calculated load PID reads 90-100%; bi-directional AFR active-fuel-control test confirms or rules out a skewed AFR sensor and isolates clogged injectors via cylinder misfire count; purge-valve and brake-booster line-clamp test with an immediate trim correction confirms an internal/hidden vacuum leak.

Owner note: Foundational triage framework, not a single-fault case study — treat as the default P0171/P0174 checklist for GMM Toyota/Lexus jobs. Preserve the presenter's own rule verbatim: 'spend 99% of your time checking, 1% replacing.' NOTE: promoted on lecture/whiteboard explanation from a trusted elite source, not live filmed evidence — see Consensus Level.

# 41. Fuel trim (STFT/LTFT) misinterpretation — reading live fuel-trim data correctly to localize lean/rich faults instead of misreading polarity or chasing a 'zero' trim

Decision tree: Pull STFT/LTFT (bank 1 and bank 2) plus AFS voltage on a scan tool with the engine fully warmed, in closed loop, idling with no accessory loads → confirm fuel system status reads CL → read polarity: positive = PCM adding fuel = lean; negative = PCM cutting fuel = rich → read behavior over magnitude: STFT actively cycling around a mildly elevated/reduced LTFT is normal adaptive compensation, never chase it to zero → if STFT is pinned/stuck while LTFT climbs/drops in the same direction and AFS voltage sits outside its normal band, that's a real active fault → compare bank 1 vs bank 2: symmetric shift = shared/upstream cause; asymmetric single-bank shift = single-cylinder/bank cause → to isolate a suspect injector without removal, use forced rich/lean Active Test with a misfire counter on screen: forced-rich misfire = leaking injector, forced-lean misfire = clogged injector.

Provenance: GMM-EX-044. Single elite trusted source (The Car Care Nut) demonstrated end-to-end on camera with two real induced-fault clips and matching before/after live-data numbers, independently frame-verified.

## Evidence source

Source: Toyota Fuel Trims and live data diagnosis tips! — The Car Care Nut

## Ordered field steps

1. Connect a scan tool with live-data graphing and bidirectional/Active Test control.

2. Bring the engine to full temperature, closed loop, idle, accessory loads off.

3. Pull Fuel System Status, Short/Long FT B1S1/B2S1, AFS Voltage B1S1/B2S1; confirm status = CL before trusting trim.

4. Read trim polarity first — positive means the PCM is adding fuel (lean), negative means cutting fuel (rich).

5. Read behavior, not just magnitude — STFT cycling around a mildly elevated/reduced LTFT is normal, never chase trim to zero.

6. Cross-check AFS voltage against its ~3.0-4.0V normal band; pinned outside plus a stuck STFT confirms a real fault.

7. Compare bank 1 vs bank 2 — symmetric = shared/upstream cause, asymmetric = single-cylinder/bank cause.

8. Understand two-trip pending-code logic — a lean fault needs to reoccur on a second cold-start cycle to go hard/MIL.

9. Confirm a repair live — Short-Term trim swings sharply opposite almost immediately if the fix worked.

10. Isolate a suspect injector without removal using forced rich/lean Active Test with a misfire-counter PID on screen.

11. Never trust fuel trim while out of closed loop.

12. Never evaluate trim while driving/revving — trim is stored as a per-RPM/load map, not one number.

## Grab list

Tools: OEM-level scan tool with live-data graphing and bidirectional/Active Test control (Toyota Techstream or equivalent aftermarket tool with matching PIDs and forced rich/lean capability)

## Safety and escalation

Not addressed — pure live-data literacy for a mobile tech already on scene with a scan tool; no tow/escalation criteria given.

## Common misdiagnosis

Assuming a lean code means fuel trim reads negative (it's reversed); chasing fuel trim to exactly 0%; reading/comparing trim while driving or revving; treating a frozen trim value during open loop as meaningful; judging magnitude alone instead of behavior.

## Confirmation standard

Cross-check AFS/O2 voltage against the ~3.0-4.0V normal band; confirm fuel system status = CL before trusting any trim reading; compare bank-to-bank trim/AFS symmetry to localize shared vs single-bank causes; use forced rich/lean Active Test with a misfire-counter PID to isolate a specific leaking or clogged injector.

Owner note: Foundational literacy underlying GMM-KB-018 (misfire triage) and GMM-KB-040 (P0171 triage, same channel) — treat as the 'how to read the numbers' prerequisite layer. GMM-KB-040 supplies specific real-world root causes; this entry is the reading-the-data layer they both depend on.

# Batch 07 completion status

Eight videos were processed via the public YouTube route (no authenticated session used, four background agents dispatched in parallel plus one catch-up agent for a skipped row) into Process Extracts GMM-EX-037 through GMM-EX-044, and Agent Knowledge GMM-KB-034 through GMM-KB-041. Coverage added: a battery-ghost-voltage plus technician's-own-key-fob false-positive parasitic-draw trap; K-Line short-to-ground isolation via sequential module disconnection; a suspected tool-damage case that was actually a chip-tuned ECU suppressing fault codes; an ABS module nearly condemned during an intermittent no-comm fault, saved by re-testing live and isolating a single unseated power-feed connector; a comm-bus-jamming radar module diagnosed via oscilloscope-guided network bisection, triggered by translating vague customer language into a testable hypothesis; a rear wiper motor's park-seek failure causing a slow parasitic drain, isolated via a door-latch hibernation trick and a multi-day voltage-soak confirmation; and a two-part Toyota/Lexus lean-code foundation — a general P0171/P0174 elimination framework (lecture-format, confidence held at 4/5) plus the live-demonstrated fuel-trim-reading literacy (confidence 5/5) it depends on.

This closes all of Priority-2 rows 12-34 (ScannerDanner/South Main, Pine Hollow/Super Mario, DiagnoseDan, Rainman Ray's, The Car Care Nut). Remaining: Priority 3 (rows 36-38, 40-41, 43-44) and Priority 4 (rows 45-49), plus the final domain-coverage audit and the still-open no-crank-car-with-tow-escalate gap.

# 42. Engine accessory noise — belt-elimination + hand-spin pulley isolation to find a failed idler/accessory bearing

Decision tree: Front-of-engine rotational noise → listen by ear to form an initial hypothesis but treat it as unverified → map every accessory each drive belt turns, noting stacked belts require sequential removal → optionally probe with a long screwdriver/pry bar resting on component mounting points while the engine runs, comparing loudness across accessories as a coarse localizer → remove belt(s) one at a time and restart the engine after each removal; noise gone with a given belt off confirms that belt's accessory path is at fault (persists after every belt is removed → escalate to internal/valve-train diagnosis, see tree 43) → with the engine off, disconnect the alternator's signal connector before testing it, since it is a known chronic false-positive noise source on some platforms → hand-spin every accessory on the confirmed belt individually, feeling for roughness or play → the pulley(s) that "sing" or feel rough are the confirmed source → pull the suspect part, bench-test its bearing by hand against a known-good/new reference, and read the stamped bearing size/part number to source a physical match → before installing, compare old vs new parts for any design/TSB update (a missing shield/seal plate on an updated part can cause a repeat failure) → press the bearing out/in with a vise and drift if servicing in-house → reassemble with correct orientation, hand-tighten only, never impact-wrench a bearing retaining nut → spin every new/serviced pulley by hand to confirm free rotation before running → restart and confirm the noise is gone.

Provenance: GMM-EX-045, GMM-EX-046. Two independent elite trusted sources (EricTheCarGuy, The Car Care Nut) demonstrating the same core method end-to-end on two different real vehicles — the manual's strongest promotion bar, merged into one tree per established practice of consolidating same-method corroborating sources rather than creating contested duplicate trees.

## Evidence source

Source: How To Find Engine Noises — EricTheCarGuy; Mechanic Tip on Engine Noise Diagnosis with Simple tools! — The Car Care Nut

## Ordered field steps

1. Start the engine and listen to characterize the noise and form an initial hypothesis, but do not commit to it.

2. Map every accessory each drive belt turns before removing anything; note when belts stack.

3. Optionally reach a long screwdriver or pry bar into the suspected area with the engine running, resting it on a mounting bolt, comparing vibration/sound loudness across accessories.

4. Support the vehicle at two independent points before working underneath.

5. Remove the outermost/front-most belt first if belts are stacked; use a bungee cord to hold a removed belt clear so the engine can be quickly restarted.

6. Start the engine with that belt off and listen: noise gone = confirmed on that belt's path; unchanged = move to the next belt.

7. Repeat belt-by-belt until the noise disappears, or, if it never disappears with every belt removed, treat it as internal to the engine and escalate.

8. With the engine off, disconnect the alternator's small signal connector (never the main charge wire) before testing it, to rule out a known false-positive noise source.

9. Hand-spin/feel every accessory the confirmed belt drives individually for roughness and play.

10. Pull the suspect component and bench-test its bearing by hand against a known-good/new bearing's spin feel.

11. Read the bearing size/part number stamped on the race or seal and physically match the replacement part before buying.

12. Inspect the belt itself — backside shine/grooves is normal wear, not a defect requiring replacement.

13. Before installing, compare old vs new parts for a design/TSB update.

14. Support the pulley in a vise, drive the old bearing out with a punch noting orientation, drive the new one in the same way.

15. Confirm any inner sleeve/washer orientation on reassembly.

16. Reinstall, hand-tighten the retaining nut only — never impact-wrench.

17. Spin every new pulley by hand to confirm no binding before running.

18. Restart and listen to confirm the fix.

## Grab list

Tools: Floor jack, jack stand, long flat-blade screwdriver or pry bar (optional coarse localizer), mechanic's stethoscope (optional), 10mm/14mm sockets, bungee cord, bench vise, punch/drift and hammer, replacement bearing matched to stamped part number

## Safety and escalation

If the noise persists after every belt (including a single serpentine belt) has been removed and the engine restarted, the source is internal to the engine — beyond belt-elimination scope; escalate to the scope-correlation method (tree 43) or shop teardown, not a mobile-van bearing swap.

## Common misdiagnosis

Trusting a confident by-ear diagnosis without verifying; condemning the alternator on sound alone instead of disconnecting its signal connector to actually test it; treating normal belt backside glazing as a defect; installing an updated idler pulley without its accompanying shield/seal plate; replacing only the loudest of several same-age worn parts; impact-wrenching a bearing retaining nut instead of hand-tightening only.

## Confirmation standard

Belt-off restart test; hand-spin/feel test on each accessory the confirmed belt drives, in place; bench spin-feel comparison of the pulled bearing against a known-good bearing; final restart-and-listen after repair.

Owner note: Merged tree combining EricTheCarGuy's systematic multi-belt sequential-removal sequence and bench-bearing-swap procedure with The Car Care Nut's alternator-signal-disconnect false-positive trick and TSB-style design-update caution. DIY-only note carried from source: on 1GR-FE/2GR-FE Toyota V6 engines the tensioner-pulley retaining bolt is reverse-threaded, allowing pulley-only replacement without pulling the full tensioner — not recommended for a paid professional repair. See tree 43 for the instrumented escalation tier.

# 43. Engine noise pinpointing via piezo probe + in-cylinder pressure-transducer phase correlation — escalation tier for exact cylinder/valve identification

Decision tree: Scan for codes/misfire data to rule out an electrical cause → clear-flood crank test to listen for a gross cranking-rhythm abnormality (screening only, don't stop here) → relative compression test (current clamp on starter + ignition-sync channel) to rule out a cylinder-to-cylinder compression difference → if compression looks normal but a real noise persists, rig a piezo contact probe (repurposed knock sensor) on one scope channel while simultaneously capturing in-cylinder pressure via a spark-plug-hole transducer on a reference cylinder → first confirm on a multi-cycle trend view that the noise is truly periodic and repeatable → zoom in and use the scope's phase-ruler/partition tool to divide one 720-degree cycle into strokes referenced to the pressure-transducer cylinder → overlay the noise spike's timing against the pressure-transducer's valve-event landmarks → if the noise lines up with the reference cylinder's exhaust-open/close events, it's that cylinder's exhaust valvetrain; if intake-open/close, it's intake; if it doesn't line up with the reference cylinder at all, repartition the phase rulers to the full cylinder count and slide the window until a match is found → use the scope-identified cylinder/valve side to justify pulling the valve cover for a targeted visual inspection instead of guessing → confirm root cause visually.

Provenance: GMM-EX-047. Single elite trusted source (Trained by Techs) demonstrating the full sequence end-to-end with real scope captures and a confirmed teardown.

## Evidence source

Source: Engine Noise Diagnostic Testing Technique — Trained by Techs

## Ordered field steps

1. Scan tool: check fault codes and misfire-counter data, rule out an electrical/ignition/fuel cause first.

2. Clear-flood crank test (screening only, not conclusive alone).

3. Relative compression test: high-amp clamp on starter circuit plus ignition-event pickup, compare cylinder-to-cylinder starter-current peaks.

4. Build or use a piezo stethoscope probe on a spare scope channel.

5. Install an in-cylinder pressure transducer in a reference cylinder's spark-plug hole on another channel.

6. Idle the engine and press the piezo probe to the suspect area, capturing both channels together.

7. Review a multi-cycle trend view first to confirm any anomaly is repeatable/periodic.

8. Zoom into one 720-degree window and apply the scope's phase-ruler tool, split into stroke-level or per-cylinder segments.

9. Identify where the pressure-transducer cylinder's exhaust valve opens and closes, and where the intake valve closes.

10. Overlay the piezo spike timing against those landmarks to identify which valve event the noise matches.

11. If it doesn't match the reference cylinder, repartition per total cylinder count and slide the window to find the actual source.

12. Use the scope-confirmed cylinder and valve side to justify pulling that bank's valve cover for a targeted inspection.

13. Confirm root cause visually before repair.

## Grab list

Tools: Scan tool, 4-channel professional automotive lab oscilloscope (PicoScope 7 Automotive or equivalent), high-amp starter current clamp, ignition-event pickup lead, in-cylinder pressure transducer, DIY piezo probe (spare knock sensor + banana-plug lead), basic hand tools for valve-cover removal

## Safety and escalation

Not a tow-worthy or safety-stopping condition on its own; escalate straight to teardown/repair once the scope method has pinpointed the specific cylinder and valve side, to avoid unnecessary parts-cannon replacement.

## Common misdiagnosis

Trying to localize a ticking noise purely by ear and pulling the wrong side's valve cover, or replacing parts on a hunch; dismissing a persistent tick as benign because drivability and idle quality are otherwise normal; assuming a clean clear-flood listen and a clean relative-compression trend together rule out a mechanical problem.

## Confirmation standard

Multi-cycle trend view proving the noise event is periodic and repeatable; phase-ruler correlation against a reference-cylinder pressure transducer proving exact valve-event alignment; final visual teardown at the valve cover confirming the physical wear/failure.

Owner note: An advanced/instrumented escalation tier, not a first-reach-for-every-ticking-noise-call tool — needs a 4-channel automotive lab scope with pressure-transducer and current-clamp accessories plus a hand-built piezo probe, a real cost/skill step above the basic belt-elimination + hand-spin method in tree 42.

# 44. Overheat first-triage — leak-volume, fan-operation, coolant-sample, and hot-upper/cool-lower hose tests to branch toward a flow-restriction cause before deeper leak- or head-gasket-specific diagnosis

Decision tree: Overheat complaint arrives → visually inspect radiator/hoses/water pump cold for an obvious external leak → open the reservoir/fill neck and note how much coolant is actually missing by topping off with water → if only a small amount is needed, a major external leak is unlikely, so branch toward a flow/circulation problem rather than a leak-hunt (if a large volume is needed, branch to the exact coolant leak localization tree) → start the engine, turn on A/C to command the electric fans and confirm both spin → drain a small coolant sample to confirm real antifreeze is present and check dilution → let the engine idle to full operating temperature while watching the gauge and overflow → hand-touch the upper radiator hose then the lower hose: hot upper + cool lower = coolant not circulating through the radiator → suspect a stuck-closed thermostat as the most likely cause, BUT this same symptom can also be produced by a failed water-pump impeller or a blocked/collapsed radiator core, so treat this as probable, not proven, until the thermostat is physically pulled/bench-tested or pump flow is separately verified → if head-gasket combustion-gas contamination is instead suspected, branch to the head-gasket suspicion stack tree.

Provenance: GMM-EX-049. Single elite trusted source (Rainman Ray's Repairs) demonstrates the full triage sequence end-to-end with real on-camera evidence.

## Evidence source

Source: Cooling System OVERHEAT! Antifreeze? Radiator? — Rainman Ray's Repairs

## Ordered field steps

1. Visually inspect the radiator, hoses, and water pump area cold for obvious external wetness/staining/leaks.

2. Lift the vehicle and check the underside for drips.

3. Open the coolant reservoir/fill neck to gauge how much coolant remains.

4. Slowly refill with water while tracking exactly how much it takes to top off.

5. Start the engine, idle, turn on the A/C to command the electric cooling fans, confirm both run.

6. Drain a small coolant sample from the radiator petcock to check color and concentration.

7. Let the engine idle to full operating temperature while watching the gauge and overflow behavior.

8. Hand-touch the upper radiator hose, then the lower radiator hose.

9. Conclude a probable stuck-closed thermostat, but before ordering parts, pull and bench-test (boil-test) the thermostat or perform a hose-squeeze flow check to confirm pump flow, rather than relying on the hose-touch test alone.

10. Cross-check any stored scan-tool DTC (e.g. an overtemperature/protection-mode code) as corroborating evidence of a real PCM-registered overheat event.

## Grab list

Tools: Flashlight/inspection light, water/coolant and funnel, drain cup, A/C controls to trigger fans, OBD-II scan tool capable of manufacturer-specific codes, basic hand tools for eventual thermostat/housing replacement

## Safety and escalation

If the customer reports the engine actually overheated and a protection-mode DTC is present, treat as a tow-in, not drive-it-in — continued driving on a confirmed overheat with a suspected stuck thermostat risks head gasket or head/block damage.

## Common misdiagnosis

Assuming any overheat means a coolant leak, without first quantifying how much coolant it actually takes to top off; treating "hot upper hose / cold lower hose" as automatic proof of a stuck-closed thermostat when it equally fits a failed water-pump impeller or a severely clogged/collapsed radiator core; overlooking fan-circuit failure as a standalone cause.

## Confirmation standard

Pull the thermostat and visually/bench-confirm it is mechanically stuck closed rather than condemning it on hose-feel alone; alternatively, squeeze the upper radiator hose by hand at operating temp; cross-check any stored overtemperature/protection-mode DTC.

Owner note: This is the missing "front door" of the overheat manual — decide leak-vs-no-leak and fan-vs-no-fan before jumping into the exact-leak-localization or head-gasket trees. This tree ends at "probable thermostat, needs physical confirmation," it does not itself confirm a repair. The source video does not demonstrate the standard hot/pressurized-cap safety procedure — that omission should not propagate; general cooling-system burn-hazard precautions still apply in full.

# 45. Parasitic drain on a module-dense vehicle — verify a flagged module's own input pins before condemning it, since another failing component can hold it falsely "on"

Decision tree: Set the vehicle up to sleep (defeat door/hood/trunk latch switches, remove the key from proximity, allow up to an hour for a module-dense car to settle) → clamp a low-amp meter/scope on the battery negative cable to read total parasitic draw → locate every fuse panel via the fuse-ID chart → probe directly across each fuse in millivolts without pulling any, working through every panel systematically until an outlier is found → convert the millivolt reading to actual current using a published fuse voltage-drop chart matched to fuse type/rating → pull the wiring diagram for that fuse to see what it powers → confirm by unplugging the flagged module directly and rechecking the amp clamp → BEFORE condemning the module, pull its full pinout diagram and back-probe every input pin with the key removed → an unexpected signal on an input pin means some OTHER failing component is holding the module "on," not the module itself → trace that unexpected signal to its real source and test the actual suspect component directly → repair the real root cause and re-verify the draw is gone.

Provenance: GMM-EX-050. Single elite trusted source (Trained by Techs) demonstrating the full method end-to-end with real oscilloscope/DMM evidence and a confirmed, verified repair.

## Evidence source

Source: Parasitic Drain Testing - Jaguar — Trained by Techs

## Ordered field steps

1. Set the vehicle up to sleep: open all doors/hood/trunk, defeat latch-integrated switches, remove the key from the vicinity, allow up to 45-60 minutes for a module-dense car to fully settle.

2. Clamp a low-amp meter or scope on the battery negative cable and read the total parasitic draw.

3. Locate every fuse panel using the printed fuse-ID chart.

4. Set a DMM to millivolts and probe directly across each fuse, one at a time, without ever pulling a fuse.

5. Work through every fuse in every panel systematically until the outlier is found.

6. Convert the millivolt reading into actual current using a published fuse voltage-drop chart matched to the fuse type and rating.

7. Pull the OEM wiring diagram for that fuse to see what it powers.

8. Confirm by unplugging the flagged module directly and rechecking the amp clamp.

9. Before condemning the module, pull its full pinout diagram and back-probe every input pin with the key removed from the ignition.

10. An unexpected signal on an input pin means another component is falsely holding the module "on" — do not replace the module yet.

11. Trace that unexpected signal to its real source and test the actual suspect component directly.

12. Repair the real root-cause component and re-verify the draw is gone with the amp clamp.

## Grab list

Tools: Low-amp clamp meter or scope, DMM with millivolt resolution, OEM wiring diagrams, a fuse voltage-drop reference chart, test light, basic hand tools for defeating door/hood/trunk latches

## Safety and escalation

Not a tow-worthy issue if the vehicle otherwise runs and drives; this is inherently a leave-behind or multi-visit diagnostic given the up-to-one-hour sleep-settle requirement common to any parasitic-draw job on a module-dense vehicle.

## Common misdiagnosis

Replacing the flagged module the moment fuse voltage-drop testing points to its fuse, without first checking whether some other failing component is falsely holding it "on"; jumping to blame visible aftermarket wiring/alarm accessories on sight without actually testing them.

## Confirmation standard

Amp-clamp draw drops to near zero when the flagged module is unplugged; back-probing the module's own input pins with the key removed shows only expected signals; draw stays eliminated after the actual root-cause component is repaired and the vehicle is re-soaked.

Owner note: Builds directly on the fuse voltage-drop isolation tree (GMM-KB-019) — this entry is the second half most techs skip: verify the flagged module's own inputs before replacing it. On module-dense/networked luxury platforms especially, a module can be an innocent victim of an upstream fault rather than the actual failure.

# 46. No crank, no start from a failed ignition switch — control-circuit fault (run/start feed dies specifically in crank), distinct from a starter-circuit voltage-drop fault

Decision tree: No crank with dash/accessories otherwise normal → connect a scan tool and read live ignition-switch/"System Power Mode" data (not just the mechanical key position) while cycling Off, Run, Accessory, Start → if the module reports an erratic/wrong state or accessories/dash flicker during key cycling → pull steering column covers and back-probe the ignition switch connector directly with a test light against the wiring diagram, checking every relevant output wire in every key position → find one output that is dim/dead specifically in crank while the dedicated start-signal wire and battery input both test fine → confirm with a brief direct jumper across the switch's input/output pins to bypass the switch → engine cranks/starts → replace the ignition switch and clear consequence codes on secondary circuits sharing the same feed.

Provenance: GMM-EX-051. Single elite trusted source (South Main Auto / Eric O.) demonstrating the full diagnosis end-to-end with real scan-tool screens, real wiring-diagram consultation, real test-light readings, and a live bypass confirmation that immediately fixes the symptom.

## Evidence source

Source: No Crank, No Start Diagnosis — South Main Auto LLC

## Ordered field steps

1. Confirm the complaint: key to Start, dash/accessories live, no crank.

2. Read live ignition-switch/power-mode data on a scan tool while cycling Off, Run, Accessory, and Start.

3. Note any erratic readback or dash/accessory flicker during key cycling.

4. Remove steering column covers to access the ignition switch connector.

5. Pull the wiring diagram to identify each switch pin's function.

6. Test-light the battery-feed input wire to confirm power is present in all positions.

7. Test-light the start-signal output wire while cranking to confirm it's working.

8. Test-light the run/start feed output wire across Run and Start, comparing against a known-good output wire, to spot an output that's dim or dead specifically in crank.

9. Confirm the diagnosis with a brief direct jumper across the relevant switch-connector pins to bypass the failed contact.

10. Replace the ignition switch and clear any consequence codes on circuits fed from the same switched feed.

## Grab list

Tools: GM-capable (or make-appropriate) scan tool with live ignition-switch/power-mode data, wiring diagram reference, test light, steering column trim tools, needle-nose pliers (confirmation-only bypass, not a repair method)

## Safety and escalation

Not a tow scenario for the diagnosis itself, fully mobile-van feasible with the right scan tool and a wiring diagram. Escalate/tow only if the required scan tool cannot read the vehicle's power-mode data on-site, or if switch replacement requires column disassembly beyond the tech's comfort/tool level.

## Common misdiagnosis

Condemning and replacing/reprogramming the PCM based on accessory flicker or programming failures that are actually caused by a failing ignition switch dropping its feed mid-cycle; assuming a no-crank complaint always means a starter-circuit or battery problem and jumping straight to a voltage-drop test on the high-current path.

## Confirmation standard

Live scan-tool ignition-switch-position/power-mode data cycled through all key positions; test-light or DMM checks on every output wire at the switch connector in every position; a brief direct-jumper bypass at the switch connector that makes the no-crank symptom disappear.

Owner note: Distinct from and complementary to GMM-KB-001's starter-circuit voltage-drop method — together the two entries now cover both major "no crank" fault domains (high-current starter circuit vs. ignition-switch control-signal circuit) from the same trust-tier channel family. Reinforces reading live ignition-switch/power-mode scan data before assuming the fault is mechanical/high-current, since skipping this check is exactly what led the first shop to wrongly condemn the PCM.

# Batch 08 completion status

Seven videos were processed via the public YouTube route (no authenticated session used, 7 background agents dispatched in parallel) into Process Extracts GMM-EX-045 through GMM-EX-051, and Agent Knowledge GMM-KB-042 through GMM-KB-046. Coverage added: a merged, two-source-corroborated engine accessory noise belt-elimination method; an instrumented piezo-probe + pressure-transducer escalation tier for exact cylinder/valve noise pinpointing; an overheat first-triage branch-logic tree that runs before the existing leak-localization and head-gasket trees; a parasitic-drain entry on module-dense vehicles that verifies a flagged module's own input pins before condemning it, cross-referencing the existing fuse voltage-drop method; and a no-crank entry for a failed ignition switch's control-signal circuit, complementing the existing starter-circuit voltage-drop tree. One source (ChrisFix, ball joint play check, GMM-EX-048) was extracted but not promoted — its clarity-only trust tier and likely overlap with the existing front/rear suspension inspection tree argued against a new entry; folding its wiggle-test mnemonic into that existing tree is flagged as an editorial option, not done automatically.

This closes all of Priority 3 (rows 36-38, 40-41, 43-44). Remaining: Priority 4 (rows 45-49), plus the final domain-coverage audit and the still-open no-crank-car-with-tow-escalate gap.

# 47. Quick engine-running cylinder power-balance and misfire-count test using a scope on battery/alternator voltage ripple (foundational literacy)

Decision tree: Suspected weak/misfiring cylinder but root cause (spark/fuel/compression) not yet isolated → clip scope Channel A to battery (or alternator if battery access/signal is poor) → run the cylinder-balance test wizard (load engine electrically via headlights/ancillaries for a clean signal, confirm cylinder count, confirm signal strength is strong) → start live capture at idle → read the per-cylinder % bar graph and misfire counter in real time → a cylinder sitting well below 100% with a misfire count climbing much faster than its neighbors relative to total samples is the weak cylinder → that result becomes the starting point for the spark/fuel/compression 3-way split (tree 18) to determine the actual root cause on that identified cylinder, rather than testing all cylinders blind.

Provenance: GMM-EX-053. Single-source, but the source is the scope manufacturer's own official training material demonstrating their own product end-to-end with a live proven fault injection.

## Evidence source

Source: Misfire diagnosis and cylinder balance testing using an oscilloscope — Pico Automotive Diagnostics

## Ordered field steps

1. Confirm the complaint is a suspected misfire/rough-running condition and a scope with cylinder-balance capability is available.

2. Connect the scope's voltage channel to the battery with clip leads; if inaccessible, connect to the back of the alternator instead (caution: belt-driven moving parts, possible heat proximity).

3. Launch the cylinder-balance test wizard.

4. Turn on headlights (and further ancillary loads if needed) for a strong signal.

5. Start the engine, let it idle.

6. Let the software auto-check charging voltage.

7. Confirm/enter cylinder count.

8. Check the signal-strength meter; add load until strong/green.

9. Finish the wizard, click Start.

10. Watch the live per-cylinder % bars, misfire counter, sample counter, RPM, and signal-strength meter together.

11. Identify any cylinder holding a depressed % with a disproportionately rising misfire count.

12. Discount bar movement seen during a signal-strength dropout — it may be noise, not an engine event.

13. Hand off the identified cylinder to the spark/fuel/compression 3-way split or other targeted test to pin the actual root cause.

## Grab list

Tools: Lab scope with a cylinder-balance/misfire-count software module (PicoScope + PicoDiagnostics or equivalent), battery voltage clip leads, laptop running the diagnostic software

## Safety and escalation

Not applicable — this is a rapid, non-invasive triage test performable roadside/in-van. Escalate only per whatever root cause is subsequently found on the flagged cylinder.

## Common misdiagnosis

Misreading a weak/dropping bar as "zero output = dead cylinder" — a fully non-firing cylinder still typically shows a nonzero % since it still acts as an air pump under compression; treating a bar's movement as a real engine event when it actually coincides with a signal-strength drop; testing with too little electrical load and getting an unreliable, noisy result.

## Confirmation standard

Repeat the capture after confirming strong signal strength; corroborate the flagged cylinder against a targeted follow-up test (coil current-ramp, injector balance test, or relative/leakdown compression test) before condemning a part.

Owner note: Fast first-pass triage tool to identify which cylinder is weak before spending time on deeper tests — pairs naturally as a front-end step to tree 18's spark/fuel/compression split. Requires the technician's scope/software to actually have a cylinder-balance module.

# 48. Relative compression testing via scope/software using only battery voltage leads, no current clamp (foundational literacy)

Decision tree: Technician suspects a cylinder-related mechanical issue or wants a fast pre-teardown compression health check → disable start (pull injector fuse or fuel pump relay) → clip scope leads to battery positive/ground only (no current clamp needed) → open the compression-test software module, select cylinder count, click Start → software auto-verifies battery voltage/polarity → crank with throttle held wide open → software reads the battery-voltage waveform produced by starter current draw across each cylinder's compression stroke and renders a per-cylinder relative-compression bar graph → a cylinder reading well below its peers flags a compression problem on that cylinder → escalate to a cylinder leakage test to localize the internal cause (valve/ring/head gasket) → optionally add a pressure transducer into one cylinder for an absolute BAR/PSI reading, which also anchors cylinder ID to the firing order.

Provenance: GMM-EX-054. Single-source but highest-authority tier — demonstrated end-to-end, on camera, by the scope manufacturer's own official training channel, including a deliberately induced fault used to prove sensor accuracy.

## Evidence source

Source: Relative compression testing using an automotive oscilloscope — picoautomotive

## Ordered field steps

1. Disable starting (pull injector fuse or fuel pump relay).

2. Connect only two leads to the vehicle: scope clip to battery positive, second clip to battery ground.

3. Open the Compression Test module.

4. Select cylinder count.

5. Click Start; software auto-checks battery voltage/polarity.

6. Crank the engine with the throttle held wide open.

7. Software renders a per-cylinder relative-compression bar graph (0-100%) from the battery-voltage waveform.

8. A cylinder well below its peers is flagged as a compression problem.

9. Optionally fit a pressure transducer into one cylinder for an absolute BAR/PSI cross-check and firing-order anchor.

10. If a cylinder is low, escalate to a cylinder leakage test to localize the actual internal cause — this test identifies WHICH cylinder is weak, not WHY.

## Grab list

Tools: Automotive lab oscilloscope (4-channel), compression-test software module, battery clip test leads, injector-fuse or fuel-pump-relay pull point, throttle-hold aid, optional pressure transducer + compression-hose adapter

## Safety and escalation

Not applicable as a standalone tow trigger — this is a fast screening test. Escalate to in-shop mechanical teardown only after this test plus a cylinder leakage test confirm and localize a genuine low-compression cylinder.

## Common misdiagnosis

Condemning a component from the relative-compression bar graph alone without a follow-up leakage test; running the crank step with the throttle closed, producing falsely low/uneven readings across ALL cylinders; testing on a weak battery, which can distort the waveform shape; forgetting to fully disable fuel/spark before cranking.

## Confirmation standard

Cylinder leakage test to localize the root internal cause once a low cylinder is flagged; optional pressure transducer for an absolute PSI/BAR cross-check; judge results against the vehicle's mileage/age-appropriate wear expectation rather than a universal hard number.

Owner note: This is the general/foundational technique entry, distinct from tree 43 which uses a similar current-clamp relative-compression channel as one step inside a different noise-pinpointing workflow. No numeric pass/fail threshold is stated by the manufacturer in the source video — do not invent a hard cutoff number.

# 49. Combined battery/starter/charging-circuit screening via a single scope-captured cranking event, auto-graded against battery spec — includes a smart-charge-alternator false-negative trap

Decision tree: Connect a voltage lead across the battery and a current clamp on the battery cable, both into a scope running an auto-grading battery-test module → enter battery spec (voltage, Ah/CCA rating, temperature, clamp range) → start capture → crank/start the engine → software reads the single voltage-dip/current-spike cranking waveform to grade battery capacity and internal resistance, calculates starter circuit resistance from the same event, then continues capturing post-start to grade charging voltage, ripple, and output current → if "not charging" is reported on an otherwise-healthy vehicle, suspect a smart-charge alternator's delayed engagement rather than a real fault; extend the capture window and rerun before condemning the alternator.

Provenance: GMM-EX-055. Single elite source, demonstrated end-to-end on one live vehicle; no second independent source and no failure-case example captured on camera — treat as a validated screening/triage workflow, not a validated fault-waveform-interpretation reference.

## Evidence source

Source: Test batteries, alternators and starter motors using an automotive oscilloscope — picoautomotive

## Ordered field steps

1. Connect a voltage test lead to Channel A across the battery.

2. Clamp a current probe around the battery cable into Channel B.

3. Open the Battery Test software module.

4. Set options: current clamp range, system voltage, battery standard, rated capacity, capture time (default ~10s, extend to 60s for smart-charge/delay alternators), and measured battery surface temperature.

5. Click Start to begin streaming capture.

6. Crank/start the engine during capture.

7. Read the auto-generated report: battery capacity/internal resistance/condition, starter circuit resistance, charging voltage/ripple/output.

8. If charging circuit reports "Not charging" with nothing else pointing to a real fault, extend the capture window and rerun before condemning the alternator.

9. Print/save the report.

## Grab list

Tools: 2-4 channel automotive lab scope, auto-grading battery-test software module, voltage test lead, current clamp (600A/2000A), battery surface temperature probe

## Safety and escalation

Not addressed in the source video — it's a screening/triage test only. A genuine failed-charging result, a battery graded "replace," or abnormally high starter resistance should escalate into a deeper diagnostic path (e.g. tree 1's no-crank voltage-drop tree), not be resolved by this screening test alone.

## Common misdiagnosis

Reading a "Not charging" result as a dead/failed alternator on vehicles with smart-charge (delayed-engagement) charging systems, when the alternator simply hadn't begun output yet inside the default ~10-second capture window.

## Confirmation standard

Rerun the capture with an extended window (20-60s) to confirm the alternator does begin charging; if it still never engages, that confirms a genuine charging fault requiring further isolation.

Owner note: This is a software-automated screening tool, not manual waveform-shape interpretation training — it never visually teaches what a bad diode's ripple sawtooth or a worn starter commutator's waveform looks like. Its one standout real teaching point is the smart-charge-alternator false-negative trap. Requires owning the scope + auto-grading software.

# 50. CAN-bus message glitch — differentiate induced electrical interference from an actual wiring/hardware fault using oscilloscope deep-memory zoom and bit rise-time analysis

Decision tree: Intermittent CAN-network symptom (routine skips, unintended behavior, message rejection) → connect a 4-channel scope to CAN-H/CAN-L, capture at a wide time base to catch many telegrams → spot extra spikes/ringing between normal packets on the live trace → freeze and use the scope's magnifier (not a new trigger/timebase) to zoom into the existing deep-memory buffer without losing the captured event → navigate to the anomaly and confirm which specific telegram, if any, was actually corrupted → zoom to microsecond/single-bit level and inspect rise time on the edges → clean sharp vertical edges = external interference is the cause → rounded/lazy rise time = genuine wiring/connector/hardware fault on the network itself, not interference → also rule out network saturation (~35-40%+ of baud rate) as a systemic, non-fault cause of rejected messages.

Provenance: GMM-EX-056. Single elite trusted source (the automotive-oscilloscope tool manufacturer's own official training channel), technique demonstrated end-to-end on camera with real scope UI evidence.

## Evidence source

Source: An Introduction to CAN Bus — picoautomotive

## Ordered field steps

1. Identify the CAN network(s) involved (a vehicle can have up to six separate CAN networks).

2. Connect scope channels to CAN-High and CAN-Low.

3. Set a relatively slow screen time (e.g. 50ms/division) to capture a broad window of normal traffic plus any transient glitch.

4. Watch the live trace for extra spikes/ringing outside normal telegram groups.

5. Freeze the capture once the anomaly is in the buffer — do not just change time base and re-trigger.

6. Open the magnifier/zoom tool and expand into the existing buffer.

7. Navigate to locate each glitch event.

8. For each glitch, zoom into the adjacent telegram to confirm whether that specific message was actually corrupted.

9. Reduce the time base further (e.g. 10us/division) to isolate a single bit edge.

10. Read the rise time: sharp/vertical = clean signal, interference is the likely cause; sloped/rounded = the network itself has a hardware/wiring/connector fault.

11. If interference is confirmed, use spare scope channels to probe suspected switched-load components for correlated current/voltage spikes.

12. Separately consider network saturation (~35-40% of baud-rate capacity) as a possible non-fault cause of rejected/skipped messages.

## Grab list

Tools: 4-channel automotive lab oscilloscope with deep memory buffer and magnify/zoom feature (PicoScope-class), CAN-H/CAN-L probe/breakout leads, laptop with scope software, spare scope channels + probes for correlating suspected interference-source components

## Safety and escalation

Not addressed in the source video (bench demo only, no case study) — apply standard escalation judgment based on whatever root cause the technique isolates.

## Common misdiagnosis

Assuming any visible spike/glitch on the CAN trace means the network wiring/module is bad — many glitches are benign induced interference that never actually corrupts a message; treating message rejection/network dropouts as a component fault when the actual cause is network saturation.

## Confirmation standard

Zoom to the specific telegram adjacent to a suspected glitch and verify whether it is actually malformed/corrupted; zoom to single-bit/microsecond level and check rise time cleanliness; cross-correlate a suspected interference source component on a spare scope channel.

Owner note: This entry is a scope-technique skill, not a case study — pairs with trees 4, 23, 35, and 38 (all no-comm/network case studies) as an added interpretation tool, not a duplicate of any of them. IMPORTANT: despite its title, this source video does NOT supply CAN-H/CAN-L differential-pair theory or termination-resistor fundamentals — if a true CAN-bus-fundamentals literacy entry is still needed, a different source video would be required; this is a residual gap, not filled here.

# Appendix — Recommended Mobile Diagnostic Tool Kit (reference, not a decision tree)

Source: Everything You Need for the ULTIMATE Diagnostic Cart — HumbleMechanic (GMM-EX-052, extracted but not promoted to a numbered Agent Knowledge tree, since it is equipment reference content with no symptom-to-root-cause structure).

Electrical test and repair (mobile-van feasible: yes): multimeter, PowerProbe family (including a wire-piercing Short Finder), LOADPro voltage-drop-under-load tester, Amp Hound current-draw tester, amp clamp, crimpers/strippers/ferrule kit, wire and connector repair stock, soldering station, label maker.

Scan tools (mobile-van feasible: yes): a primary make-appropriate scan tool with a laptop interface, plus a generic backup scanner for other makes.

Scope and graphing (mobile-van feasible: yes, with modification): a lab oscilloscope (handheld unit is more van-native than a boom-mounted cart rig), plus a parasitic-draw graphing accessory.

Mechanical and engine-adjacent (mobile-van feasible: yes): NOID light kit, manometer for crankcase pressure/PCV diagnosis, borescope, paint thickness gauge.

Thermal and temperature (mobile-van feasible: yes): AC differential thermometer kit, thermal imaging camera, infrared temp gun.

Battery and power (mobile-van feasible: partial): handheld battery tester and a battery maintainer are portable; a cart-mounted UPS is not a van need.

Cart and organization furniture (mobile-van feasible: no): rolling toolbox chest, monitor mount, mounted magnifiers, connector storage bins — GMM's van equivalent should be a compact modular bag/case system carrying the instruments above, not this shop-bay furniture.

Genuinely new tool categories flagged by this source, not otherwise referenced elsewhere in this manual: manometer for crankcase pressure/PCV diagnosis, LOADPro voltage-drop-under-load tester, Amp Hound/fuse-loop current-draw method, magnetic field viewer for wheel-bearing sensors, PowerProbe Short Finder, and the parasitic-draw graphing box accessory.

# Batch 09 completion status

Five videos were processed via the public YouTube route (no authenticated session used, 5 background agents dispatched in parallel) into Process Extracts GMM-EX-052 through GMM-EX-056, and Agent Knowledge GMM-KB-047 through GMM-KB-050. Coverage added: a foundational cylinder power-balance and misfire-count scope technique using only battery-ripple sampling; a foundational relative-compression scope technique needing no current clamp; a narrowly-scoped, heavily-caveated battery/starter/charging screening entry (confidence held at 3/5 since the source teaches an automated software wizard, not manual waveform-shape interpretation); and a CAN-bus glitch-versus-wiring-fault differentiation technique via oscilloscope deep-memory zoom, with an explicit flag that true CAN differential-pair fundamentals remain a residual, unfilled gap. One source (HumbleMechanic's diagnostic-cart tour) was extracted but not promoted to a numbered tree since it is equipment/reference content, not a diagnosis case — captured instead as a standalone Recommended Tool Kit appendix, organized by mobile-van feasibility, with several genuinely new tool categories flagged for the manual (manometer, LOADPro, Amp Hound, magnetic field viewer, wire-piercing short finder, parasitic-draw graphing box).

This closes ALL of Priority 4 (rows 45-49) and, with it, the entire 56-source queue. Manual is now 50 promoted Agent Knowledge trees plus one reference appendix, drawn from 56 verified sources. Remaining before final completion: the 16-domain coverage audit, a final full-manual render pass, and the still-open gap for a genuine mobile no-crank car service call demonstrating a tow/escalate decision point (opened by GMM-EX-021 in Batch 03).

# Front-End Diagnostic Expansion — Complete Routing Matrix

Use this routing page before condemning a front-end part. A wheel-wiggle position is only a starting clue; a helper must watch the exact joint, hub, arm, shaft, or mount for relative movement, and the suspension must be supported in the design-specific loaded or unloaded state required by service information.

Full-lock rhythmic click/pop under light power → GMM-KB-051, outer CV joint branch.

Whole-vehicle shudder under acceleration that stops promptly off-throttle → GMM-KB-051, inner CV/plunge-joint branch; also rule out engine/transmission mounts.

Play or clunk during a 3-and-9 wheel rock → GMM-KB-052; observe outer tie rod, inner tie rod, rack housing/mounts, knuckle, and hub rather than calling it a tie rod automatically.

Play during a 12-and-6 wheel rock or vertical pry test → GMM-KB-053 plus existing GMM-KB-010; separate upper/lower ball-joint socket motion from hub-to-knuckle bearing motion.

Clunk, braking shift, wander, or unstable alignment with no loose joint → GMM-KB-053; inspect control-arm/wishbone/thrust-arm bushings, arms, and subframe.

Small sharp rattle over alternating bumps/driveway transitions → GMM-KB-054; neutralize sway-bar preload and move the bar itself while watching the links and frame bushings.

Clunk felt through the steering wheel, steering bind, or steering-wheel motion before the tires respond → GMM-KB-055; inspect steering intermediate-shaft U-joints, slip joint, pinch hardware, column/rack mounts, and then tie rods.

After a curb, pothole, or collision: pull, gross toe change, rubbing, off-center wheel, or wheel wobble → GMM-KB-056; inspect before driving and escalate to alignment/frame measurement when geometry is uncertain.

Road-speed vibration that is not throttle-, brake-, or turn-sensitive → GMM-KB-057; separate tire/wheel imbalance, runout, loaded road-force variation, centering, and existing GMM-KB-010 hub/bearing causes.

Grinding/hum that changes with corner load, ABS anomaly, or hub play → existing GMM-KB-010 wheel-bearing/hub tree.

Brake pull, pulsation, heat, or scrape → existing complete brake-inspection tree; do not treat brake symptoms as suspension faults.

Bounce, bottoming, leaking damper, broken spring, strut-mount noise, or uneven ride height → existing GMM-KB-006/GMM-KB-008 suspension and shock/strut trees.

Safety rule: tow for a joint/shaft near separation, loose steering hardware, severe pull/wander or gross toe, steering bind/delayed response, tire contact, damaged tire/wheel, cracked/bent load path, or uncertain post-impact structure.

# 51. CV axle / CV-joint diagnosis — outer full-lock click versus inner acceleration shudder

Decision tree: Front-end click, shake, or grease sling → inspect inner/outer boots and shaft first → rhythmic click/pop only on low-speed full-lock turns under light power points to an outer CV joint → whole-vehicle low-frequency shake only under acceleration/load that smooths promptly off-throttle points to an inner CV/plunge joint → vibration purely road-speed dependent with no throttle effect points back to tire/wheel/hub → confirm the suspect axle physically and rule out tire rub, loose wheel/brake shield, hub bearing, engine/trans mounts, bent wheel, and steering/suspension play before replacement.

Provenance: GMM-EX-057 and GMM-EX-058. Two complementary visual sources: one clear outer-boot/full-lock procedure and one inner-versus-outer road-test symptom split. Turn-direction side rules are not treated as absolute.

## Evidence source

How to Diagnose a Bad CV Axle — 1A Auto: https://www.youtube.com/watch?v=zknhHQZQnXo

Inner or Outer CV Joint From the Driver's Seat — European Auto Repair: https://www.youtube.com/watch?v=zJKOWHXB2uM

## Ordered field steps

1. Capture exactly when the symptom occurs: turn angle, throttle/load, road speed, braking, and coast.

2. Inspect both inner and outer boots, clamps, grease sling, shaft damage/corrosion, and impact marks.

3. In a clear lot, windows down, perform slow circles at full lock both directions under light throttle; a repeatable wheel-speed click/pop supports outer-joint wear.

4. On a safe straight road, apply light/moderate acceleration; a large shake that stops promptly on lift-off supports inner-joint wear.

5. Chock, lift, and support correctly; compare both axles while checking hub, tie rod, ball joint, and mounts.

6. Do not select the side by turn direction alone; confirm with physical evidence and manufacturer service information.

7. After repair, confirm no click at lock, no load-dependent shake, no leaks, and correct fastener torque/alignment state.

## Grab list

Tools: wheel chocks, inspection light, gloves, safe road-test route/lot, floor jack, jack stands/lift, helper, manufacturer service information.

## Safety and escalation

No-drive/tow for severe shake, binding, grinding, loss of steering control, a visibly fractured/bent shaft, or a joint near separation. Never work under a vehicle supported only by a jack.

## Common misdiagnosis

Calling any turn click a CV axle without ruling out tire/body contact, brake shield/hardware, loose wheel, hub, or steering play; calling any acceleration shake an inner CV without checking engine/trans mounts and wheel/tire condition.

## Confirmation standard

Repeat both full-lock-circle and load/on-off-throttle tests. The original symptom must be gone and no new leak or vibration may be present.

Owner note: A torn boot proves contamination risk, not always that the joint is already noisy. Inspect condition and part availability before quoting boot, joint, or complete axle options.

# 52. Inner/outer tie-rod and steering-play localization — separate joint play from rack, hub, and ball-joint movement

Decision tree: Steering clunk/looseness/wander or uneven toe wear → check tire/wheel hardware first → safely raised 3-and-9 wheel rock with a helper touching/watching each joint → relative motion at outer stud/socket = outer tie-rod end → motion at inner joint/shaft inside rack boot = inner tie rod → rack housing/mount movement = rack/bushing/subframe issue → whole knuckle/hub movement without tie-rod socket movement = hub/ball-joint branch → any replaced tie rod requires final alignment, not just thread counting.

Provenance: GMM-EX-059 with GMM-EX-006 and GMM-EX-063 supporting. Elite HumbleMechanic demonstration plus the existing professional suspension sweep and collision-geometry case.

## Evidence source

How To Check and Replace a Bad Tie Rod — HumbleMechanic: https://www.youtube.com/watch?v=0SsV_pbEM7A

## Ordered field steps

1. Verify the complaint; inspect tire pressure/wear, lug torque, wheel damage, and collision evidence.

2. Chock, lift, and support at service-data-approved points.

3. Rock the wheel at 3-and-9 while a helper observes/touches the outer end, inner joint/rack boot, rack housing/mounts, knuckle, and hub.

4. Repeat with controlled small steering inputs on the ground when safe.

5. Localize relative movement instead of relying on where the hand feels a clunk.

6. Inspect boots, corrosion, bent rods, loose jam nuts/hardware, rack leakage, and witness marks.

7. Compare side-to-side and measure against service data when a dial-indicator procedure exists.

8. After repair, confirm no abnormal play, torque fasteners, and obtain a four-wheel alignment.

## Grab list

Tools: wheel chocks, floor jack, jack stands/lift, light, gloves, helper, pry bar, dial indicator if specified, service information, alignment access.

## Safety and escalation

No-drive/tow for a joint near separation, loose steering hardware, gross toe change, rack/shaft bind, tire contact, or unsafe wander/pull.

## Common misdiagnosis

Treating all 3-and-9 movement as a tie rod when the hub, knuckle, rack housing, or mounts are moving; watching only the outer end and missing an inner joint.

## Confirmation standard

No abnormal relative joint movement; steering input produces immediate knuckle movement; final alignment printout is within manufacturer specification and the road test has no wander/clunk.

Owner note: Counting turns or paint-marking threads is only a rough starting point for toe and never replaces alignment.

# 53. Upper/lower ball-joint plus control-arm/wishbone-bushing diagnosis — design-aware load/unload and observed relative movement

Decision tree: Front clunk, wander, braking shift, or tire wear → identify the suspension design and which ball joint carries spring/vehicle load → support the vehicle so the joint under test is unloaded per service information → 12-and-6 rock and controlled pry/load while a helper watches upper joint, lower joint, hub, and control-arm/wishbone bushings → relative stud/socket movement at one joint confirms that joint; hub/rotor moving together relative to knuckle points to hub bearing; arm/sleeve movement, torn rubber, or failure to spring back to center points to control-arm/wishbone/thrust-arm bushing → use measured service limits where supplied, never a universal zero-play rule.

Provenance: GMM-EX-006, GMM-EX-009, GMM-EX-048, and GMM-EX-061. Four-source merged tree covering the professional sweep, controlled dynamic load, upper/lower wiggle/pry visual, and bushing spring-back demonstration.

## Evidence source

Professional steering/suspension inspection — Royalty Auto Service: https://www.youtube.com/watch?v=pS-GDeZyv44

Advanced ball-joint/silent-block load test — Garage Gurus: https://www.youtube.com/watch?v=H78F1hFEPFw

Basic upper/lower ball-joint play check — ChrisFix: https://www.youtube.com/watch?v=62bVn7E8gyk

Control-arm bushing condition check — European Auto Repair: https://www.youtube.com/watch?v=g58K51IwOgM

## Ordered field steps

1. Identify every upper/lower joint and arm plus the spring/load path from service information.

2. Inspect boots, studs/nuts, arms for bend, bushings for cracking/separation/leakage/shift, subframe mounts, and tire contact.

3. Chock/lift/support in the exact configuration that unloads the load-carrying joint; repeat loaded if the manufacturer requires it.

4. Rock at 12-and-6 while a helper watches each joint and the hub-to-knuckle interface.

5. Apply a small controlled pry/load under the tire or at the approved arm location; observe relative movement rather than only feeling the tire.

6. Load each bushing in its working direction: healthy rubber deforms and returns; failed rubber shows excess travel, knock, separation, or stays displaced.

7. Use a dial indicator and OEM limit where specified.

8. Rule out tie rod, hub, strut mount, sway link, brake hardware, loose subframe, and collision geometry.

9. Confirm after repair with a repeat play test, ride-height torque where required, alignment, and road test.

## Grab list

Tools: service information, wheel chocks, jack/stands or lift, light, gloves, helper, small pry bar, dial indicator/magnetic base where specified, alignment access.

## Safety and escalation

No-drive/tow for a joint near separation, loose/missing retaining hardware, cracked/bent arm, separated load-bearing bushing, shifted wheel/arm, tire contact, or collision deformation.

## Common misdiagnosis

Using 12-and-6 as an automatic ball-joint verdict when hub play can feel identical; testing a load-carrying joint while still loaded and getting a false negative; condemning surface weather-checking without separation or excess movement.

## Confirmation standard

Observed relative movement identifies the exact joint/bushing and exceeds its vehicle-specific limit, or the bushing separates/fails to recenter. Repeat test and alignment/road test must confirm the repair.

Owner note: Wishbone, control arm, tension strut, radius arm, and thrust arm are naming/layout variants. Record the exact failed position; do not quote a generic front-end kit without evidence.

# 54. Sway-bar end-link and bar-bushing clunk isolation — remove preload and move the bar, not just the link

Decision tree: Small sharp rattle/clunk over alternating bumps or driveway transitions → inspect link boots/nuts and bar-to-frame bushings → put suspension in the service-data-approved load state so bar preload does not mask play → grasp the sway bar near each link and move it up/down while touching/watching the joint → relative click/socket motion confirms the link → excess bar movement/contact at a frame bushing confirms bushing/mount issue → if silent/tight, continue to strut mount, spring seat, control-arm bushing, ball joint, tie rod, brake hardware, exhaust/shield, and subframe branches.

Provenance: GMM-EX-060 merged with existing GMM-EX-010 suspension-noise inspection.

## Evidence source

Correct way to check sway bar links — European Auto Repair: https://www.youtube.com/watch?v=0vAek7t1t1g

Checking suspension noise — EricTheCarGuy: https://www.youtube.com/watch?v=scFbb43fwqk

## Ordered field steps

1. Reproduce the noise at low speed and note whether single-wheel/alternating bumps trigger it.

2. Chock and support safely; unload or equally load both sides as required to neutralize bar twist.

3. Inspect link boots, studs, nuts, brackets, bar shape, frame-bushing rubber, and witness marks.

4. Hold the sway bar itself close to the end link and move it straight up/down while a helper touches/watches both sockets.

5. Load the bar gently while observing bar-to-frame bushings for knock or excess shift.

6. Rule out nearby sources; use chassis ears or controlled component loading if intermittent.

7. Repeat the same bump route and physical test after repair.

## Grab list

Tools: wheel chocks, jack/stands or lift, light, gloves, helper, small pry bar, optional chassis ears, service information.

## Safety and escalation

Keep clear of bar pinch points. Tow when broken/loose components contact tire, brake, steering, axle, or body, or when another unsafe joint is found.

## Common misdiagnosis

Squeezing/wiggling the link body and missing play held by preload; replacing a link because it looks old; overlooking loose hardware, frame bushings, brake hardware, or strut/spring noise.

## Confirmation standard

Feel/see the exact link socket or bar bushing move and reproduce the click; post-repair physical check and original route are quiet.

Owner note: The required loaded/unloaded condition varies. Follow service information instead of forcing one universal setup.

# 55. Steering intermediate-shaft/U-joint clunk or bind — localize delayed input between wheel, shaft, rack, and front wheels

Decision tree: Clunk felt at the steering wheel from tiny inputs or bumps, or steering-wheel input with delayed tire response → two-person dry-park test → watch/touch column/intermediate-shaft joints, slip section, rack mounts, and tie rods → movement across a U-joint cup/yoke or clunk at the shaft confirms looseness; stiff articulation confirms binding → if shaft is tight, continue to column, rack mounts/internal rack, tie rods, hub, and suspension branches → any joint near separation is immediate no-drive/tow.

Provenance: GMM-EX-062. Clear professional visual showing a sample joint test and an actual near-separated lower steering-shaft joint.

## Evidence source

Steering Feels & Sounds Clunky? Diagnose Steering Shaft Joints — 1A Auto: https://www.youtube.com/watch?v=iLNnPkjxuk4

## Ordered field steps

1. Verify the clunk/bind with small steering inputs; do not cycle violently at full lock.

2. Watch a front tire while a helper makes tiny steering-wheel movements and note delay/dead zone.

3. From a safe position, watch/touch each accessible shaft U-joint and slip joint from firewall to rack/box.

4. Check pinch bolts, corrosion, witness marks, collapsed/slipped sections, and rack/column mounts.

5. Twist across accessible/removed joints and articulate them; there must be no cup/yoke play or binding.

6. Distinguish shaft play from tie-rod/rack/knuckle/hub movement.

7. Follow OEM indexing, steering-wheel restraint, battery/airbag/clock-spring, and fastener procedure.

8. Confirm immediate tire response and smooth, quiet travel after repair.

## Grab list

Tools: helper, wheel chocks, inspection light, gloves, manufacturer service information.

## Safety and escalation

Any separating, cracked, grossly loose, or binding steering shaft/U-joint, loose pinch hardware, or loss of steering response is no-drive/tow. Secure the steering wheel and follow clock-spring/airbag precautions before disconnection.

## Common misdiagnosis

Replacing tie rods or rack because the clunk is heard low in the dash without touching the shaft; mistaking rack-mount movement for shaft-joint play.

## Confirmation standard

No relative movement at joints/mounts, no binding, immediate front-wheel response, and no clunk on repeat dry-park/road test.

Owner note: Never rotate the steering wheel and rack independently after disconnection; follow the exact OEM procedure.

# 56. Post-curb/pothole/collision front-end triage — separate wheel damage from steering/suspension/structural geometry

Decision tree: Impact followed by off-center wheel, pull/wander, rubbing, wobble, noise, or abnormal toe → stop and inspect tire/wheel before road test → supported spin/runout plus witness-mark and steering-clearance check → compare both sides for wheelbase/setback, ride height/gaps, arms, tie rods, knuckle, strut, subframe/rack mounts, hub, joints, and bushings → substitute a known-good wheel/tire only when safe → persistent contact, gross geometry change, or visually uncertain bent part requires alignment printout and/or frame/body-point measurement → repair confirmed faults, align, then road-test.

Provenance: GMM-EX-063 with GMM-EX-006, GMM-EX-010, and GMM-EX-011 supporting. Real curb-impact case promoted as triage/escalation, not as permission for trial-and-error parts replacement.

## Evidence source

Inspecting Collision Damage (Suspension) — EricTheCarGuy: https://www.youtube.com/watch?v=dSWdf_7HcqI

## Ordered field steps

1. Record impact direction, wheel angle, speed, and immediate symptom changes.

2. Inspect tire sidewall/tread, wheel, lug seating, brake/hose/ABS wiring, and leaks before moving.

3. Chock/lift/support; hand-spin and measure wheel/tire runout per service information.

4. Check 12-and-6 and 3-and-9 play while observing components.

5. Move steering safely through its range engine-off and locate rubbing/witness marks.

6. Compare left/right geometry and all load-path components, including subframe/rack mounts.

7. Use a known-good wheel/tire only when it can be installed and rotated without risk.

8. Do not judge a part straight by eye; obtain four-wheel alignment and frame/body-point measurements when readings, contact, or centering are abnormal.

9. Repair evidence-backed faults, align, and road-test only when safe.

10. Advise recheck of the impacted bearing and tire because delayed failure is possible.

## Grab list

Tools: wheel chocks, jack/stands or lift, light, helper, torque wrench, straightedge/dial indicator when specified, known-good wheel/tire, alignment/frame-measurement access, OEM dimensions.

## Safety and escalation

Tow for tire/wheel structural damage, rubbing, severe pull/wander, gross toe, steering bind/delayed response, leak, cracked/bent load path, loose joint, airbag event, or uncertain structure.

## Common misdiagnosis

Road-testing before safety is established; blaming a bent wheel for all contact; calling a part straight by eye; replacing parts until alignment comes in without measuring structure/subframe.

## Confirmation standard

Alignment/frame measurements document geometry; no contact through travel; runout and joint play are within spec; safe road test tracks, brakes, and returns normally with centered wheel.

Owner note: Alignment is a measurement/final confirmation, not a substitute for mechanical inspection. Preserve before/after printouts and warn that hidden collision damage can expand the estimate.

# 57. Road-speed tire/wheel vibration — distinguish imbalance, loaded road-force variation, runout, and hub/bearing causes

Decision tree: Vibration rises with road speed → note steering-wheel versus seat/floor location and sensitivity to throttle/brake/turning → inspect pressure, tire damage/wear, debris, wheel damage/centering/lug torque → check hub/bearing play and radial/lateral runout → conventional balance correct but vibration remains = test road-force variation under load → match-mount tire stiff/high spot to wheel low spot per machine → remeasure, final-balance, and road-test → replace tire/wheel if still outside current machine/OEM/application limits → if vibration changes mainly with throttle, braking, or corner load, leave this branch for CV/driveline/brake/hub diagnosis.

Provenance: GMM-EX-064 paired with existing GMM-EX-011 hub/wheel-bearing confirmation tree.

## Evidence source

Diagnosing and repairing a tire vibration (road-force variation) — CollegeAutomotiveClasses: https://www.youtube.com/watch?v=6_RHSQt--TQ

## Ordered field steps

1. Record speed band, vibration location, throttle/coast/brake/turn sensitivity, and recent tire work/impact.

2. Inspect/set pressure; check bulge/separation, irregular wear, flat spots, debris, bent/cracked wheel, centering surfaces, and lug torque.

3. Check steering/suspension and hub/bearing play; hand-spin and measure radial/lateral runout using service data.

4. Perform conventional dynamic balance with correct centering.

5. If vibration persists, road-force test at specified pressure.

6. Read loaded road force and wheel runout.

7. Mark tire stiff/high point and wheel low point per machine; match-mount, reinflate, and repeat measurement while protecting TPMS.

8. Final-balance after match-mounting.

9. Replace tire/wheel if still outside current limits.

10. Confirm across the original road-speed band and recheck lug torque.

## Grab list

Tools: pressure gauge, tire inspection tools, torque wrench, wheel chocks, jack/stands, dial indicator and service information as needed; road-force balancer and tire machine for shop escalation.

## Safety and escalation

No-drive/tow for a bulged/separated/damaged tire, cracked/severely bent wheel, loose wheel/hub, severe wobble, or unsafe steering. Road-force correction requires trained shop equipment.

## Common misdiagnosis

Assuming a zero-ounce balance means the assembly must run smoothly; balancing a tire with excessive radial-force variation or a bent wheel; blaming the hub before checking centering/runout; using one internet road-force number as a universal limit; missing throttle-dependent inner-CV or braking-dependent rotor issues.

## Confirmation standard

Runout/play within vehicle limits, road-force within current machine/OEM/application limit after match-mount/final balance, and symptom absent on repeat road test.

Owner note: The demonstrated case improved from 35 lb to 13 lb, but those numbers are evidence from one assembly—not a universal threshold.

# Front-End Expansion Completion Status

Eight videos were fully processed through metadata, transcript, and real-frame review into GMM-EX-057 through GMM-EX-064. Seven decision trees were promoted as GMM-KB-051 through GMM-KB-057. The complete pipeline now contains 64 processed sources and 57 promoted diagnosis trees. This expansion closes the explicit CV axle, inner/outer tie rod, upper/lower ball joint, control-arm/wishbone bushing, sway-bar link/bushing, steering-shaft, collision/alignment, and road-force tire-vibration gaps while preserving the existing wheel-hub/bearing, strut/shock, general suspension, and brake trees instead of duplicating them.

# Four-Domain Whole-Car Expansion — Completion Status

Transmission/driveline (GMM-KB-058 through 063 and GMM-KB-065), exhaust/restriction (GMM-KB-064), and HVAC (GMM-KB-070 through 074) are now fully processed — see the sections above. The four residual coverage gaps (CAN-bus fundamentals, tire tread/TPMS, brake hydraulics, and a real no-crank/tow case) are closed as GMM-KB-066 through 069. Hybrid/EV high-voltage safety was permanently removed from this manual's scope per ownership decision: Gold Mobile Mechanic does not service hybrid/EV vehicles.

# Transmission and Driveline Expansion — Complete Routing Matrix

Suspect an automatic-transmission electronic control component (TFT sensor, shift solenoid, TCC solenoid, line-pressure solenoid) → GMM-KB-058 for the scan-tool-guided test sequence; if resistance and behavior disagree on a ground-switched circuit, cross to GMM-KB-065 for the current-limiting short pitfall a static ohms check will miss. Driveline noise, vibration, or leak → GMM-KB-059 (U-joint/carrier bearing hand-flex-through-dial-indicator) or GMM-KB-060 (rear differential howl) by symptom location; a 4WD-specific leak or fault light → GMM-KB-061 (GM transfer case). Manual-transmission internal noise → GMM-KB-062 to isolate input-shaft/main-shaft/countershaft bearing by operating condition; a suspected worn/slipping clutch before a $1,500-plus R&R → GMM-KB-063 for the pedal-test-first confirmation sequence.

# 58. Suspect automatic-transmission electronic control component (sensor/solenoid) -- scan-tool-first test sequence before parts replacement

Decision tree: When a shift-quality complaint, code, or suspect TCM-controlled component (TFT sensor, speed sensor, on/off or PWM shift solenoid, TCC solenoid, line-pressure solenoid) points at the electronic control system, use the scan tool's guided component data first, then run the specific built-in test for that component type (resistance/duty-cycle/functional pressure), cross-check against current OEM specs, and always close out a repair with the platform-correct adaptive relearn.

Provenance: GMM-EX-065. Elite single-source (Snap-on's own national diagnostic trainer, live scan-tool demo on a real 2014 Ford F-250 6R140)

## Evidence source

Automatic Transmission Controls and Testing - Snap-on Live Training Episode 35 — Snap-on Diagnostics: https://www.youtube.com/watch?v=s9-kUJtwgFs

## Ordered field steps

1. Pull Component Information via scan tool guided test for the exact platform.

2. Shift solenoids: key off, disconnect, run Resistance Test vs OEM spec.

3. TCC solenoid: yellow to TCC(+), black to known-good ground, road test at cruise, watch Duty Cycle Test step up to OEM lockup threshold.

4. Line-pressure solenoid: gauge on line-pressure port, command pressure via scan tool, compare to OEM chart, respect max-pressure limit.

5. On applicable platforms run Transmission Characterization then Solenoid Bands to ID the exact part before ordering.

6. After any repair, run the platform-specific adaptive relearn -- preconditions vary drastically by manufacturer.

7. If electrically good but complaint persists, check for pinched harness or internal metal contamination before condemning the transmission or reordering parts.

## Grab list

Tools: Scan tool with guided component tests + scope/meter, DVOM, 0-500 PSI pressure gauge/transducer, back-probe pins, current OEM service info

## Safety and escalation

Escalate to shop teardown when the transmission requires disassembly (pan/valve body drop) to reach or confirm a component, or when internal contamination/metal debris is suspected. Verify OEM procedure before any adaptive reset since preconditions are strictly platform-specific.

## Common misdiagnosis

Condemning an electrically-good solenoid without checking for internal mechanical contamination; treating a component that 'won't test correctly' as bad before checking the harness for a factory pinch point; testing a dual-range NTC-style TFT sensor by voltage alone (can read the same voltage at two very different temperatures); assuming any shift-delay complaint is a solenoid before considering worn clutch packs

## Confirmation standard

TCC: duty-cycle behavior during a real road test at operating temperature, not a static resistance check alone. Shift solenoid: in-spec resistance is necessary but not sufficient -- confirm live scan-tool PID mirrors commanded state and the fault clears after the required adaptive relearn.

Owner note: Every specific number in this source (resistance, duty-cycle %, pressure, band/part numbers) belongs to the specific vehicle shown and must never be reused as a universal spec -- enforce via OEM/service info lookup for the vehicle actually in the bay. Cross-reference: GMM-KB-065 shows a real case where this resistance-test-first method would be fooled -- a shorted solenoid that trips current-limiting protection reads deceptively via a static ohms check; use KB-065s amp-clamp/scope method when resistance and behavior disagree.

# 59. U-joint and driveline inspection -- field hand-flex triage through precise dial-indicator confirmation, plus center support (carrier) bearing

Decision tree: Driveline noise/vibration/leak complaint -> in the field, grab/flex each accessible U-joint in place and flex/inspect the carrier (center support) bearing for separation -> if suspect, remove the driveshaft and confirm precisely with a zeroed dial indicator measuring axial and radial U-joint endplay plus slip-yoke spline play against current OEM spec (hand-feel alone is below the real spec threshold in both directions -- it can miss real wear AND flag a joint that's actually fine) -> repair and re-verify.

Provenance: GMM-EX-068 and GMM-EX-069. Merged two complementary sources: an elite WSU instructor's precise dial-indicator measurement method (EX-068) and a clear field hand-flex triage/carrier-bearing demo (EX-069) -- together they cover triage through confirmation. No numeric OEM spec was given in either source; every number shown belongs to the specific joints on screen.

## Evidence source

Universal Joints - Part 4: U-joint and Driveline Inspection — WeberAuto: https://www.youtube.com/watch?v=irTL3USCZdc

How to Diagnose Truck Drive Shaft Problems - Vibrations and Noise — 1A Auto: Repair Tips & Secrets Only Mechanics Know: https://www.youtube.com/watch?v=iDPp0MqR2bc

## Ordered field steps

1. In the field: grab/twist each accessible U-joint in place, feel for tight-loose-tight catching; flex the carrier bearing for free hinge/separation (unsafe if separating).

2. If suspect, remove driveshaft and bench-inspect.

3. Flex front and rear U-joints through full range; grease if only dry, not if binding.

4. Pull a bearing cap, inspect needle bearings for grease/rust/discoloration.

5. Zero a dial indicator and measure axial AND radial endplay at each U-joint against current OEM spec -- zero measured play is also a defect (spalling risk), not just excess play.

6. Measure slip-yoke spline play with the dial indicator; heavy wear causes shaft-whipping vibration.

7. Inspect snap rings for correct style/seated tang; confirm coated rings/caps on aluminum shafts.

8. At the strap flange, confirm centering tab, no pinion leak, and replace strap bolts -- never reuse.

9. Identify staked (non-serviceable) joints -- plan full shaft replacement, never a DIY grind conversion.

10. If similar wear/age across joints, replace all while the shaft is out.

11. Confirm a vent path exists at the slip-yoke end.

12. Do not drive on a separating carrier bearing or a seized/near-separation U-joint -- tow or remove the shaft first.

## Grab list

Tools: Dial indicator with magnetic base, grease gun, jack stands + wheel chocks, hand tools for driveshaft R&R, shop light, current OEM service info

## Safety and escalation

No-drive/tow for a separating or free-hinging center support (carrier) bearing, a seized/near-separation U-joint, or any loose/missing strap-bolt hardware. Full R&R on a staked (non-serviceable) joint requires a shaft swap, not a field snap-ring conversion.

## Common misdiagnosis

Trusting hand-feel alone in either direction -- real spec play is below hand-feel threshold, so 'feels tight' does not clear a joint and 'feels a little loose' does not always condemn one; jumping straight to wheel-bearing or engine-mount causes without first isolating via the driveshaft-removed dial-indicator check; reusing half-round strap bolts; torching a staked snap ring out instead of pressing it

## Confirmation standard

Zeroed dial-indicator axial/radial U-joint endplay and slip-yoke spline-play measurement against current OEM spec is the confirming test -- hand-flex is triage only, not confirmation in either direction.

Owner note: Do not stop at hand-feel in a field triage -- it is the correct first pass, but the dial-indicator step is what actually confirms or clears a joint per this manual's evidence standard.

# 60. Rear differential/rear-end noise -- symptom triage through backlash and gear-pattern confirmation

Decision tree: Rear-end howl on acceleration that stops on deceleration and can be feathered with throttle (or generic clunk/rattle/grind) -> localize via stethoscope at the pinion-bearing zone vs each carrier-bearing zone -> check fluid level/leak for a cheaper standalone cause before assuming gear wear -> if gear wear suspected, measure backlash with a dial indicator and read the gear-marking-compound contact pattern -> replace ring & pinion/bearings and correct shim if confirmed -> re-verify backlash and pattern before return to service.

Provenance: GMM-EX-070. Single-source, moderate confidence -- gear-pattern interpretation is demonstrated on camera (before/after backlash numbers and a real contact-pattern reveal) but not taught as a transferable skill; not corroborated by a second source

## Evidence source

Rear End Noise? Diagnose and Fix a Differential in Your Car, Truck, or SUV — 1A Auto: Repair Tips & Secrets Only Mechanics Know: https://www.youtube.com/watch?v=KwoMUejhs_Q

## Ordered field steps

1. Reproduce noise, note throttle/speed correlation.

2. On a lift, stethoscope-check pinion-bearing area vs each side carrier-bearing area -- stay clear of the spinning driveshaft/yoke.

3. Check fluid level and leak/seal condition.

4. Dial-indicator backlash check before teardown.

5. Gear-marking compound + pry bar to read drive-side/coast-side contact pattern.

6. Replace ring & pinion, all bearings, and correct shim if worn/off-pattern.

7. Re-measure backlash.

8. Re-check pattern both sides.

9. Road-test to confirm symptom gone.

## Grab list

Tools: Dial indicator (magnetic base), gear marking compound, brush, pry bar, mechanic's stethoscope, lift/jack stands, hand tools for cover/bearing R&R

## Safety and escalation

Not stated in the source -- treat as: loud metallic grinding/clunking suggesting a chipped/broken ring-gear tooth, visible metal shavings in fluid, very low/empty fluid with dry-running signs, or a driveline clunk suggesting a spun bearing all risk sudden diff lockup or drive-loss and should not be driven. Gear-pattern interpretation and shim correction should be escalated to a specialist/shop with OE pattern references.

## Common misdiagnosis

Wheel-bearing or driveshaft/U-joint noise (see GMM-KB-059) mistaken for differential howl; tire noise or exhaust drone mistaken for gear howl; jumping straight to 'ring & pinion is bad' without first checking fluid level/leak, which can be a cheaper standalone fix

## Confirmation standard

Throttle-modulated reproduction of the howl (load-dependent = gear-noise signature); on-lift stethoscope localization to pinion vs carrier-bearing zone; backlash measurement outside case-specific spec; gear-marking-compound pattern outside a centered-oval norm

Owner note: No specific vehicle make/model identified in the source -- treat all backlash/pattern numbers as this-unit-only, never a general spec.

# 61. GM 4WD transfer case -- fluid leaks and 4WD electrical/encoder faults, symptom-location routing

Decision tree: Branch on symptom location, not a single root cause. Leak at input/front of case -> input seal (case-out repair) or case-half pinhole (bench repair). Leak at rear tail-shaft/driveshaft yoke -> tail-shaft seal (field-serviceable) or, if it leaks again after reseal, the tail-shaft bushing. 4WD fault light or CEL with no major leak -> encoder-motor position sensor (most common) or the full encoder motor.

Provenance: GMM-EX-071. Single-source, moderate confidence (3/5) -- very shallow source runtime (4:16) for 4 distinct failure modes; only the input-seal branch has a demonstrated (though not fully captured) confirm test

## Evidence source

Common GM Truck and SUV Transfer Case Problems — 1A Auto: Repair Tips & Secrets Only Mechanics Know: https://www.youtube.com/watch?v=2SGFYZ5Ks3c

## Ordered field steps

1. Confirm 4WD/AWD (2WD has no transfer case).

2. Support vehicle safely -- driveline can rotate, keep clear.

3. Visually inspect case exterior at input seal, case-half seam, and tail-shaft seal.

4. Input-seal check: drain pan under fill plug, pull plug -- excess fluid volume confirms a bad input seal.

5. Case-half check: inspect both halves for a pinhole/weep near the internal pump.

6. Tail-shaft leak: pull rear driveshaft, replace tail-shaft seal (drain hole faces down); if it leaks again, replace the tail-shaft bushing.

7. 4WD fault/CEL: scan for DTC, suspect the encoder-motor position sensor before the whole motor -- remove front driveshaft, remove encoder motor, disconnect connector, open motor, replace sensor.

8. Match symptom to specific failure mode before ordering parts.

## Grab list

Tools: Drain pan/funnel, basic hand tools, jack stands/lift, scan tool for 4WD DTC

## Safety and escalation

Tail-shaft seal/bushing and encoder-motor/sensor work are mobile-van feasible. Input-seal and case-half repairs require pulling the whole transfer case -- feasible if equipped for case R&R, but the internal repair itself is a bench job, not curbside.

## Common misdiagnosis

Replacing the tail-shaft seal alone when the bushing behind it is actually worn (the video explicitly flags this 'leaks again' scenario); replacing the whole encoder motor when only the internal sensor failed

## Confirmation standard

Fill-plug drain-volume check (input seal); direct visual inspection (case-half pinhole); reseal + road test (tail-shaft seal vs. bushing); scan-tool DTC pull (encoder motor vs. sensor)

Owner note: GM-platform-specific only -- no transfer case model, part number, torque, or fluid spec is stated in the source; every number a tech encounters in the field must come from that vehicle's actual OEM/service data.

# 62. Manual transmission internal noise -- isolating input shaft / main (output) shaft / countershaft bearing by operating condition

Decision tree: Branch on when the noise occurs: neutral-only -> input shaft bearing. Present in gear, absent in neutral -> main shaft bearing. Present in all forward gears except the direct-drive gear -> countershaft bearing. Unchanged when coasting in neutral at speed -> not the transmission (wheel/driveline/differential). Changes/stops when coasting in neutral at speed -> confirms transmission. Noise tied to clutch pedal travel -- from first movement = throwout bearing, only at full travel = pilot bearing (see GMM-KB-063 for full clutch-wear diagnosis).

Provenance: GMM-EX-072. Single-source, theory-based over a real static cutaway transmission (not corroborated by a hands-on fault demonstration or a second source) -- confidence held at 3/5 pending a corroborating source

## Evidence source

Noises associated with manual transmissions — BYUI AutomotiveVideos: https://www.youtube.com/watch?v=6WfPnb7MUCM

## Ordered field steps

1. Record exact complaint conditions (neutral/gear/all gears/moving/clutch pedal).

2. Engine running, neutral, clutch engaged: present only here = input shaft bearing.

3. Cycle each forward gear: absent in neutral but present in gear = main shaft bearing; present in every gear except the direct-drive gear = countershaft bearing.

4. Road test to 40-50mph then shift to neutral rolling: unchanged = NOT the transmission; changes/stops = confirms transmission.

5. If noise only occurs on the clutch pedal: first-movement throbbing = throwout bearing; only at full travel = pilot bearing.

6. Cross-check all three tests before naming a component.

7. Escalate -- internal bearing or clutch/pilot-bearing work requires transmission removal.

## Grab list

Tools: None specialized for triage -- safe road access and working gear/clutch controls; a mechanic's stethoscope is standard practice though not shown in the source

## Safety and escalation

Once an internal transmission bearing (input, main/output, countershaft) or the clutch throwout/pilot bearing is implicated, that repair requires transmission removal and is not a mobile-van job. This tree is diagnosis-only.

## Common misdiagnosis

Replacing a wheel bearing (or vice versa) without running the neutral-coast-at-speed test first; replacing the throwout bearing when it's actually the pilot bearing (or vice versa) without the graduated clutch-pedal-travel test; jumping to full transmission replacement without isolating input/main/countershaft first

## Confirmation standard

Neutral-vs-in-gear noise pattern; all-gears-except-direct-drive pattern; 40-50mph neutral-coast test; clutch-pedal-travel test. Require agreement across tests before naming a component.

Owner note: The '4th gear = direct drive, bypasses the countershaft' branch is specific to this 5-speed RWD-style layout -- verify the actual direct-drive gear/power-flow diagram per transmission model before applying to a specific vehicle.

# 63. Manual transmission clutch wear/slip diagnosis -- pre-replacement confirmation before a $1,500-$2,000+ R&R

Decision tree: Suspect worn clutch -> run cheap, no-drive pedal engagement-height and pedal-feel checks first -> if the pedal rides high with minimal free play, rule out hydraulic air (spongy pedal + grinding gears even at full travel) via bleed before condemning the clutch -> confirm with a road slip test (RPM/speed divergence) and/or a stationary parking-brake stall test -> only then recommend replacement.

Provenance: GMM-EX-073. Single-source, moderate-trust (3/5) -- pedal tests confirmed hands-on with real hardware; the road slip test was narrated/simulated only since the demo vehicle's clutch wasn't actually slipping, so treat that specific test's execution details as unverified on camera

## Evidence source

How to diagnose a clutch — Way of the Wrench: https://www.youtube.com/watch?v=ehHi3Taqd2g

## Ordered field steps

1. Confirm terminology and actuation type (mechanical/cable-adjustable vs. non-adjustable hydraulic).

2. Pedal engagement-height test, engine off: note where in travel the clutch catches -- last-third engagement suggests wear.

3. Pedal-feel test: press to first hint of resistance -- minimal free play before that suggests advanced wear.

4. Check pedal stroke for smooth build/return -- jerkiness suggests a pressure-plate/lubrication issue, not disc wear.

5. Rule out hydraulic air first: spongy pedal + grinding gears even at full travel means bleed and re-test before condemning the clutch.

6. Road slip test: watch tach and speedometer together while driving -- RPM rising while speed stays flat (especially low-to-high gear or on a hill) confirms slip.

7. Stationary stall-test confirm: high gear, parking brake fully set, apply throttle while releasing the clutch as if starting -- should stall if life remains, won't stall if slipping.

8. Only recommend replacement once the pedal test plus at least one active test (road slip or stall) both confirm.

## Grab list

Tools: None for pedal tests (hand/foot only); factory tachometer/speedometer for the road test; a fully-set parking brake for the stall test

## Safety and escalation

Not a tow condition by itself -- a slipping/worn clutch is generally still drivable short-distance/light-load until scheduled repair. If a hydraulic bleed doesn't fix a spongy/hard-to-shift pedal, escalate to hydraulic-system diagnosis before assuming clutch replacement is needed.

## Common misdiagnosis

Diagnosing clutch wear from a spongy/hard-to-shift pedal without first ruling out hydraulic air; assuming a cable-adjustment fix on a vehicle that is actually non-adjustable hydraulic without verifying actuation type first

## Confirmation standard

Road slip test (RPM/speed divergence while driving) and/or stationary stall test (high gear + full parking brake + throttle) -- require agreement, don't condemn off the pedal test alone

Owner note: Never treat the ~1-1.5in pedal free-play figure as universal -- it is this video's rough reference only, no OEM spec was given. Keep the hydraulic-vs-disc-wear misdiagnosis caution prominent -- it's the single most valuable field lesson in this source.

# 65. Shorted shift-solenoid circuit vs. current-limited-driver diagnostic pitfall -- when a test light or resistance check gives a false 'good' or ambiguous result

Decision tree: A ground-side-switched shift-solenoid circuit that appears electrically fine by static resistance yet the gear won't engage may be hiding a short that trips the TCM driver's current-limiting protection, which pulses the circuit on/off too fast (tens of microseconds) for a test light or the naked eye to register as anything but 'stuck.' Isolate the driver from the solenoid first with a substituted test light, then use an amp-clamp/lab-scope capture at microsecond resolution -- not a static ohms reading -- to confirm a real short versus a driver problem.

Provenance: GMM-EX-066. Single elite source (ScannerDanner, ASE-instructor-tier trusted channel elsewhere in this manual), real single-vehicle case study with every major claim visually confirmed against scope/scan-tool screens.

## Evidence source

Testing Transmission Shift Solenoids - (P0700, P0974) — ScannerDanner: https://www.youtube.com/watch?v=Sr7woeza8to

## Ordered field steps

1. Pull full code scan, identify the actionable module-specific code.

2. Road test to confirm the code matches the actual symptom.

3. Pull the wiring diagram for the affected circuit.

4. Backprobe the control wire, engine running, confirm B+ present when commanded off (feed side intact).

5. Command the solenoid on/off via scan-tool bi-directional control while watching a graphing meter to confirm the TCM attempts to switch it.

6. Isolate the TCM driver from the solenoid: unplug the TCM connector, substitute a loaded test light on the control pin, confirm correct on/off response in isolation.

7. Reconnect the real solenoid, repeat in-circuit -- if the light shows no visible change, do NOT jump to 'driver problem.'

8. Visually inspect the connector for contamination; clean if found.

9. When an in-circuit test light gives an ambiguous result, run an amp-clamp/lab-scope capture instead of concluding a driver fault.

10. Zoom the scope to microsecond/low-millisecond resolution to resolve current-limiting pulses and capture true peak current plus voltage pulldown.

11. Run the identical test on a known-good adjacent solenoid circuit as an in-vehicle healthy baseline.

12. Reason through whether a resistance check can actually isolate the fault before running one -- skip it if both circuit wires could be shorted together anywhere inside the transmission case.

13. Conclude and document; exact physical fault location (solenoid winding vs. internal harness) is not determinable without teardown.

14. Pull the OEM repair procedure and scope the full R&R chain before quoting labor.

## Grab list

Tools: Scan tool with bi-directional/guided controls and live PIDs, OEM wiring diagrams and repair info, an adjustable-load test light, a graphing multimeter, a low-amp current clamp, a lab scope capable of microsecond-resolution captures, an inspection mirror, brake cleaner, eye protection

## Safety and escalation

Once electrical diagnosis confirms the short is internal to the transmission, escalate to a shop with a lift for the repair phase -- platforms requiring a front subframe drop (e.g. GM 4T45-E) to reach the transmission side cover are not a mobile/driveway job, and the exact fault location (solenoid vs. harness) isn't visible until the cover is off anyway.

## Common misdiagnosis

Calling it a bad TCM driver or an open control circuit from a test light that appears 'stuck' with the solenoid plugged in -- the real cause can be a shorted solenoid tripping current-limiting, pulsing too fast to catch by eye. Trusting a static resistance check to tell you whether a short is in the solenoid winding or the internal harness -- it can't, if the two wires are shorted together inside the case. Wiring a relay onto the control circuit to force it on and clear the code -- turns off the light without fixing the missing gear and will likely set a different code.

## Confirmation standard

Amp-clamp + lab-scope capture on the suspect circuit at microsecond time base while cycling the solenoid via scan-tool bi-directional control, compared directly against an identical capture on a known-good adjacent solenoid circuit on the same connector. A shorted circuit shows an abnormally high current spike with incomplete pulldown toward ground; a healthy circuit shows a clean full pulldown at low steady current.

Owner note: Deliberately kept as a SEPARATE tree from GMM-KB-058 (Snap-on's broader resistance-test-first methodology) rather than merged -- a technician trained only on KB-058's resistance-first method would be exactly the one fooled by this failure mode. Cross-reference this tree from GMM-KB-058's Owner Notes. All current/voltage/resistance/labor-hour numbers are case-specific to the 2010 Chevy Cobalt 4T45-E shown -- never reuse on another platform.

# Exhaust and Restriction Expansion — Complete Routing Matrix

Severe low-power complaint with no obvious misfire → GMM-KB-064. Rules out fuel delivery (WOT O2 test) and breathing efficiency (volumetric-efficiency calculation) first, then confirms restriction with a steady-3,000-RPM-in-Park exhaust-backpressure reading against the general under-2-PSI threshold — the decisive test over any circumstantial evidence — before condemning a catalytic converter or O2 sensor.

# 64. Low-power/restricted-exhaust diagnosis -- backpressure-confirmed, with fuel-delivery and breathing-efficiency ruled out first

Decision tree: Severe low-power complaint with no obvious misfire -> confirm fuel delivery is NOT the cause via a WOT road test watching upstream O2 (narrowband flatline rich, or wideband tracking rich) -> optionally cross-check engine breathing via a Volumetric Efficiency calculation (80%+ passes) -> the DECISIVE test is exhaust backpressure: gauge threaded into the upstream O2 sensor bung (or an alternate DPFE/EGR port if inaccessible), read at idle (near-zero expected) then held at a STEADY 3,000 RPM in Park (general spec: no more than 2 PSI) -> a confirmed restriction (several PSI, needle burying) with a fixed-lean O2 sensor and continuous over-fueling is a classic cause of a melted/plugged catalytic converter -> do NOT trust circumstantial evidence (symptom feel, cooler cat-outlet temperature readings, low idle vacuum, snap-throttle vacuum spikes, or the debunked 'idle vs 3,000 RPM in Park' vacuum-only comparison) over the direct backpressure reading -- all of those can point toward a plugged exhaust that the backpressure gauge then disproves, or vice versa confirm what circumstantial evidence only suggested.

Provenance: GMM-EX-074, GMM-EX-075, and GMM-EX-076. Three-source merged tree: EX-074 demonstrates the full test methodology on a case that ultimately rules OUT exhaust restriction (a true-negative), EX-075 demonstrates a clean known-bad-vs-known-good comparison across 3 vehicles plus 3 alternate test-port procedures, and EX-076 demonstrates a real true-positive melted-cat case caused by a fixed-lean O2 sensor. All three independently restate the same <2 PSI/steady-3,000-RPM backpressure spec as a working general rule, not a single vehicle's factory number -- strong convergent evidence for that specific threshold.

## Evidence source

Low Power Diagnosis: Fuel Delivery or Exhaust Restriction? | Part 1 — ScannerDanner: https://www.youtube.com/watch?v=8q1OaCYe8bY

Exhaust Backpressure Testing (known bad compared to known good) — ScannerDanner: https://www.youtube.com/watch?v=Lafv2c4szZY

O2 sensor (fixed lean) and restricted exhaust test (melted cat) - GM — ScannerDanner: https://www.youtube.com/watch?v=UkyM1L_xbaU

## Ordered field steps

1. Road-test and confirm the complaint; rule out fuel delivery first via a WOT O2 test (narrowband flatline rich >800mV, or wideband tracking rich).

2. Optional: run a VE calculation (displacement, RPM, MAF, IAT, barometric estimate, humidity estimate) -- 80%+ passes, rules out a breathing restriction as the cause.

3. If an O2 sensor is reading a lean signal that doesn't match reality (e.g. black smoke on a throttle snap = real rich, contradicting a 'lean' O2 report), verify the sensor itself: unplug it engine-running and read bias voltage (~450mV general GM reference), then ground the harness-side signal wire and confirm the scan tool pulls to ~0mV to prove wiring integrity before condemning the sensor.

4. Install a backpressure gauge in the upstream O2 sensor bung (or an alternate DPFE-hose/EGR-valve/EGR-transducer port if inaccessible -- disconnect the EGR solenoid first on the DPFE method to prevent a false vacuum reading).

5. Read at idle (near-zero expected, up to ~0.5 PSI normal) then hold a STEADY 3,000 RPM in Park and read again -- disregard snap-throttle spikes, only the steady-state reading counts.

6. Apply the general threshold: no more than 2 PSI at steady 3,000 RPM; several PSI or a buried needle confirms restriction. Cross-check against a known-good comparison vehicle when available.

7. If a restriction is confirmed alongside a fixed-lean O2 history, the causal chain is: dead/lying O2 sensor -> continuous ECM over-fueling -> melted/plugged catalytic converter. Replace the O2 sensor AND the exhaust/converter, not just one.

8. On converter replacement, physically inspect the removed unit for broken substrate pieces that could lodge downstream in the muffler and cause a repeat restriction after the 'fix.'

## Grab list

Tools: Scan tool with live narrowband/wideband O2 and fuel-trim graphing, VE calculator software/spreadsheet, mechanical exhaust backpressure gauge + O2-bung adapter, alternate DPFE/EGR adapter fittings, jumper/ground wire for O2 wiring-integrity check

## Safety and escalation

No hot-exhaust/CO/burn-hazard content is taught in any of the three source videos despite all of them working directly around a hot, actively-running exhaust system (one source explicitly notes a real burn moment and gives a spoken CO caution only during an indoor smoke-color demo) -- apply standard PPE, cool-down, and CO-ventilation practice independently, do not treat the absence of on-camera warning as permission to skip it. Actual converter/exhaust fabrication work may exceed mobile-van scope depending on equipment -- one elite source's own shop subcontracted a comparable job.

## Common misdiagnosis

Condemning an O2 sensor for a lean code without first proving via bias-voltage and grounded-wire tests that the wiring (not the sensor) is good; trusting cat inlet/outlet temperature comparisons as diagnostic (explicitly flagged as unreliable by the elite source itself); using the debunked idle-vs-3,000-RPM-in-Park vacuum-only comparison as a restriction test; reading a snap-throttle backpressure spike as the test result instead of the steady-state 3,000 RPM reading; replacing only the catalytic converter without also fixing the fixed-lean O2 sensor that caused it (guarantees a comeback)

## Confirmation standard

Steady-3,000-RPM-in-Park backpressure reading against the <2 PSI general threshold, cross-checked against a known-good comparison vehicle, is the decisive confirming test -- described by the elite source as never having failed him. O2 sensor wiring integrity is separately confirmed via the bias-voltage-unplugged plus grounded-signal-wire-pulldown pair of tests.

Owner note: This tree deliberately teaches the test on both a true-negative (EX-074) and true-positive (EX-075/076) case so a tech learns what 'exhaust is fine' looks like on the same gauge, not just what 'exhaust is bad' looks like -- that symmetry is the main reason these three were merged into one tree instead of three.

# HVAC Expansion — Complete Routing Matrix

No cold air with the compressor confirmed running → GMM-KB-070 for manifold-gauge pressure-pattern classification against an ambient/refrigerant-specific chart. Compressor clutch not engaging at all → GMM-KB-071 for input/output/relay/wiggle-test circuit isolation before parts-changing. Blower not working, running slow, or missing a speed → GMM-KB-072 for resistor-block/PWM-module voltage-drop diagnosis. One zone's blend door not responding on a dual-zone automatic system → GMM-KB-073 for PID isolation before committing to teardown labor. Electric/HV compressor service questions route to GMM-KB-074 for reference/awareness only — Gold Mobile Mechanic does not service hybrid/EV vehicles.

# 70. AC no-cold-air diagnosis -- compressor/clutch engagement triage through manifold-gauge pressure-pattern classification

Decision tree: Confirm AC compressor clutch actually engages when commanded -> if not, verify refrigerant charge present first (no charge can block engagement entirely on many systems) -> test clutch coil power/ground, trace relay/fuse/wiring if absent -> replace clutch if mechanically slipping/not grabbing with power present -> once compressor runs, retest for cold air -> if still no cold air, connect evacuated manifold gauges and record static then dynamic high/low pressures against an ambient/refrigerant-specific chart -> classify the 5-way pressure pattern (at-target-but-no-cold-air = downstream airflow not refrigerant; low/low = undercharge; high/high = overcharge/air/condenser restriction; high-low/low-high = worn compressor/clutch OR metering device stuck open, differentiate by off-equalization speed; low/high = restriction/contamination).

Provenance: GMM-EX-078. Single source (Repair Geek, not the manual's ASE-instructor-tier trusted channel), but the pressure-pattern framework is standard industry-textbook material demonstrated end-to-end on a real 2008 Honda Civic multi-fault case worked to resolution. Recommend a second source corroborate the compressor-vs-metering-device differentiation before treating as fully authoritative.

## Evidence source

Vehicle AC System Diagnosis for Beginners. A/C System Troubleshooting — Repair Geek: https://www.youtube.com/watch?v=QAJv4U1JeRA

## Ordered field steps

1. Confirm compressor clutch engages when AC commanded on.

2. If not, verify refrigerant charge present first.

3. Test clutch coil power/ground with AC on; trace relay then fuse/wiring/switch if absent.

4. If power/ground present but clutch slips, replace clutch (mechanical fault, can stack independently on top of an electrical fault).

5. Retest for cold air once compressor confirmed running.

6. If still no cold air, evacuate gauge hoses with a vacuum pump, then connect manifold gauges (red/high, blue/low, physically keyed).

7. Record static pressure (confirms charge present only), then dynamic high/low pressures with engine running and AC on max.

8. Pull an ambient-temperature-specific pressure/temperature chart for the refrigerant type in use.

9. Classify the reading pattern into the 5-way branch to identify the fault category.

10. For the high-low/low-high ambiguous case, shut AC off and watch equalization speed to differentiate worn compressor/clutch from a metering device stuck open.

## Grab list

Tools: Manifold gauge set, vacuum pump, test light, ambient pressure/temperature chart for the refrigerant type in use (R134a or 1234yf)

## Safety and escalation

Not addressed in the source video beyond a passing mention of recovering refrigerant -- no EPA-compliant recovery/evacuation procedure demonstrated. Escalate any refrigerant recovery/EPA-compliance step to certified equipment/process, not covered by this tree.

## Common misdiagnosis

Chasing a refrigerant-pressure diagnosis before confirming the compressor clutch even engages; stopping at a static (engine-off) pressure reading and treating it as diagnostic; assuming only one fault exists once the first (e.g. electrical) cause is found -- a mechanical clutch fault can be a second, independent stacked problem; using a generic PSI number instead of an ambient/refrigerant-specific chart.

## Confirmation standard

Cold air present at vents once compressor is confirmed running and electrically/mechanically sound. Dynamic high/low pressures matching the ambient/refrigerant-specific chart target. For the ambiguous high-low/low-high pattern, rapid pressure equalization on AC-off points to metering device stuck open rather than compressor wear.

Owner note: Never treat a specific PSI number from this source as universal -- target pressures are chart- and ambient-temperature-dependent by design. Refrigerant recovery/EPA-compliant evacuation remains a residual gap, not covered on camera.

# 71. AC compressor clutch not engaging -- input/output/relay/wiggle-test circuit isolation method

Decision tree: When the AC clutch will not engage despite the system having refrigerant, work the circuit systematically from input to output: confirm the module sees the AC request, confirm it commands the output (bi-directional test), confirm the relay clicks and its output pin shows continuity to ground through the clutch coil via test light, then physically wiggle-test the harness while watching the light to pinpoint an intermittent open. Do not assume relay or clutch failure without this sequence -- connector/harness faults are common and easy to misdiagnose as a bad relay or bad clutch.

Provenance: GMM-EX-079. Single-source, ScannerDanner (the manual's designated ASE-instructor-tier trusted channel) -- promoted per elite-source rule, real evidence (scan tool screens, fuse-box test-light work, wiggle-test isolation, gauge verification).

## Evidence source

A/C Compressor Clutch Circuit Testing [All Cars] + Recharge — ScannerDanner: https://www.youtube.com/watch?v=sBueOct0qXY

## Ordered field steps

1. Confirm static gauge pressure only proves refrigerant presence, not an electrical fault.

2. Full code scan.

3. Verify AC request input toggles in live data.

4. Check output PID/status.

5. Bi-directional test to force output, bypassing input.

6. Optional tap test on clutch face.

7. Locate relay via diagram.

8. Test as-found (engine running, before disturbing anything) -- confirm relay clicks.

9. ID pins: 2 hot, 2 control.

10. Test light on battery positive at load-side output pin -- should show continuity to ground through clutch coil when commanded.

11. No light -> wiggle-test harness/connectors toward clutch watching for flicker to isolate the open.

12. Repair the isolated fault -- re-terminate and re-secure the harness itself, not just the connector, to prevent recurrence.

13. Re-verify with wiggle test (light stays solid) before closing up.

## Grab list

Tools: Scan tool with bi-directional control, test light, fuse/relay box diagram, terminal re-tensioning tool, harness hold-down/clip, manifold gauge set

## Safety and escalation

Intermittent fault that does not reproduce under wiggle test (may need extended data-logging over a real drive cycle). Harness damage requiring a full pigtail/splice repair kit not carried on the van. Clutch coil found internally open (not a harness fault) needing clutch/coil replacement, especially if access requires a lift.

## Common misdiagnosis

Replacing the relay or clutch coil based on 'clutch does not engage' alone without isolating the fault -- the actual fault is frequently a harness/connector issue upstream or downstream of the relay. Treating equal static pressure as proof the electrical side is fine -- it only proves refrigerant presence. Disturbing wiring/connectors before testing as-found, which can mask an intermittent contact fault.

## Confirmation standard

Test light at the relay load-side output pin shows continuity to ground through the clutch coil when commanded. Wiggle-test along the harness while watching the light -- a flicker at a specific point confirms that segment as the fault. Post-repair: wiggle-test again confirming no flicker, plus physical clutch engagement and gauge pressure response.

Owner note: Relay pinout and connector location are Ford F-150 5.0L-specific; the METHOD generalizes across makes, physical layout does not. Recharge segment is a partial low-loss top-off, not a substitute for certified recovery/evacuation on a system that needs to be opened.

# 72. Blower motor not working / running slow / missing a speed -- resistor-block and PWM-module circuit topology + voltage-drop diagnostic method

Decision tree: Blower speed circuits step voltage down through a resistor block (older) or a PWM-driven module (newer) between LOW and HIGH, with HIGH always bypassing the speed-control component for full battery voltage. Diagnose with a strategy-based voltage/ground check at each speed tap against the vehicle's actual wiring diagram, not assumption -- a single missing speed usually isolates to one resistor tap, connector, or switch contact, not the whole block or the motor.

Provenance: GMM-EX-080. Single-source, non-trusted-tier channel (RockAuto Tech Garage is reputable but not the manual's ASE-instructor-tier channel) -- promoted on the same basis as GMM-KB-070: standard textbook-accurate automotive electrical fundamentals demonstrated end-to-end on a real wired bench circuit with partial real-meter confirmation (HIGH-bypass independently verified at 12.42V).

## Evidence source

Understand Blower Motor Circuits to Better Diagnose Problems (Season 5/E12) — RockAuto Auto Parts: https://www.youtube.com/watch?v=KjQ0PKDiRDw

## Ordered field steps

1. Pull the vehicle-specific wiring diagram first -- verify market variant.

2. No speed commanded -> confirm 0V at motor supply; if near-battery-voltage with motor not running, suspect the motor itself.

3. Ground check via meter referenced to battery positive at motor ground -- should read near battery voltage if ground is good.

4. Test each speed tap LOW to HIGH, confirming each shows its diagram-predicted stepped voltage.

5. HIGH must show full unrestricted battery voltage via the bypass relay -- if not, suspect the relay or its control circuit.

6. One bad speed, others good -> isolate to that tap's resistor element, connector, or switch contact.

7. On PWM-module vehicles, same power/ground logic applies but may require scan-tool-commanded duty-cycle tests.

## Grab list

Tools: DVOM (DC volts), vehicle-specific wiring diagram, basic hand tools for resistor block/module and connector access, scan tool for PWM-module duty-cycle commands

## Safety and escalation

PWM blower-control-module faults needing bidirectional scan-tool commands or module-level fault codes beyond simple voltage/ground checks. Standard resistor-block diagnosis is fully mobile-van workable with just a DVOM and the wiring diagram -- no tow needed for the base method.

## Common misdiagnosis

Condemning the whole resistor block or the blower motor when only one speed tap, connector, or switch contact has failed. Testing against the wrong market-variant wiring diagram and chasing a component not actually in that vehicle's real circuit. Assuming a low-speed voltage reading is a fault when it's the resistor block working as designed.

## Confirmation standard

After any repair, re-test every speed position and confirm each shows its diagram-predicted stepped voltage, with HIGH always reading full unrestricted battery voltage via the bypass relay.

Owner note: Bench-board demo of a generic legacy resistor-block topology, not a live vehicle case. Many modern vehicles use PWM blower-control modules instead -- same power/ground/wiring-diagram logic applies, but the physical component and required tooling differ.

# 73. Automatic dual-zone HVAC blend door not responding to control (one zone) -- PID isolation + electrical-isolation-before-teardown method

Decision tree: When one zone's blend door will not respond on a dual-zone automatic HVAC system, first confirm dual-zone mode is actually enabled (single-zone mode masks a bad side by dragging it with the good side), then use scan-tool live-data PIDs to prove which specific door circuit is not changing when commanded. Before condemning the actuator and committing to a multi-hour teardown, electrically isolate the fault at the actuator connector with a test light or amp clamp -- the DTC alone cannot tell you whether the fault is wiring, the module, the actuator, or a stuck/bound door.

Provenance: GMM-EX-081. Single-source, ScannerDanner (the manual's designated ASE-instructor-tier trusted channel) -- promoted per elite-source rule with extensive real on-camera evidence. Confidence capped at 4/5 because the source is diagnostic-only and does not verify a final repair on camera -- it explicitly documents skipping the correct isolation step, which is preserved here as the actionable lesson.

## Evidence source

2013 Ford Expedition Temperature Blend Door Diagnosis — ScannerDanner: https://www.youtube.com/watch?v=9ETVzE90DVo

## Ordered field steps

1. Confirm exact zone + function affected by complaint.

2. Full-vehicle auto-scan, triage unrelated codes separately.

3. Read the HVAC-specific DTC in full.

4. Listening test after CONFIRMING dual-zone mode is on (critical gotcha).

5. Pull Live Data blend-door PIDs, ID which PID maps to which door.

6. Confirm working side's PID changes live when commanded; confirm suspect side's does not.

7. Optional: compare to a same-platform reference vehicle.

8. Pull OEM wiring diagram + OEM pinpoint test doc for the DTC.

9. BEFORE parts-changing: test light or amp-clamp isolation at the actuator connector on both motor-drive wires to separate wiring/module faults from actuator/door faults.

10. Only commit to teardown labor after isolation confirms the fault location.

## Grab list

Tools: Scan tool with full auto-scan + live-data PID capability, OEM wiring diagrams, OEM pinpoint test documentation, test light, amp clamp

## Safety and escalation

Once isolated to the actuator/door itself, hard-to-access-side R&R commonly requires dash or center-console teardown (8.7hrs on this platform) -- a shop/lift job, not a mobile-van roadside repair. Escalate/schedule shop time once the diagnostic step is complete; do not commit to teardown before that.

## Common misdiagnosis

Testing a zone's blend door function while still in single-zone (non-dual) mode -- the good side drags the bad side and creates a false 'it's moving' read. Jumping straight from a vague blend-door DTC to 'bad actuator' without electrical isolation, risking many hours of labor on a wrong guess. Treating unrelated historical DTCs surfaced in a full-system scan as related to the HVAC complaint.

## Confirmation standard

OEM pinpoint test for the specific DTC (differentiates wiring/terminals/connectors, actuator, module, or stuck/bound door/linkage). Test light or amp-clamp response in both directions at the actuator connector proves wiring/module are good and isolates the fault downstream to the actuator/door.

Owner note: DTC number, PID names/ranges, and labor-guide times are 2013 Ford Expedition/Navigator-platform specific. The underlying METHOD (dual-zone-mode gotcha check, PID isolation, electrical isolation before parts-changing) generalizes to any potentiometer-feedback blend/mode door actuator on any platform.

# 74. Electric (HV) AC compressor service safety -- POE-vs-PAG dielectric requirement + HV de-energization/testing procedure (REFERENCE/AWARENESS ONLY -- GMM does not service hybrid/EV vehicles)

Decision tree: Electric AC compressors run at lethal HV (300V+) and require certification-gated procedures: OEM-specific de-energization with a mandatory capacitor-discharge wait, dual continuity + insulation-resistance bench testing to distinguish an open winding from an insulation breakdown to case, Class 0 HV gloves during insulation testing, an equipotential bonding check after any HV component replacement, and -- the specific safety point this source was selected for -- POE oil (not PAG) because PAG is measurably more electrically conductive and unsuitable near HV compressor internals, plus a non-conductive-rated UV dye.

Provenance: GMM-EX-083. Single-source, but treated as elite/trusted for this narrow crossover topic specifically: filmed at Bosch with Bosch's own calibrated equipment, presenter obtains relevant certification on camera, and the core dielectric-safety claim is quantitatively demonstrated with real, legible meter readings on isolated oil samples.

## Evidence source

Fixing Your EV AC System the Right Way: Edd China Shows You How! — Edd China: https://www.youtube.com/watch?v=NAl3_Yh34Q8

## Ordered field steps

1. Certification required before HV component work.

2. ID refrigerant + oil family from vehicle sticker + refrigerant identifier before servicing an unknown-history vehicle.

3. Recover refrigerant with dedicated machine.

4. Pull OEM HV de-energization procedure via professional diagnostic software.

5. Follow exactly: cordon/PPE/disconnect, then WAIT the OEM-specified capacitor-discharge time.

6. Bench test removed component: continuity (open winding check) AND insulation resistance to case (short/breakdown check).

7. Class 0 HV gloves mandatory holding probes during insulation testing.

8. After install: equipotential bonding test <0.1 ohm before re-energizing.

9. Refill with POE oil only for electric compressors; never mix oil families.

10. Use only non-conductive-rated UV dye.

11. Recharge to spec, verify with vent-vs-ambient temperature check.

## Grab list

Tools: Dedicated refrigerant recovery/recharge machine, refrigerant identifier, OEM diagnostic software with HV procedure access, Class 0 HV insulating gloves, dedicated HV insulation resistance tester, standard multimeter, POE oil, non-conductive UV dye, thermometer

## Safety and escalation

GMM POLICY: hybrid/EV vehicles are outside accepted scope entirely -- any such request should be declined/referred to a certified EV/HV specialist shop, never serviced with standard ICE tools, standard PAG oil, or without EV/HV certification, dedicated HV test equipment, and Class-0-rated PPE. This tree exists so the team recognizes WHY that policy matters for AC/refrigerant work specifically, not to enable in-house EV service.

## Common misdiagnosis

Assuming any AC refrigerant oil is interchangeable -- PAG is correct on many conventional systems but measurably unsuitable for an electric compressor. Assuming a generic/older UV dye is safe near HV components without confirming a non-conductive formulation. Trusting an unknown vehicle's service history without independently verifying refrigerant/oil identity. Treating an open-winding fault and an insulation-breakdown-to-case fault as the same finding -- they require different tests.

## Confirmation standard

Continuity test isolates an open/broken winding. Insulation-resistance test isolates a short/breakdown to case. Equipotential bonding test (<0.1 ohm) confirms no dangerous potential difference before re-energizing. Post-repair: vent-outlet temperature vs measured ambient confirms functional cooling.

Owner note: REFERENCE/AWARENESS content only, per GMM's standing policy of not accepting hybrid/EV vehicles (Thomas's explicit, twice-stated instruction). Retained specifically so the team understands the refrigerant-oil-safety reasoning behind that boundary, not as an in-house EV service procedure. All voltage/resistance/timing specs are 2012 Nissan Leaf-platform-specific; the general principles generalize across EV/HV platforms.

# Residual Gap Closures — CAN, Tread/TPMS, Brake Hydraulics, No-Crank Case

A U-code or no-comm condition implicating the high-speed CAN network → GMM-KB-066 for terminating-resistor and waveform triage (a partial closure of the CAN-fundamentals gap only — see its Owner Notes for what remains open). A tread-wear pattern or a TPMS dash light → GMM-KB-067 for the visual-triage-plus-relearn branch. A spongy or failing brake pedal → GMM-KB-068 for master-cylinder diagnosis through to the ABS-trapped-air scan-tool bleed and its manual-bleed-first boundary. A genuine no-crank/no-start ending in a real tow-worthy mechanical failure → GMM-KB-069, the first verified real-car case replacing the prior invalid riding-lawn-tractor example.

# 66. CAN-bus differential-pair identification + terminating-resistor resistance test (U-code/no-comm triage)

Decision tree: When a U-code or no-comm condition implicates the high-speed CAN network, confirm bus wiring health with a resistance check across CAN-H/CAN-L at the DLC before condemning any module; confirm live communication with a scoped CAN-H/CAN-L capture under commanded traffic. NARROW SCOPE: this tree does NOT cover key-off precautions or working per-module network isolation -- both remain open sub-gaps, see Owner Notes.

Provenance: GMM-EX-084. Single source, PARTIALLY demonstrated -- LOW-MODERATE consensus. This is narrower than the full CAN-fundamentals gap named in the coverage audit.

## Evidence source

Network Communication Testing: K-Line, Serial and CAN Bus — Adam's Analysis: https://www.youtube.com/watch?v=m9SOBEzwio8

## Ordered field steps

1. Identify CAN-H (pin 6) and CAN-L (pin 14) at the DLC via breakout box or wiring diagram (SAE-standardized on 16-pin DLCs).

2. Key off before connecting meter leads for resistance testing (added caution -- general electrical best practice, not taught by the source).

3. DVOM across pin 6 and pin 14: expect roughly 60 ohms on a bus terminated by two 120-ohm resistors in parallel -- confirm the vehicle's actual termination count/spec before treating 60 ohms as gospel for every vehicle.

4. A reading far from expected (near 0 = likely short; open/infinite = broken termination or harness break) indicates a wiring-level fault to chase before condemning a module.

5. With key on and a scan tool commanding traffic, scope CAN-H and CAN-L together -- expect crisp mirrored digital packets; a noisy or flat trace on only one line points at that line.

6. Consult the wiring diagram to know which modules share the affected segment before disconnecting connectors.

7. A working per-module ping/isolation tool (e.g. GM's Data Bus Diagnostic Tool) would be the next step where available -- NOT independently verified working in the source material reviewed for this tree; confirm tool/device setup before relying on it.

## Grab list

Tools: 4-channel scope (PicoScope or equivalent), CAN-capable OBDII breakout box, auto-ranging DVOM, scan tool to generate bus traffic, OEM wiring diagrams

## Safety and escalation

If wiring/resistance work points to a fault inside a module, behind extensive teardown, or requires OEM per-module ping tooling not confirmed working, escalate/tow to a shop with full factory scan-tool access. An open-circuited CAN network can itself cause crank/no-start or no-crank on many vehicles.

## Common misdiagnosis

Condemning a module before checking bus-wide terminating-resistor resistance. Assuming activity on an unrelated K-line/serial breakout pin proves CAN-bus health -- different systems, different pins. Trusting a network-ping tool's 'detected' state without confirming it actually connected -- the source material for this tree shows exactly that failure mode on screen (GM Data Bus Diagnostic Tool repeatedly read 'No default device detected' throughout its demo).

## Confirmation standard

~60-ohm parallel-terminating-resistor DVOM reading; clean mirrored CAN-H/CAN-L waveform under commanded scan-tool traffic.

Owner note: DOES NOT FULLY CLOSE the CAN-bus-fundamentals residual gap flagged by GMM-KB-050. Confirmed present: differential-pair identification, terminating-resistor test. Confirmed ABSENT: key-off precaution (zero mentions in a full-transcript keyword search), a successfully-executed network-isolation demo (the on-camera attempt errors out and never connects). A second source demonstrating both of those is still needed before this gap can be called closed.

# 67. Tire tread-wear pattern reading (steering/suspension/inflation clue) + TPMS sensor-vs-pressure fault diagnosis, replacement, and relearn

Decision tree: Two complementary tire-domain branches: (1) visual tread-wear pattern points to a likely mechanical/inflation cause and should trigger a targeted follow-up check, not stand alone as a diagnosis; (2) a TPMS dash light requires a per-wheel tool readout to isolate a true sensor fault from a pressure fault, replacement if needed, and an OEM-sourced vehicle-specific relearn (never a memorized universal sequence) before confirming clear.

Provenance: GMM-EX-085 and GMM-EX-086. Two independent complementary sources -- MODERATE-HIGH consensus. Each covers a different but adjacent sub-domain (visual pattern reading vs. sensor/relearn procedure) rather than duplicating the same claim.

## Evidence source

Tire Wear Patterns And What They Mean? — JP Auto & Fleet Services: https://www.youtube.com/watch?v=Xe91Uryw2J4

TPMS Sensor Diagnose Replace Relearn — Bruce O (Automotive Fundamentals): https://www.youtube.com/watch?v=iEfKJ6XmL7k

## Ordered field steps

1. TREAD-WEAR BRANCH: Check tread-wear indicator bars -- flush with tread means replace.

2. Identify the pattern (cupping/center/outer-edge/single-side/diagonal-swipe/heel-toe/feathering/flat-spot).

3. Map to likely cause: cupping=worn shocks/struts; center=overinflation; outer-edge=underinflation; single-side=camber/ball-joint/spring/alignment; diagonal-swipe=bad toe (esp. FWD/loaded trucks); heel-toe=improper rotation/belt separation/worn ball joints or bushings; feathering=bad toe, sometimes worn wheel bearings/ball joints; flat-spots=prolonged parking or hard-braking/skid.

4. Use as an entry point into a targeted mechanical check -- not a standalone conclusion.

5. TPMS BRANCH: Note dash TPMS icon plus per-wheel pressure readout to spot a blank/faulted wheel.

6. Dedicated TPMS tool reads each sensor's PSI/temp/frequency/battery; 'NO SENSOR DETECTED' (distinct from 'battery low') signals true sensor failure.

7. Careful dismount respecting the sensor's drop-center position.

8. Replace sensor, re-check live with tool before remount.

9. Pull the VEHICLE-SPECIFIC relearn procedure from OEM service information.

10. Perform relearn per wheel via tool trigger in OEM order, confirmed by vehicle beep.

11. Confirm the dash light stays off through a subsequent key cycle.

## Grab list

Tools: Flashlight (tread read); handheld TPMS diagnostic/relearn tool; OEM service-information subscription; tire mount/demount equipment; replacement TPMS sensor; gloves

## Safety and escalation

Tread-wear patterns and a properly-relearned TPMS system are informational, not tow-worthy by themselves. Escalate if tread wear reveals a genuine safety condition (cords showing, sidewall cracking/damage) or the TPMS fault won't clear after a proper relearn attempt (deeper module/wiring fault).

## Common misdiagnosis

Treating a tread pattern as a finished diagnosis instead of a lead-in to a mechanical check. Treating any TPMS light as a simple low-pressure issue without checking the per-wheel readout. Conflating 'no sensor detected' with 'low battery' on the TPMS tool.

## Confirmation standard

Tread: a targeted component check matching the pattern (shock/strut inspection, alignment reading, tire-pressure check) confirms or rules out the suspected cause. TPMS: tool reads valid PSI/temp/frequency/battery on all 4 wheels; dash TPMS light stays off through a subsequent key-off/key-on cycle post-relearn.

Owner note: Tread-wear branch confidence 4/5 (visual-only, no live on-vehicle walkthrough or tread-depth-gauge demo). TPMS branch confidence 5/5 (full real end-to-end case). The vehicle-specific relearn requirement is satisfied by the source's own design, not just by this manual's framing -- do not let a future entry invent a universal relearn sequence.

# 68. Brake hydraulics -- spongy/failing pedal diagnosis (master-cylinder internal/external leak) through to ABS-trapped-air scan-tool bleed and its manual-bleed-first boundary

Decision tree: Spongy/low pedal complaint -> inspect master cylinder/booster for external leak signature and reservoir fluid condition -> remove master cylinder, inspect booster bore for internal contamination (dry = booster OK, wet = booster also compromised) -> inspect calipers and bleeder screws -> disassemble/inspect master-cylinder internals for swollen/rolled seals -> if a leak/failure is found, TOW GATE (visible master-cylinder brake-fluid leak is unsafe to drive) -> replace master cylinder, perform full MANUAL bleed first per manufacturer sequence -> if pedal remains soft on a modern ABS vehicle even after a correct manual bleed, air may be trapped in the ABS hydraulic control unit -> perform a scan-tool bi-directional ABS bleed with a capable scan tool, cycling each wheel per tool prompts, ONLY after the manual bleed is done -> confirm firm pedal / scan tool's own completion message.

Provenance: GMM-EX-087 and GMM-EX-088. Moderate. Two sources cover complementary, non-overlapping halves (master-cylinder diagnosis + tow gate vs. scan-tool bleed procedure + its service boundary). NEITHER source teaches the caliper-piston-clamping/bleeder-open caution -- that remains an added, non-sourced safety practice, not a verified field-video finding.

## Evidence source

Soft, Squishy Brakes? Hard to Stop? How to Diagnose Brake Master Cylinder Leak! — 1A Auto: Repair Tips & Secrets Only Mechanics Know: https://www.youtube.com/watch?v=RZJkFcvP65c

How to bleed brakes with a scan tool. No pumping necessary — The Modern Mechanic: https://www.youtube.com/watch?v=JRlncdEOKz8

## Ordered field steps

1. Inspect master cylinder/booster for external leak signature (corroded paint, fluid at reservoir seal); check reservoir fluid level/condition.

2. Remove master cylinder, inspect booster bore for internal contamination.

3. Inspect calipers (pad wear, manual piston push-back) and bleeder screws for corrosion/seizure before starting the job.

4. Disassemble/inspect master-cylinder internals for swollen/rolled seals to confirm internal failure.

5. If a leak/failure is found: TOW -- do not drive with a visible master-cylinder brake-fluid leak.

6. Replace master cylinder; perform a full MANUAL bleed first per manufacturer sequence.

7. If pedal remains soft/spongy on an ABS-equipped vehicle after a correct manual bleed, connect a bi-directional scan tool with a vehicle-specific ABS-bleed function.

8. Confirm reservoir fluid level; select the Bleed Brakes (ABS) function.

9. Per tool prompt, assistant holds pedal, open the bleeder screw at the indicated wheel, press OK -- ABS pump/valves cycle, fluid+air exit through the open bleeder; close bleeder; repeat per wheel per tool prompts.

10. Confirm tool's own completion message and a firm final pedal.

## Grab list

Tools: Hand tools for master-cylinder R&R, bench vise, shop towels, replacement master cylinder, brake fluid, manufacturer bleed-sequence reference, bi-directional scan tool with vehicle-specific ABS-bleed function, bleeder-screw wrench, catch bottle/tube, pedal depressor or second person

## Safety and escalation

Immediately upon discovering any visible master-cylinder brake-fluid leak -- do not drive, call a tow truck. The ABS scan-tool bleed step is shop/lift procedure, not a roadside fix for a leak.

## Common misdiagnosis

Assuming a spongy pedal is 'just air' without checking for an actual fluid leak. Treating an external master-cylinder leak as the whole story without checking internal booster contamination. Condemning a caliper for a rotted bleeder screw when the caliper body itself may be fine. Assuming persistent softness after a correct manual bleed means a bad master cylinder/caliper when it's actually air trapped in the ABS hydraulic control unit. Attempting the scan-tool bleed as a SUBSTITUTE for, rather than a follow-up to, a proper manual bleed.

## Confirmation standard

Dry booster bore after master-cylinder removal (rules out internal contamination). Firm pedal feel after a full manual bleed. If still soft on an ABS vehicle: scan tool's own completion message per wheel plus a firm final pedal.

Owner note: The caliper-piston-clamping caution gap is STILL OPEN after both sources -- do not claim it is closed anywhere downstream; flag for a dedicated caliper-service source if one becomes available. GMM-EX-088's on-screen vehicle text says 'Jeep Wrangler' while its YouTube description says 'Jeep Cherokee' -- trust on-screen video content over description text.

# 69. No-crank, no-start diagnosis -- systematic electrical-first-then-mechanical differentiation on a real passenger car, ending in a seized-engine/broken-connecting-rod escalation

Decision tree: Verify complaint and distinguish no-crank-no-start vs crank-no-start -> quick no-tool battery check via headlight brightness -> visual/wiggle battery-terminal and cable inspection -> check all fuses with a test light -> inspect starter connections, hammer-tap as a field trick (not a fix) -> if electrical checks out, suspect a mechanical (engine-side) rotation problem -> dipstick oil check + spark-plug removal/inspection for oil-burning residue -> hand-rotate crank bolt in the correct direction; if it will not turn at all, probe each cylinder via spark-plug hole with a screwdriver comparing piston position on pairs that should move together -> abnormal/inconsistent piston position confirms internal mechanical failure -- engine is seized, cannot move under its own power, needs replacement.

Provenance: GMM-EX-089. Single-source. First genuine real-car no-crank example in the manual, replacing the invalid riding-lawn-tractor example flagged in the prior coverage audit. Treat as a strong single source until corroborated by an additional independent case.

## Evidence source

No Crank, No Start Diagnosis - EricTheCarGuy — EricTheCarGuy: https://www.youtube.com/watch?v=BrqkhZmDnHQ

## Ordered field steps

1. Verify complaint firsthand.

2. Distinguish no-crank-no-start vs crank-no-start.

3. Quick headlight-brightness battery/connection check.

4. Visual/wiggle inspection of battery terminals and cables.

5. Check all fuses (interior + exterior box) with a test light.

6. Inspect starter connections; hammer-tap test as a field trick.

7. If electrical reasonably ruled out, suspect mechanical.

8. Dipstick oil check.

9. Remove spark plugs, inspect for oil-burning residue on multiple cylinders.

10. Hand-rotate crank bolt in correct direction with an open-hand push.

11. If it will not turn, probe each cylinder via spark-plug hole with a screwdriver, comparing piston position on pairs that should move together.

12. Confirm: no rotation + abnormal piston-position pattern = broken connecting rod / seized engine.

## Grab list

Tools: Test light, brass hammer, spark-plug socket/ratchet, crank-bolt socket (size is engine-specific), long screwdriver, shop rag. Ideal but not shown on camera: battery load/conductance tester, voltmeter for a proper voltage-drop test.

## Safety and escalation

Once the crankshaft is confirmed unable to rotate at all, the vehicle cannot start or move under its own power -- requires towing/transport to a shop for engine replacement, beyond mobile-van repair scope. This conclusion is clearly INFERABLE from the diagnosis but is NOT an explicit on-camera tow statement in this source (full-transcript search for 'tow' returns zero matches) -- reported honestly, not overclaimed.

## Common misdiagnosis

Jumping to 'bad starter' on hearing a click without first confirming the engine itself can rotate. Over-complicating the diagnosis by suspecting sensors/electronics before ruling out battery, connections, and fuses. Condemning and replacing a starter for a mechanical engine-seizure problem the starter could never have overcome regardless of its condition.

## Confirmation standard

Hand-rotate the crankshaft at the crank bolt in the engine's correct direction (confirm direction per vehicle). If it will not turn at all, use a screwdriver probe through the spark-plug holes to compare piston position on cylinder pairs that should move together -- an inconsistent/abnormal position confirms internal mechanical failure over an electrical/starter fault.

Owner note: Tow language is inferred, not verbatim/explicit, in this source. If a future source adds an explicit tow statement for a comparable seized-engine/no-crank case, prefer quoting that verbatim alongside this one rather than replacing this entry. Vehicle independently confirmed via frames (HONDA-branded valve cover), not assumed from title.